GATE 2027

Agricultural Engineering (AG)

Full-Length Mock Paper

Paper Code: AG (Agricultural Engineering)

Duration: 180 minutes (3 hours)

Maximum Marks: 100

Total Questions: 65 (General Aptitude: 10 | Agricultural Engineering: 55)

General Instructions & Marking Scheme

- The paper contains three question types: Multiple Choice Questions (MCQ), Multiple Select Questions (MSQ), and Numerical Answer Type (NAT) questions.

- MCQ: Four options are given, of which exactly ONE is correct. For a 1-mark MCQ, a wrong answer carries a negative mark of 1/3. For a 2-mark MCQ, a wrong answer carries a negative mark of 2/3. There is no negative marking for a question left unanswered.

- MSQ: Four options are given, of which ONE OR MORE are correct. The candidate must select all correct options for credit. There is no negative marking and no partial credit for MSQ.

- NAT: The candidate must enter a numerical answer using the on-screen keypad. No options are displayed. There is no negative marking for NAT questions.

- Q.1 to Q.5 (General Aptitude) carry 1 mark each; Q.6 to Q.10 (General Aptitude) carry 2 marks each.

- Q.11 to Q.35 (Agricultural Engineering) carry 1 mark each; Q.36 to Q.65 (Agricultural Engineering) carry 2 marks each.

- Unless stated otherwise, use g = 9.81 m/s². All questions are to be answered using only the data supplied within the question; no external data book is required or permitted.

# Section A — Question Paper

Answer all 65 questions. Marks and question type are indicated alongside each question number.

Q.1  [MCQ · 1 mark]

A tractor's fuel tank holds 45 litres. During 6 hours of continuous ploughing it uses 3/5 of a full tank. How many litres remain in the tank after 6 hours?

(A) 15

(B) 18

(C) 21

(D) 27

Q.2  [MCQ · 1 mark]

A tractor ploughs along the boundary of a rectangular field of length 120 m and width 80 m, completing one full round of the boundary at an average speed of 4 km/h. How many minutes does it take to complete one round?

(A) 4

(B) 5

(C) 6

(D) 8

Q.3  [MCQ · 1 mark]

Choose the word closest in meaning to 'meticulous'.

(A) Careless

(B) Painstaking

(C) Hasty

(D) Indifferent

Q.4  [MCQ · 1 mark]

Statements: (1) All combine harvesters are machines. (2) Some machines are tractors. Conclusions: I. Some combine harvesters are tractors. II. Some tractors are machines. Which conclusion(s) logically follow(s) from the statements?

(A) Only conclusion I

(B) Only conclusion II

(C) Both conclusions I and II

(D) Neither conclusion I nor II

Q.5  [MCQ · 1 mark]

The table shows the area (in hectares) under four crops in a village for two years.

Crop | 2024 | 2025

Wheat | 120 | 150

Rice | 90 | 100

Maize | 60 | 90

Pulses | 30 | 40

What is the percentage increase in the total cropped area from 2024 to 2025?

(A) 20.00%

(B) 23.33%

(C) 26.67%

(D) 30.00%

Q.6  [MCQ · 2 marks]

Two identical irrigation pumps, A and B, are used to fill a tank. Pump A alone fills the tank in 8 hours and Pump B alone fills it in 12 hours. Both pumps run together, but Pump B is switched off after 4 hours. In how many additional hours will Pump A alone fill the remaining tank?

(A) 1.00 h

(B) 1.20 h

(C) 1.33 h

(D) 1.50 h

Q.7  [MCQ · 2 marks]

The ratio of the present ages of a father and his son is 7:2. After 10 years, the ratio of their ages will be 9:4. What is the son's present age, in years?

(A) 8

(B) 10

(C) 12

(D) 14

Q.8  [MCQ · 2 marks]

Choose the option that best completes the sentence: 'Despite the heavy rainfall, the irrigation officer insisted that the scheduled canal inspection ____ postponed.'

(A) is not

(B) not be

(C) not being

(D) was not

Q.9  [MCQ · 2 marks]

In a certain code, FARM is written as HCTO. Following the same coding pattern, how is SEED written?

(A) UGGF

(B) UFFE

(C) TGFE

(D) UHHF

Q.10  [MCQ · 2 marks]

A square sheet of paper is folded in half twice — first vertically, then horizontally — to form a smaller square of one-quarter the original area. A single small triangular notch is then cut from one corner of the folded square. [Figure description: after the two folds, the folded square's four corners correspond, on the original unfolded sheet, to: (i) the centre of the original sheet, (ii) the midpoint of one edge, (iii) the midpoint of the adjacent edge, and (iv) one outer corner of the original sheet. The notch is cut only at the corner in position (i), i.e., the corner that all four folded layers share at the centre.] When the paper is unfolded, how many triangular holes appear, and where are they located?

(A) One hole, at the centre of the original sheet

(B) Two holes, symmetric about the centre

(C) Four holes, one in each quadrant, symmetric about the centre

(D) Four holes, all at the outer corners

Q.11  [MCQ · 1 mark]

Evaluate: lim(x→0) [ (sin 3x) / (tan 5x) ]

(A) 0.2

(B) 0.6

(C) 1.0

(D) 1.5

Q.12  [MCQ · 1 mark]

What is the integrating factor for the linear first-order differential equation dy/dx + (2/x)y = x³, for x > 0?

(A) x

(B) x²

(C) x³

(D) 1/x²

Q.13  [MCQ · 1 mark]

What are the order and degree, respectively, of the differential equation d²y/dx² + (dy/dx)³ + y = 0?

(A) 2 and 1

(B) 2 and 3

(C) 3 and 2

(D) 1 and 2

Q.14  [MCQ · 1 mark]

A bag contains 4 red and 6 green balls. Two balls are drawn at random without replacement. What is the probability that both balls drawn are red?

(A) 1/15

(B) 2/15

(C) 3/15

(D) 4/15

Q.15  [NAT · 1 mark]

The mean of a set of 8 observations is 15. If one observation of value 25 is removed from the set, what is the mean of the remaining 7 observations? (Round off to two decimal places.)

Q.16  [MCQ · 1 mark]

In a simple gear train used in a seed-cum-fertilizer drill, a driver gear with 20 teeth meshes with a driven gear having 60 teeth. If the driver gear rotates at 300 rpm, what is the speed of the driven gear?

(A) 60 rpm

(B) 100 rpm

(C) 150 rpm

(D) 900 rpm

Q.17  [MCQ · 1 mark]

In a seed drill, the term 'seed rate' refers to:

(A) The forward speed of the drill during sowing

(B) The quantity of seed sown per unit area of land

(C) The number of seed-drop openings per row

(D) The depth of seed placement below the soil surface

Q.18  [MCQ · 1 mark]

The fixed costs of operating a farm tractor include all of the following EXCEPT:

(A) Depreciation

(B) Interest on investment

(C) Fuel and lubricant cost

(D) Insurance and taxes

Q.19  [MCQ · 1 mark]

Which of the following is classified as a non-renewable source of farm power?

(A) Biogas

(B) Solar photovoltaic

(C) Diesel (petroleum-based fuel)

(D) Wind

Q.20  [MCQ · 1 mark]

The ratio of the drawbar pull to the dynamic weight on the driving wheels of a tractor is called:

(A) Slip

(B) Coefficient of traction

(C) Rolling resistance ratio

(D) Tractive efficiency

Q.21  [MCQ · 1 mark]

In a tractor power transmission system, the component that allows the two rear wheels to rotate at different speeds while turning is the:

(A) Clutch

(B) Differential

(C) Final drive

(D) Gear box

Q.22  [NAT · 1 mark]

The calorific value of diesel is 45,000 kJ/kg and its specific gravity is 0.85. Calculate the calorific value of diesel on a volume basis, in MJ per litre. (Round off to two decimal places.)

Q.23  [MCQ · 1 mark]

The time taken by runoff to travel from the hydraulically most remote point in a watershed to its outlet is called:

(A) Lag time

(B) Time of concentration

(C) Recession time

(D) Base time

Q.24  [MCQ · 1 mark]

In the Universal Soil Loss Equation (USLE), the factor that accounts for the combined effect of ground slope steepness and slope length on soil erosion is:

(A) K factor

(B) C factor

(C) LS factor

(D) P factor

Q.25  [MCQ · 1 mark]

Which of the following erosion-control structures is primarily used to reduce the velocity of concentrated flow in a gully and trap sediment?

(A) Contour bunding

(B) Check dam

(C) Terracing

(D) Grassed waterway

Q.26  [MCQ · 1 mark]

In levelling operations using a level instrument and a levelling staff, the reading taken on a staff held at a point of known elevation, at the start of a levelling operation, is called the:

(A) Foresight

(B) Intermediate sight

(C) Back sight

(D) Change point

Q.27  [MCQ · 1 mark]

The specific speed of a centrifugal pump is a parameter used primarily to:

(A) Determine the pump's power rating directly

(B) Classify pump type (radial, mixed, or axial flow) and select suitable impeller geometry

(C) Calculate the total dynamic head

(D) Determine the pump's electrical efficiency

Q.28  [NAT · 1 mark]

A tube well discharges 25 litres per second. If it operates continuously for 6 hours, calculate the total volume of water pumped, in cubic metres (m³). (Round off to the nearest whole number.)

Q.29  [MCQ · 1 mark]

The falling-rate period of drying of an agricultural product is characterized by:

(A) Constant surface moisture, with drying rate controlled by external air conditions

(B) Drying rate controlled by internal moisture diffusion within the product

(C) A drying rate that increases continuously with time

(D) No change in moisture content with time

Q.30  [MCQ · 1 mark]

The angle of repose of a granular agricultural material is defined as the angle:

(A) Between the direction of applied force and the material surface during shear failure

(B) Made by the free-flowing surface of a heaped pile of the material with the horizontal

(C) Between the major and minor axes of an individual grain

(D) At which the material begins to flow under vibration alone

Q.31  [NAT · 1 mark]

A wheat grain has axial dimensions of length 6.0 mm, width 3.2 mm, and thickness 2.8 mm. Calculate its geometric mean diameter, in mm. (Round off to two decimal places.)

Q.32  [NAT · 1 mark]

A cylindrical grain storage bin has an internal diameter of 4 m and is filled with wheat to a depth of 6 m. If the bulk density of wheat is 780 kg/m³, calculate the mass of wheat stored, in tonnes. (Round off to one decimal place.)

Q.33  [NAT · 1 mark]

A sample of paddy intended for storage weighs 500 g on a wet basis. After complete drying to remove all moisture, it weighs 430 g. Calculate the initial moisture content on a wet basis, in percent. (Round off to two decimal places.)

Q.34  [MCQ · 1 mark]

Pasteurization of milk is primarily intended to:

(A) Sterilize the milk completely, destroying all microorganisms including spores

(B) Destroy pathogenic microorganisms and reduce spoilage organisms while retaining most nutritional and sensory qualities

(C) Increase the fat content of milk

(D) Remove all enzymes present in raw milk

Q.35  [MSQ · 1 mark]

Which of the following statement(s) regarding food preservation methods is/are correct?

(A) Canning relies on hermetic sealing combined with heat treatment to destroy spoilage microorganisms

(B) Freezing preserves food primarily by increasing the water activity within the product

(C) Osmotic dehydration reduces water activity by using a concentrated solution (e.g., sugar or salt) to draw water out of the food

(D) Irradiation preserves food solely by raising its temperature to destroy microorganisms

Q.36  [MCQ · 2 marks]

The value of the definite integral ∫(0 to π/2) sin²x cos x dx is:

(A) 1/6

(B) 1/4

(C) 1/3

(D) 1/2

Q.37  [NAT · 2 marks]

Using the Taylor series expansion of f(x) = eˣ about x = 0, up to and including the term in x³, estimate the value of e^0.3. (Round off to three decimal places.)

Q.38  [MCQ · 2 marks]

The eigenvalues of the matrix A = [[4, 1], [2, 3]] are:

(A) 2 and 3

(B) 2 and 5

(C) 3 and 4

(D) 1 and 6

Q.39  [NAT · 2 marks]

Find the value of the determinant of the matrix B = [[2, 0, 1], [1, 3, 0], [0, 1, 4]].

Q.40  [NAT · 2 marks]

A single mouldboard plough has a width of cut of 30 cm and operates at a depth of 20 cm in a soil with a specific draft resistance of 55 kN/m². Calculate the draft required by the plough, in kN. (Round off to two decimal places.)

Q.41  [NAT · 2 marks]

A tractor pulls a disc harrow requiring a draft of 8.5 kN at a forward speed of 6 km/h. Calculate the drawbar power required, in kW. (Round off to two decimal places.)

Q.42  [NAT · 2 marks]

A V-belt drive connects a 15 kW electric motor pulley (300 mm diameter, running at 1440 rpm) to a driven pulley of 450 mm diameter used to run a chaff cutter. Assuming no slip, calculate the speed of the driven pulley, in rpm. (Round off to the nearest whole number.)

Q.43  [NAT · 2 marks]

A boom sprayer with 12 nozzles spaced 0.5 m apart is operated at a forward speed of 4 km/h. Each nozzle discharges 0.8 L/min. Calculate the spray application rate, in litres per hectare. (Round off to the nearest whole number.)

Q.44  [NAT · 2 marks]

A biogas plant produces 15 m³ of biogas per day, having a calorific value of 22 MJ/m³. If this biogas is used in a dual-fuel engine with an overall thermal efficiency of 25%, calculate the useful energy output per day, in MJ. (Round off to the nearest whole number.)

Q.45  [NAT · 2 marks]

A tractor weighing 18 kN has 70% of its weight on the rear (driving) wheels. If the coefficient of traction between the tyres and the soil is 0.55, calculate the maximum drawbar pull, in kN. (Round off to two decimal places.)

Q.46  [NAT · 2 marks]

A four-stroke, single-cylinder diesel engine has a bore of 100 mm and a stroke of 120 mm. Calculate the swept volume of the cylinder, in cm³. (Round off to one decimal place.)

Q.47  [NAT · 2 marks]

An engine operates on the ideal Otto cycle with a compression ratio of 8. Taking the ratio of specific heats γ = 1.4, calculate the theoretical thermal efficiency of the cycle, in percent. (Round off to one decimal place.)

Q.48  [NAT · 2 marks]

A tractor operator is exposed to a whole-body vibration with an acceleration of 0.8 m/s² for a continuous duration of 4 hours in a working day. The permissible vibration dose is taken as proportional to (acceleration)² × time. If the acceleration increases to 1.2 m/s², calculate the exposure duration, in hours, that would give the same permissible dose. (Round off to two decimal places.)

Q.49  [NAT · 2 marks]

A small agricultural watershed has an area of 25 hectares and a runoff coefficient of 0.35, and it receives a design rainfall intensity of 60 mm/h. Using the Rational Method, Q = CIA/360 (Q in m³/s, I in mm/h, A in hectares), calculate the peak runoff rate, in m³/s. (Round off to two decimal places.)

Q.50  [NAT · 2 marks]

In a watershed, the total rainfall in a storm was 80 mm, and the resulting direct runoff depth was 32 mm. If the effective duration of the rainfall producing runoff was 4 hours, calculate the φ-index (phi-index), in mm/h, using the relation: infiltration loss = total rainfall − direct runoff. (Round off to two decimal places.)

Q.51  [NAT · 2 marks]

Water flows over a rectangular sharp-crested weir of length 1.5 m under a head of 0.25 m. Using the discharge formula Q = 1.84 L H^1.5 (Q in m³/s, L and H in m), calculate the discharge over the weir, in m³/s. (Round off to three decimal places.)

Q.52  [NAT · 2 marks]

Water flows through a horizontal pipe that gradually contracts from a diameter of 300 mm to 150 mm. The velocity of flow in the 300 mm section is 1.2 m/s. Using the continuity equation, calculate the velocity of flow in the 150 mm section, in m/s. (Round off to two decimal places.)

Q.53  [NAT · 2 marks]

A field plot of area 400 m² recorded a soil loss of 18 kg during a rainstorm. Express this soil loss rate in tonnes per hectare. (Round off to two decimal places.)

Q.54  [NAT · 2 marks]

A saturated soil sample has a bulk unit weight of 19.5 kN/m³ and a water content of 25% (dry-weight basis). Given the unit weight of water as 9.81 kN/m³, calculate the dry unit weight of the soil, in kN/m³. (Round off to two decimal places.)

Q.55  [NAT · 2 marks]

A confined aquifer of 12 m thickness has a coefficient of permeability of 30 m/day. A fully penetrating well is pumped at a steady rate of 1500 m³/day. The drawdowns observed in two observation wells at radial distances of 15 m and 60 m from the pumped well are s₁ and s₂ respectively. Using the Thiem equation, Q = 2πKb(s₁ − s₂)/ln(r₂/r₁), calculate (s₁ − s₂), in metres. (Round off to two decimal places.)

Q.56  [MSQ · 2 marks]

For a loam soil with a field capacity of 28% and a permanent wilting point of 12% (both on a dry-weight basis), and a bulk density of 1.4 g/cm³, which of the following statement(s) is/are correct regarding the available water?

(A) The available water content on a mass basis is 16%

(B) The available water content on a mass basis is 40%

(C) The available water depth for a 1 m root zone is approximately 224 mm

(D) The available water depth for a 1 m root zone is approximately 160 mm

Q.57  [NAT · 2 marks]

An irrigation stream of 40 litres per second is applied to a border strip and is cut off after 90 minutes. If the water infiltrated into the root zone within the field is 150 m³, calculate the application efficiency, in percent. (Round off to one decimal place.)

Q.58  [NAT · 2 marks]

A subsurface drainage system is to be designed using the steady-state equation S² = 4Kh(2D + h)/q, where S is the drain spacing (m), K is the hydraulic conductivity = 0.5 m/day, h is the height of the water table above the drains at the midpoint = 0.6 m, D is the depth of the equivalent impermeable layer below the drains = 2 m, and q is the drainage coefficient = 0.01 m/day. Calculate the drain spacing S, in metres. (Round off to one decimal place.)

Q.59  [NAT · 2 marks]

A batch of paddy weighing 1000 kg (wet basis) with an initial moisture content of 22% (wet basis) is to be dried to a final moisture content of 13% (wet basis). Calculate the mass of water to be removed, in kg. (Round off to one decimal place.)

Q.60  [NAT · 2 marks]

During the constant-rate period of drying of a food product, the drying rate is 0.15 kg water/(kg dry solid·h). A sample containing 4 kg of dry solid is dried, entirely within the constant-rate period, from a moisture content of 0.80 kg water/kg dry solid to 0.50 kg water/kg dry solid. Calculate the time required, in hours. (Round off to two decimal places.)

Q.61  [MSQ · 2 marks]

In the size reduction of agricultural materials, which of the following statement(s) is/are correct?

(A) Rittinger's law assumes the energy required for size reduction is proportional to the new surface area created

(B) Kick's law assumes the energy required is proportional to the square of the reduction ratio

(C) Bond's law states that the energy required is inversely proportional to the square root of the product particle size, minus that of the feed particle size

(D) Hammer mills reduce material size primarily through shearing action between two smooth rollers

Q.62  [NAT · 2 marks]

A belt conveyor, 500 mm wide, moving at 1.2 m/s, carries grain forming a uniform cross-sectional area of 0.025 m² on the belt. Given the bulk density of the grain as 750 kg/m³, calculate the conveying capacity, in tonnes per hour. (Round off to one decimal place.)

Q.63  [NAT · 2 marks]

A flat slab of a food product, 4 cm thick, has a thermal conductivity of 0.5 W/(m·K). Under steady-state conditions, the temperature on one face is 60°C and on the other face is 20°C. Calculate the heat flux through the slab, in W/m². (Round off to the nearest whole number.)

Q.64  [NAT · 2 marks]

1000 kg of milk is to be cooled from 35°C to 5°C in a plate heat exchanger. Taking the specific heat of milk as 3.9 kJ/(kg·K), calculate the total heat to be removed, in MJ. (Round off to the nearest whole number.)

Q.65  [MSQ · 2 marks]

Which of the following statement(s) about the thermal processing of food is/are correct?

(A) The D-value (decimal reduction time) represents the time required at a given temperature to reduce the microbial population by 90% (one log cycle)

(B) The z-value represents the temperature change required to change the D-value by one log cycle (i.e., by a factor of 10)

(C) A higher D-value indicates that the microorganism is less heat-resistant

(D) The 12D concept, commonly applied to Clostridium botulinum in low-acid canned foods, refers to a process that reduces the spore population by 12 orders of magnitude (log cycles)

# Section B — Answer Key

| Q.No. | Section | Sub-topic | Type | Marks | Answer |
| --- | --- | --- | --- | --- | --- |
| 1 | General Aptitude | Quantitative Aptitude | MCQ | 1 | B |
| 2 | General Aptitude | Quantitative Aptitude | MCQ | 1 | C |
| 3 | General Aptitude | Verbal Aptitude | MCQ | 1 | B |
| 4 | General Aptitude | Analytical Aptitude | MCQ | 1 | B |
| 5 | General Aptitude | Data Interpretation | MCQ | 1 | C |
| 6 | General Aptitude | Quantitative Aptitude | MCQ | 2 | C |
| 7 | General Aptitude | Quantitative Aptitude | MCQ | 2 | B |
| 8 | General Aptitude | Verbal Aptitude | MCQ | 2 | B |
| 9 | General Aptitude | Analytical Aptitude | MCQ | 2 | A |
| 10 | General Aptitude | Spatial Aptitude | MCQ | 2 | C |
| 11 | Engineering Mathematics | Calculus | MCQ | 1 | B |
| 12 | Engineering Mathematics | Differential Equations | MCQ | 1 | B |
| 13 | Engineering Mathematics | Differential Equations | MCQ | 1 | A |
| 14 | Engineering Mathematics | Probability and Statistics | MCQ | 1 | B |
| 15 | Engineering Mathematics | Probability and Statistics | NAT | 1 | 13.50 to 13.60 |
| 16 | Farm Machinery | Machine Design (Machine Elements) | MCQ | 1 | B |
| 17 | Farm Machinery | Equipment for Sowing/Planting/Fertilizer Application | MCQ | 1 | B |
| 18 | Farm Machinery | Cost Analysis of Implements and Tractors | MCQ | 1 | C |
| 19 | Farm Power | Sources of Power | MCQ | 1 | C |
| 20 | Farm Power | Tractors & Power Tillers — Traction & Mechanics of Tractor Chassis | MCQ | 1 | B |
| 21 | Farm Power | Power Transmission (Clutch, Gear Box, Differential, Final Drive, Brakes) | MCQ | 1 | B |
| 22 | Farm Power | Fuels, Combustion and Lubricants | NAT | 1 | 38.00 to 38.50 |
| 23 | Soil & Water Conservation Engineering | Hydrology | MCQ | 1 | B |
| 24 | Soil & Water Conservation Engineering | Soil and Water Erosion | MCQ | 1 | C |
| 25 | Soil & Water Conservation Engineering | Soil and Water Erosion | MCQ | 1 | B |
| 26 | Soil & Water Conservation Engineering | Surveying and Levelling | MCQ | 1 | C |
| 27 | Irrigation & Drainage Engineering | Wells and Pumps | MCQ | 1 | B |
| 28 | Irrigation & Drainage Engineering | Wells and Pumps | NAT | 1 | 538 to 542 |
| 29 | Agricultural Process Engineering | Evaporation and Drying | MCQ | 1 | B |
| 30 | Agricultural Process Engineering | Engineering Properties of Agricultural Produce | MCQ | 1 | B |
| 31 | Agricultural Process Engineering | Engineering Properties of Agricultural Produce | NAT | 1 | 3.70 to 3.85 |
| 32 | Agricultural Process Engineering | Storage Systems | NAT | 1 | 58.00 to 59.60 |
| 33 | Agricultural Process Engineering | Storage Systems | NAT | 1 | 13.90 to 14.10 |
| 34 | Dairy & Food Engineering | Preservation of Food | MCQ | 1 | B |
| 35 | Dairy & Food Engineering | Preservation of Food | MSQ | 1 | AC |
| 36 | Engineering Mathematics | Calculus | MCQ | 2 | C |
| 37 | Engineering Mathematics | Calculus | NAT | 2 | 1.345 to 1.355 |
| 38 | Engineering Mathematics | Linear Algebra | MCQ | 2 | B |
| 39 | Engineering Mathematics | Linear Algebra | NAT | 2 | 24.50 to 25.50 |
| 40 | Farm Machinery | Farm Machinery — Tillage & Forces on a Tillage Tool | NAT | 2 | 3.20 to 3.40 |
| 41 | Farm Machinery | Farm Machinery — Tillage & Forces on a Tillage Tool | NAT | 2 | 13.90 to 14.40 |
| 42 | Farm Machinery | Machine Design (Machine Elements) | NAT | 2 | 955 to 965 |
| 43 | Farm Machinery | Equipment for Spraying | NAT | 2 | 235 to 245 |
| 44 | Farm Power | Sources of Power | NAT | 2 | 80 to 85 |
| 45 | Farm Power | Tractors & Power Tillers — Traction & Mechanics of Tractor Chassis | NAT | 2 | 6.80 to 7.05 |
| 46 | Farm Power | Farm Power — Engine Components & I.C. Engine Systems | NAT | 2 | 935.0 to 950.0 |
| 47 | Farm Power | Thermodynamic Principles & I.C. Engine Cycles | NAT | 2 | 55.50 to 57.50 |
| 48 | Farm Power | Human Engineering, Ergonomics and Safety | NAT | 2 | 1.70 to 1.85 |
| 49 | Soil & Water Conservation Engineering | Hydrology | NAT | 2 | 1.40 to 1.50 |
| 50 | Soil & Water Conservation Engineering | Hydrology | NAT | 2 | 11.50 to 12.50 |
| 51 | Soil & Water Conservation Engineering | Fluid Mechanics | NAT | 2 | 0.335 to 0.355 |
| 52 | Soil & Water Conservation Engineering | Fluid Mechanics | NAT | 2 | 4.70 to 4.90 |
| 53 | Soil & Water Conservation Engineering | Soil and Water Erosion | NAT | 2 | 0.40 to 0.50 |
| 54 | Soil & Water Conservation Engineering | Soil Mechanics | NAT | 2 | 15.40 to 15.80 |
| 55 | Irrigation & Drainage Engineering | Groundwater Hydrology | NAT | 2 | 0.80 to 1.00 |
| 56 | Irrigation & Drainage Engineering | Soil-Water-Plant Relationship | MSQ | 2 | AC |
| 57 | Irrigation & Drainage Engineering | Irrigation Water Conveyance and Application Methods | NAT | 2 | 68.50 to 70.50 |
| 58 | Irrigation & Drainage Engineering | Agricultural Drainage | NAT | 2 | 23.00 to 24.00 |
| 59 | Agricultural Process Engineering | Evaporation and Drying | NAT | 2 | 101.00 to 106.00 |
| 60 | Agricultural Process Engineering | Evaporation and Drying | NAT | 2 | 1.90 to 2.10 |
| 61 | Agricultural Process Engineering | Size Reduction and Material Handling | MSQ | 2 | AC |
| 62 | Agricultural Process Engineering | Size Reduction and Material Handling | NAT | 2 | 79.50 to 82.50 |
| 63 | Dairy & Food Engineering | Heat and Mass Transfer | NAT | 2 | 495 to 505 |
| 64 | Dairy & Food Engineering | Heat and Mass Transfer | NAT | 2 | 115 to 119 |
| 65 | Dairy & Food Engineering | Preservation of Food | MSQ | 2 | ABD |

# Section C — Detailed Solutions

Q.1

General Aptitude  ›  Quantitative Aptitude   |   MCQ · 1 mark

Question:

A tractor's fuel tank holds 45 litres. During 6 hours of continuous ploughing it uses 3/5 of a full tank. How many litres remain in the tank after 6 hours?

(A) 15

(B) 18

(C) 21

(D) 27

Answer: B

Explanation:

Fuel used = (3/5) × 45 = 27 L. Fuel remaining = 45 − 27 = 18 L. (A) 15 wrongly assumes 2/3 tank used. (C) 21 wrongly assumes 24/45 used. (D) 27 is the amount used, not the amount remaining — a common mix-up between 'used' and 'left'.

Q.2

General Aptitude  ›  Quantitative Aptitude   |   MCQ · 1 mark

Question:

A tractor ploughs along the boundary of a rectangular field of length 120 m and width 80 m, completing one full round of the boundary at an average speed of 4 km/h. How many minutes does it take to complete one round?

(A) 4

(B) 5

(C) 6

(D) 8

Answer: C

Explanation:

Perimeter = 2(120+80) = 400 m = 0.4 km. Time = distance/speed = 0.4/4 = 0.1 h = 6 min. (A) 4 and (B) 5 come from using only one pair of sides or a wrong speed conversion. (D) 8 results from using half the correct speed.

Q.3

General Aptitude  ›  Verbal Aptitude   |   MCQ · 1 mark

Question:

Choose the word closest in meaning to 'meticulous'.

(A) Careless

(B) Painstaking

(C) Hasty

(D) Indifferent

Answer: B

Explanation:

'Meticulous' means showing great attention to detail; 'painstaking' carries the same sense of careful, detailed effort. (A) Careless and (D) Indifferent are near-opposites in meaning. (C) Hasty (done quickly, without care) is also broadly opposite in spirit to meticulous.

Q.4

General Aptitude  ›  Analytical Aptitude   |   MCQ · 1 mark

Question:

Statements: (1) All combine harvesters are machines. (2) Some machines are tractors. Conclusions: I. Some combine harvesters are tractors. II. Some tractors are machines. Which conclusion(s) logically follow(s) from the statements?

(A) Only conclusion I

(B) Only conclusion II

(C) Both conclusions I and II

(D) Neither conclusion I nor II

Answer: B

Explanation:

Statement (2), 'Some machines are tractors', can always be logically converted to 'Some tractors are machines' — so conclusion II follows directly. Conclusion I does not necessarily follow: 'combine harvesters' is only one subset of 'machines', and nothing guarantees this particular subset overlaps with the subset that are tractors. (A) wrongly accepts I. (C) wrongly accepts both. (D) wrongly rejects the valid conversion in II.

Q.5

General Aptitude  ›  Data Interpretation   |   MCQ · 1 mark

Question:

The table shows the area (in hectares) under four crops in a village for two years.

Crop | 2024 | 2025

Wheat | 120 | 150

Rice | 90 | 100

Maize | 60 | 90

Pulses | 30 | 40

What is the percentage increase in the total cropped area from 2024 to 2025?

(A) 20.00%

(B) 23.33%

(C) 26.67%

(D) 30.00%

Answer: C

Explanation:

Total area in 2024 = 120+90+60+30 = 300 ha. Total area in 2025 = 150+100+90+40 = 380 ha. Increase = 380 − 300 = 80 ha. Percentage increase = (80/300) × 100 = 26.67%. (A), (B) and (D) result from using an incorrect base (e.g., the 2025 total) or a rounding slip.

Q.6

General Aptitude  ›  Quantitative Aptitude   |   MCQ · 2 marks

Question:

Two identical irrigation pumps, A and B, are used to fill a tank. Pump A alone fills the tank in 8 hours and Pump B alone fills it in 12 hours. Both pumps run together, but Pump B is switched off after 4 hours. In how many additional hours will Pump A alone fill the remaining tank?

(A) 1.00 h

(B) 1.20 h

(C) 1.33 h

(D) 1.50 h

Answer: C

Explanation:

Rate of A = 1/8 tank/h; rate of B = 1/12 tank/h. In 4 h together they fill 4×(1/8 + 1/12) = 4×(5/24) = 5/6 of the tank. Remaining fraction = 1/6. Time for A alone to fill this = (1/6)/(1/8) = 8/6 = 1.33 h. (A), (B) and (D) arise from arithmetic slips in adding the fractional rates or in the final division.

Q.7

General Aptitude  ›  Quantitative Aptitude   |   MCQ · 2 marks

Question:

The ratio of the present ages of a father and his son is 7:2. After 10 years, the ratio of their ages will be 9:4. What is the son's present age, in years?

(A) 8

(B) 10

(C) 12

(D) 14

Answer: B

Explanation:

Let father's age = 7x and son's age = 2x. After 10 years: (7x+10)/(2x+10) = 9/4. Cross-multiplying: 4(7x+10) = 9(2x+10) → 28x + 40 = 18x + 90 → 10x = 50 → x = 5. Son's present age = 2x = 10 years. (A), (C) and (D) arise from sign or cross-multiplication errors while solving the linear equation.

Q.8

General Aptitude  ›  Verbal Aptitude   |   MCQ · 2 marks

Question:

Choose the option that best completes the sentence: 'Despite the heavy rainfall, the irrigation officer insisted that the scheduled canal inspection ____ postponed.'

(A) is not

(B) not be

(C) not being

(D) was not

Answer: B

Explanation:

The verb 'insisted that' introduces a mandative subjunctive construction, which in formal English takes the bare/base form of the verb: '...insisted that the inspection not be postponed.' (A), (C) and (D) use tensed or non-finite forms that do not fit the subjunctive construction required after 'insisted that'.

Q.9

General Aptitude  ›  Analytical Aptitude   |   MCQ · 2 marks

Question:

In a certain code, FARM is written as HCTO. Following the same coding pattern, how is SEED written?

(A) UGGF

(B) UFFE

(C) TGFE

(D) UHHF

Answer: A

Explanation:

Each letter of FARM is shifted forward by 2 positions in the alphabet: F→H, A→C, R→T, M→O, giving HCTO. Applying the same +2 shift to SEED: S→U, E→G, E→G, D→F, giving UGGF. (B), (C) and (D) apply an inconsistent or wrong shift to one or more letters.

Q.10

General Aptitude  ›  Spatial Aptitude   |   MCQ · 2 marks

Question:

A square sheet of paper is folded in half twice — first vertically, then horizontally — to form a smaller square of one-quarter the original area. A single small triangular notch is then cut from one corner of the folded square. [Figure description: after the two folds, the folded square's four corners correspond, on the original unfolded sheet, to: (i) the centre of the original sheet, (ii) the midpoint of one edge, (iii) the midpoint of the adjacent edge, and (iv) one outer corner of the original sheet. The notch is cut only at the corner in position (i), i.e., the corner that all four folded layers share at the centre.] When the paper is unfolded, how many triangular holes appear, and where are they located?

(A) One hole, at the centre of the original sheet

(B) Two holes, symmetric about the centre

(C) Four holes, one in each quadrant, symmetric about the centre

(D) Four holes, all at the outer corners

Answer: C

Explanation:

The corner cut corresponds to the point where all four folded layers coincide at the sheet's centre. Cutting through all four layers there produces one small triangular hole in each of the four quadrants of the unfolded sheet, arranged symmetrically about the centre. (A) and (B) underestimate the number of layers cut through. (D) misidentifies which corner of the fold the notch was cut from.

Q.11

Engineering Mathematics  ›  Calculus   |   MCQ · 1 mark

Question:

Evaluate: lim(x→0) [ (sin 3x) / (tan 5x) ]

(A) 0.2

(B) 0.6

(C) 1.0

(D) 1.5

Answer: B

Explanation:

As x→0, sin 3x ≈ 3x and tan 5x ≈ 5x, so the limit = 3x/5x = 3/5 = 0.6. (A), (C) and (D) result from using the wrong coefficients (e.g., 5/3 inverted, or treating sin and tan as equal to x with the same coefficient).

Q.12

Engineering Mathematics  ›  Differential Equations   |   MCQ · 1 mark

Question:

What is the integrating factor for the linear first-order differential equation dy/dx + (2/x)y = x³, for x > 0?

(A) x

(B) x²

(C) x³

(D) 1/x²

Answer: B

Explanation:

Integrating factor = exp(∫(2/x)dx) = exp(2 ln x) = x². (A) uses only one power of the coefficient. (C) confuses the integrating factor with the right-hand side term. (D) inverts the sign of the exponent.

Q.13

Engineering Mathematics  ›  Differential Equations   |   MCQ · 1 mark

Question:

What are the order and degree, respectively, of the differential equation d²y/dx² + (dy/dx)³ + y = 0?

(A) 2 and 1

(B) 2 and 3

(C) 3 and 2

(D) 1 and 2

Answer: A

Explanation:

The order is the highest derivative present, which is d²y/dx² (order 2). The degree is the power of the highest-order derivative term once the equation is a polynomial in derivatives, which here is 1 (d²y/dx² appears to the first power). (B) mistakes the exponent on the lower-order term (dy/dx)³ for the degree. (C) and (D) swap or misidentify order and degree.

Q.14

Engineering Mathematics  ›  Probability and Statistics   |   MCQ · 1 mark

Question:

A bag contains 4 red and 6 green balls. Two balls are drawn at random without replacement. What is the probability that both balls drawn are red?

(A) 1/15

(B) 2/15

(C) 3/15

(D) 4/15

Answer: B

Explanation:

P(both red) = (4/10) × (3/9) = 12/90 = 2/15. (A) results from an arithmetic slip in the multiplication. (C) and (D) result from using the wrong denominator (e.g., not reducing the pool after the first draw, or drawing with replacement).

Q.15

Engineering Mathematics  ›  Probability and Statistics   |   NAT · 1 mark

Question:

The mean of a set of 8 observations is 15. If one observation of value 25 is removed from the set, what is the mean of the remaining 7 observations? (Round off to two decimal places.)

Answer: 13.50 to 13.60

Explanation:

Sum of the original 8 observations = 8 × 15 = 120. After removing 25, the sum of the remaining 7 observations = 120 − 25 = 95. New mean = 95/7 = 13.57.

Q.16

Farm Machinery  ›  Machine Design (Machine Elements)   |   MCQ · 1 mark

Question:

In a simple gear train used in a seed-cum-fertilizer drill, a driver gear with 20 teeth meshes with a driven gear having 60 teeth. If the driver gear rotates at 300 rpm, what is the speed of the driven gear?

(A) 60 rpm

(B) 100 rpm

(C) 150 rpm

(D) 900 rpm

Answer: B

Explanation:

For meshing gears, N₂ = N₁ × (T₁/T₂) = 300 × (20/60) = 100 rpm. (A) and (C) invert or misapply the teeth ratio. (D) multiplies instead of dividing by the ratio.

Q.17

Farm Machinery  ›  Equipment for Sowing/Planting/Fertilizer Application   |   MCQ · 1 mark

Question:

In a seed drill, the term 'seed rate' refers to:

(A) The forward speed of the drill during sowing

(B) The quantity of seed sown per unit area of land

(C) The number of seed-drop openings per row

(D) The depth of seed placement below the soil surface

Answer: B

Explanation:

Seed rate is defined as the quantity (mass) of seed sown per unit area of land, usually expressed in kg/ha. (A), (C) and (D) describe other operational or design parameters of a seed drill, not the seed rate itself.

Q.18

Farm Machinery  ›  Cost Analysis of Implements and Tractors   |   MCQ · 1 mark

Question:

The fixed costs of operating a farm tractor include all of the following EXCEPT:

(A) Depreciation

(B) Interest on investment

(C) Fuel and lubricant cost

(D) Insurance and taxes

Answer: C

Explanation:

Fixed costs are incurred irrespective of the extent of tractor use — depreciation, interest on investment, insurance, and taxes are all fixed costs. Fuel and lubricant cost varies directly with hours of operation and is therefore a variable (operating) cost, not a fixed cost.

Q.19

Farm Power  ›  Sources of Power   |   MCQ · 1 mark

Question:

Which of the following is classified as a non-renewable source of farm power?

(A) Biogas

(B) Solar photovoltaic

(C) Diesel (petroleum-based fuel)

(D) Wind

Answer: C

Explanation:

Diesel is derived from petroleum, a fossil fuel that is finite and non-renewable. Biogas, solar, and wind are all renewable energy sources commonly used in agriculture.

Q.20

Farm Power  ›  Tractors & Power Tillers — Traction & Mechanics of Tractor Chassis   |   MCQ · 1 mark

Question:

The ratio of the drawbar pull to the dynamic weight on the driving wheels of a tractor is called:

(A) Slip

(B) Coefficient of traction

(C) Rolling resistance ratio

(D) Tractive efficiency

Answer: B

Explanation:

The coefficient of traction is defined as the ratio of the maximum drawbar pull developed to the dynamic (normal) weight on the driving wheels. (A) Slip refers to the relative loss of forward travel due to wheel spin, not a force ratio. (C) and (D) describe different, unrelated performance parameters.

Q.21

Farm Power  ›  Power Transmission (Clutch, Gear Box, Differential, Final Drive, Brakes)   |   MCQ · 1 mark

Question:

In a tractor power transmission system, the component that allows the two rear wheels to rotate at different speeds while turning is the:

(A) Clutch

(B) Differential

(C) Final drive

(D) Gear box

Answer: B

Explanation:

The differential permits the two driving wheels to rotate at different speeds when the tractor turns, while still receiving power from a common source. (A) The clutch engages/disengages engine power. (C) The final drive provides the last speed reduction and torque increase. (D) The gear box selects the overall speed ratio.

Q.22

Farm Power  ›  Fuels, Combustion and Lubricants   |   NAT · 1 mark

Question:

The calorific value of diesel is 45,000 kJ/kg and its specific gravity is 0.85. Calculate the calorific value of diesel on a volume basis, in MJ per litre. (Round off to two decimal places.)

Answer: 38.00 to 38.50

Explanation:

Mass of 1 litre of diesel = specific gravity × density of water = 0.85 × 1 kg/L = 0.85 kg. Calorific value per litre = 45,000 kJ/kg × 0.85 kg = 38,250 kJ/L = 38.25 MJ/L.

Q.23

Soil & Water Conservation Engineering  ›  Hydrology   |   MCQ · 1 mark

Question:

The time taken by runoff to travel from the hydraulically most remote point in a watershed to its outlet is called:

(A) Lag time

(B) Time of concentration

(C) Recession time

(D) Base time

Answer: B

Explanation:

Time of concentration is defined as the time required for runoff to travel from the most hydraulically distant point of the watershed to the outlet. (A) Lag time is measured from the centroid of rainfall excess to the peak of the hydrograph. (C) and (D) refer to other segments of the hydrograph, not travel time.

Q.24

Soil & Water Conservation Engineering  ›  Soil and Water Erosion   |   MCQ · 1 mark

Question:

In the Universal Soil Loss Equation (USLE), the factor that accounts for the combined effect of ground slope steepness and slope length on soil erosion is:

(A) K factor

(B) C factor

(C) LS factor

(D) P factor

Answer: C

Explanation:

The LS factor (topographic factor) in USLE combines the effects of slope length (L) and slope steepness (S) on soil loss. (A) K is the soil erodibility factor, (B) C is the cover-management factor, and (D) P is the support-practice factor — none of these represent topography.

Q.25

Soil & Water Conservation Engineering  ›  Soil and Water Erosion   |   MCQ · 1 mark

Question:

Which of the following erosion-control structures is primarily used to reduce the velocity of concentrated flow in a gully and trap sediment?

(A) Contour bunding

(B) Check dam

(C) Terracing

(D) Grassed waterway

Answer: B

Explanation:

A check dam is a small structure built across a gully or drainage line specifically to reduce flow velocity and trap sediment. (A) Contour bunding and (C) terracing are used on the general land slope to control sheet/rill erosion, while (D) a grassed waterway safely conveys concentrated flow rather than obstructing it.

Q.26

Soil & Water Conservation Engineering  ›  Surveying and Levelling   |   MCQ · 1 mark

Question:

In levelling operations using a level instrument and a levelling staff, the reading taken on a staff held at a point of known elevation, at the start of a levelling operation, is called the:

(A) Foresight

(B) Intermediate sight

(C) Back sight

(D) Change point

Answer: C

Explanation:

The back sight is the first staff reading taken on a point of known elevation (e.g., a benchmark) after setting up the instrument, and is added to the elevation to obtain the height of the instrument. (A) Foresight is the last reading taken from a setup, on a point whose elevation is being determined. (B) Intermediate sights are readings taken between the back sight and foresight. (D) A change point is the point where both a foresight and a subsequent back sight are taken.

Q.27

Irrigation & Drainage Engineering  ›  Wells and Pumps   |   MCQ · 1 mark

Question:

The specific speed of a centrifugal pump is a parameter used primarily to:

(A) Determine the pump's power rating directly

(B) Classify pump type (radial, mixed, or axial flow) and select suitable impeller geometry

(C) Calculate the total dynamic head

(D) Determine the pump's electrical efficiency

Answer: B

Explanation:

Specific speed is a dimensionless (or dimensional index) parameter derived from discharge, head, and rotational speed at the best efficiency point, and is primarily used to classify the pump type and guide impeller design/selection. (A), (C) and (D) describe quantities calculated separately, not the purpose of specific speed.

Q.28

Irrigation & Drainage Engineering  ›  Wells and Pumps   |   NAT · 1 mark

Question:

A tube well discharges 25 litres per second. If it operates continuously for 6 hours, calculate the total volume of water pumped, in cubic metres (m³). (Round off to the nearest whole number.)

Answer: 538 to 542

Explanation:

Volume = discharge × time = 25 L/s × (6 × 3600 s) = 25 × 21,600 = 540,000 L = 540 m³.

Q.29

Agricultural Process Engineering  ›  Evaporation and Drying   |   MCQ · 1 mark

Question:

The falling-rate period of drying of an agricultural product is characterized by:

(A) Constant surface moisture, with drying rate controlled by external air conditions

(B) Drying rate controlled by internal moisture diffusion within the product

(C) A drying rate that increases continuously with time

(D) No change in moisture content with time

Answer: B

Explanation:

In the falling-rate period, the rate of drying is governed by the rate at which moisture can diffuse from the interior of the product to its surface, and the drying rate decreases as moisture content decreases. (A) describes the constant-rate period instead. (C) and (D) do not describe any real drying period.

Q.30

Agricultural Process Engineering  ›  Engineering Properties of Agricultural Produce   |   MCQ · 1 mark

Question:

The angle of repose of a granular agricultural material is defined as the angle:

(A) Between the direction of applied force and the material surface during shear failure

(B) Made by the free-flowing surface of a heaped pile of the material with the horizontal

(C) Between the major and minor axes of an individual grain

(D) At which the material begins to flow under vibration alone

Answer: B

Explanation:

Angle of repose is the angle that the sloping (free) surface of a naturally heaped pile of a granular material makes with the horizontal plane. (A) describes an angle of internal friction/shear concept, (C) describes grain shape, and (D) is not a standard definition.

Q.31

Agricultural Process Engineering  ›  Engineering Properties of Agricultural Produce   |   NAT · 1 mark

Question:

A wheat grain has axial dimensions of length 6.0 mm, width 3.2 mm, and thickness 2.8 mm. Calculate its geometric mean diameter, in mm. (Round off to two decimal places.)

Answer: 3.70 to 3.85

Explanation:

Geometric mean diameter (GMD) = (L × W × T)^(1/3) = (6.0 × 3.2 × 2.8)^(1/3) = (53.76)^(1/3) ≈ 3.77 mm.

Q.32

Agricultural Process Engineering  ›  Storage Systems   |   NAT · 1 mark

Question:

A cylindrical grain storage bin has an internal diameter of 4 m and is filled with wheat to a depth of 6 m. If the bulk density of wheat is 780 kg/m³, calculate the mass of wheat stored, in tonnes. (Round off to one decimal place.)

Answer: 58.00 to 59.60

Explanation:

Volume of grain = (π/4) × d² × h = (π/4) × 4² × 6 = (π/4) × 96 = 75.40 m³. Mass = volume × bulk density = 75.40 × 780 = 58,810 kg = 58.8 t.

Q.33

Agricultural Process Engineering  ›  Storage Systems   |   NAT · 1 mark

Question:

A sample of paddy intended for storage weighs 500 g on a wet basis. After complete drying to remove all moisture, it weighs 430 g. Calculate the initial moisture content on a wet basis, in percent. (Round off to two decimal places.)

Answer: 13.90 to 14.10

Explanation:

Moisture content (wet basis) = (initial mass − dry mass)/initial mass × 100 = (500 − 430)/500 × 100 = 70/500 × 100 = 14.00%.

Q.34

Dairy & Food Engineering  ›  Preservation of Food   |   MCQ · 1 mark

Question:

Pasteurization of milk is primarily intended to:

(A) Sterilize the milk completely, destroying all microorganisms including spores

(B) Destroy pathogenic microorganisms and reduce spoilage organisms while retaining most nutritional and sensory qualities

(C) Increase the fat content of milk

(D) Remove all enzymes present in raw milk

Answer: B

Explanation:

Pasteurization is a mild heat treatment designed to destroy pathogenic and most spoilage microorganisms while preserving the nutritional value, flavour, and other sensory qualities of milk. (A) describes sterilization, a more severe process. (C) and (D) are not the purpose of pasteurization.

Q.35

Dairy & Food Engineering  ›  Preservation of Food   |   MSQ · 1 mark

Question:

Which of the following statement(s) regarding food preservation methods is/are correct?

(A) Canning relies on hermetic sealing combined with heat treatment to destroy spoilage microorganisms

(B) Freezing preserves food primarily by increasing the water activity within the product

(C) Osmotic dehydration reduces water activity by using a concentrated solution (e.g., sugar or salt) to draw water out of the food

(D) Irradiation preserves food solely by raising its temperature to destroy microorganisms

Answer: AC

Explanation:

Canning combines hermetic (airtight) sealing with a heat process sufficient to destroy spoilage and pathogenic organisms, confirming (A). Freezing lowers, not increases, the water activity of a product by converting free water into ice, so (B) is incorrect. Osmotic dehydration uses a concentrated solution to draw water out of food by osmosis, lowering water activity, confirming (C). Irradiation is a non-thermal preservation method that inactivates microorganisms through ionizing radiation, not primarily by heating, so (D) is incorrect.

Q.36

Engineering Mathematics  ›  Calculus   |   MCQ · 2 marks

Question:

The value of the definite integral ∫(0 to π/2) sin²x cos x dx is:

(A) 1/6

(B) 1/4

(C) 1/3

(D) 1/2

Answer: C

Explanation:

Let u = sin x, du = cos x dx. The integral becomes ∫(0 to 1) u² du = [u³/3] from 0 to 1 = 1/3. (A), (B) and (D) result from errors in the substitution limits or from mis-integrating u² as u²/2 or u.

Q.37

Engineering Mathematics  ›  Calculus   |   NAT · 2 marks

Question:

Using the Taylor series expansion of f(x) = eˣ about x = 0, up to and including the term in x³, estimate the value of e^0.3. (Round off to three decimal places.)

Answer: 1.345 to 1.355

Explanation:

eˣ ≈ 1 + x + x²/2 + x³/6. At x = 0.3: 1 + 0.3 + (0.09/2) + (0.027/6) = 1 + 0.3 + 0.045 + 0.0045 = 1.3495 ≈ 1.350.

Q.38

Engineering Mathematics  ›  Linear Algebra   |   MCQ · 2 marks

Question:

The eigenvalues of the matrix A = [[4, 1], [2, 3]] are:

(A) 2 and 3

(B) 2 and 5

(C) 3 and 4

(D) 1 and 6

Answer: B

Explanation:

Characteristic equation: (4−λ)(3−λ) − (1)(2) = 0 → λ² − 7λ + 10 = 0 → (λ−5)(λ−2) = 0, so λ = 5, 2. (A) uses the diagonal entries directly without solving the characteristic equation. (C) and (D) result from sign or arithmetic errors in expanding the determinant.

Q.39

Engineering Mathematics  ›  Linear Algebra   |   NAT · 2 marks

Question:

Find the value of the determinant of the matrix B = [[2, 0, 1], [1, 3, 0], [0, 1, 4]].

Answer: 24.50 to 25.50

Explanation:

Expanding along the first row: det(B) = 2×(3×4 − 0×1) − 0×(1×4 − 0×0) + 1×(1×1 − 3×0) = 2×12 − 0 + 1×1 = 24 + 1 = 25.

Q.40

Farm Machinery  ›  Farm Machinery — Tillage & Forces on a Tillage Tool   |   NAT · 2 marks

Question:

A single mouldboard plough has a width of cut of 30 cm and operates at a depth of 20 cm in a soil with a specific draft resistance of 55 kN/m². Calculate the draft required by the plough, in kN. (Round off to two decimal places.)

Answer: 3.20 to 3.40

Explanation:

Draft = specific draft resistance × width of cut × depth of operation = 55 kN/m² × 0.30 m × 0.20 m = 55 × 0.06 = 3.30 kN.

Q.41

Farm Machinery  ›  Farm Machinery — Tillage & Forces on a Tillage Tool   |   NAT · 2 marks

Question:

A tractor pulls a disc harrow requiring a draft of 8.5 kN at a forward speed of 6 km/h. Calculate the drawbar power required, in kW. (Round off to two decimal places.)

Answer: 13.90 to 14.40

Explanation:

Convert speed: 6 km/h = 6000/3600 = 1.667 m/s. Drawbar power = Draft × speed = 8.5 kN × 1.667 m/s = 14.17 kW.

Q.42

Farm Machinery  ›  Machine Design (Machine Elements)   |   NAT · 2 marks

Question:

A V-belt drive connects a 15 kW electric motor pulley (300 mm diameter, running at 1440 rpm) to a driven pulley of 450 mm diameter used to run a chaff cutter. Assuming no slip, calculate the speed of the driven pulley, in rpm. (Round off to the nearest whole number.)

Answer: 955 to 965

Explanation:

For a belt drive with no slip: N₂ = N₁ × (D₁/D₂) = 1440 × (300/450) = 960 rpm.

Q.43

Farm Machinery  ›  Equipment for Spraying   |   NAT · 2 marks

Question:

A boom sprayer with 12 nozzles spaced 0.5 m apart is operated at a forward speed of 4 km/h. Each nozzle discharges 0.8 L/min. Calculate the spray application rate, in litres per hectare. (Round off to the nearest whole number.)

Answer: 235 to 245

Explanation:

Effective spray swath width = 12 × 0.5 = 6 m. Total nozzle discharge = 12 × 0.8 = 9.6 L/min. Forward speed = 4 km/h = 4000/60 = 66.67 m/min. Area covered per minute = swath × speed = 6 × 66.67 = 400 m²/min. Application rate = (discharge/area) × 10,000 m²/ha = (9.6/400) × 10,000 = 240 L/ha.

Q.44

Farm Power  ›  Sources of Power   |   NAT · 2 marks

Question:

A biogas plant produces 15 m³ of biogas per day, having a calorific value of 22 MJ/m³. If this biogas is used in a dual-fuel engine with an overall thermal efficiency of 25%, calculate the useful energy output per day, in MJ. (Round off to the nearest whole number.)

Answer: 80 to 85

Explanation:

Energy content of biogas produced = 15 m³ × 22 MJ/m³ = 330 MJ. Useful energy output = 330 × 0.25 = 82.5 MJ.

Q.45

Farm Power  ›  Tractors & Power Tillers — Traction & Mechanics of Tractor Chassis   |   NAT · 2 marks

Question:

A tractor weighing 18 kN has 70% of its weight on the rear (driving) wheels. If the coefficient of traction between the tyres and the soil is 0.55, calculate the maximum drawbar pull, in kN. (Round off to two decimal places.)

Answer: 6.80 to 7.05

Explanation:

Dynamic weight on driving wheels = 18 × 0.70 = 12.6 kN. Maximum drawbar pull = dynamic weight × coefficient of traction = 12.6 × 0.55 = 6.93 kN.

Q.46

Farm Power  ›  Farm Power — Engine Components & I.C. Engine Systems   |   NAT · 2 marks

Question:

A four-stroke, single-cylinder diesel engine has a bore of 100 mm and a stroke of 120 mm. Calculate the swept volume of the cylinder, in cm³. (Round off to one decimal place.)

Answer: 935.0 to 950.0

Explanation:

Swept volume V = (π/4) × d² × L, with d = 10 cm and L = 12 cm. V = (π/4) × 10² × 12 = (π/4) × 1200 = 942.5 cm³.

Q.47

Farm Power  ›  Thermodynamic Principles & I.C. Engine Cycles   |   NAT · 2 marks

Question:

An engine operates on the ideal Otto cycle with a compression ratio of 8. Taking the ratio of specific heats γ = 1.4, calculate the theoretical thermal efficiency of the cycle, in percent. (Round off to one decimal place.)

Answer: 55.50 to 57.50

Explanation:

For the Otto cycle: η = 1 − 1/r^(γ−1), with r = 8 and γ−1 = 0.4. 8^0.4 = 2.297. η = 1 − 1/2.297 = 1 − 0.4353 = 0.5647, i.e., 56.5%.

Q.48

Farm Power  ›  Human Engineering, Ergonomics and Safety   |   NAT · 2 marks

Question:

A tractor operator is exposed to a whole-body vibration with an acceleration of 0.8 m/s² for a continuous duration of 4 hours in a working day. The permissible vibration dose is taken as proportional to (acceleration)² × time. If the acceleration increases to 1.2 m/s², calculate the exposure duration, in hours, that would give the same permissible dose. (Round off to two decimal places.)

Answer: 1.70 to 1.85

Explanation:

For equal dose: a₁²t₁ = a₂²t₂ → 0.8² × 4 = 1.2² × t₂ → 2.56 = 1.44 × t₂ → t₂ = 1.78 h.

Q.49

Soil & Water Conservation Engineering  ›  Hydrology   |   NAT · 2 marks

Question:

A small agricultural watershed has an area of 25 hectares and a runoff coefficient of 0.35, and it receives a design rainfall intensity of 60 mm/h. Using the Rational Method, Q = CIA/360 (Q in m³/s, I in mm/h, A in hectares), calculate the peak runoff rate, in m³/s. (Round off to two decimal places.)

Answer: 1.40 to 1.50

Explanation:

Q = CIA/360 = (0.35 × 60 × 25)/360 = 525/360 = 1.46 m³/s.

Q.50

Soil & Water Conservation Engineering  ›  Hydrology   |   NAT · 2 marks

Question:

In a watershed, the total rainfall in a storm was 80 mm, and the resulting direct runoff depth was 32 mm. If the effective duration of the rainfall producing runoff was 4 hours, calculate the φ-index (phi-index), in mm/h, using the relation: infiltration loss = total rainfall − direct runoff. (Round off to two decimal places.)

Answer: 11.50 to 12.50

Explanation:

Infiltration (and other) losses = total rainfall − direct runoff = 80 − 32 = 48 mm. φ-index = total loss / effective duration = 48/4 = 12.00 mm/h.

Q.51

Soil & Water Conservation Engineering  ›  Fluid Mechanics   |   NAT · 2 marks

Question:

Water flows over a rectangular sharp-crested weir of length 1.5 m under a head of 0.25 m. Using the discharge formula Q = 1.84 L H^1.5 (Q in m³/s, L and H in m), calculate the discharge over the weir, in m³/s. (Round off to three decimal places.)

Answer: 0.335 to 0.355

Explanation:

Q = 1.84 × 1.5 × (0.25)^1.5. (0.25)^1.5 = 0.25 × √0.25 = 0.25 × 0.5 = 0.125. Q = 1.84 × 1.5 × 0.125 = 0.345 m³/s.

Q.52

Soil & Water Conservation Engineering  ›  Fluid Mechanics   |   NAT · 2 marks

Question:

Water flows through a horizontal pipe that gradually contracts from a diameter of 300 mm to 150 mm. The velocity of flow in the 300 mm section is 1.2 m/s. Using the continuity equation, calculate the velocity of flow in the 150 mm section, in m/s. (Round off to two decimal places.)

Answer: 4.70 to 4.90

Explanation:

By continuity, A₁V₁ = A₂V₂, so V₂ = V₁ × (D₁/D₂)² = 1.2 × (300/150)² = 1.2 × 4 = 4.80 m/s.

Q.53

Soil & Water Conservation Engineering  ›  Soil and Water Erosion   |   NAT · 2 marks

Question:

A field plot of area 400 m² recorded a soil loss of 18 kg during a rainstorm. Express this soil loss rate in tonnes per hectare. (Round off to two decimal places.)

Answer: 0.40 to 0.50

Explanation:

Soil loss per m² = 18/400 = 0.045 kg/m². Soil loss per hectare (10,000 m²) = 0.045 × 10,000 = 450 kg/ha = 0.45 t/ha.

Q.54

Soil & Water Conservation Engineering  ›  Soil Mechanics   |   NAT · 2 marks

Question:

A saturated soil sample has a bulk unit weight of 19.5 kN/m³ and a water content of 25% (dry-weight basis). Given the unit weight of water as 9.81 kN/m³, calculate the dry unit weight of the soil, in kN/m³. (Round off to two decimal places.)

Answer: 15.40 to 15.80

Explanation:

Dry unit weight γ_d = γ_bulk / (1 + w) = 19.5 / (1 + 0.25) = 19.5/1.25 = 15.60 kN/m³.

Q.55

Irrigation & Drainage Engineering  ›  Groundwater Hydrology   |   NAT · 2 marks

Question:

A confined aquifer of 12 m thickness has a coefficient of permeability of 30 m/day. A fully penetrating well is pumped at a steady rate of 1500 m³/day. The drawdowns observed in two observation wells at radial distances of 15 m and 60 m from the pumped well are s₁ and s₂ respectively. Using the Thiem equation, Q = 2πKb(s₁ − s₂)/ln(r₂/r₁), calculate (s₁ − s₂), in metres. (Round off to two decimal places.)

Answer: 0.80 to 1.00

Explanation:

Rearranging the Thiem equation: (s₁ − s₂) = Q × ln(r₂/r₁) / (2πKb). ln(60/15) = ln 4 = 1.386. Denominator = 2π × 30 × 12 = 2261.9. (s₁ − s₂) = 1500 × 1.386 / 2261.9 = 2079.4 / 2261.9 = 0.92 m.

Q.56

Irrigation & Drainage Engineering  ›  Soil-Water-Plant Relationship   |   MSQ · 2 marks

Question:

For a loam soil with a field capacity of 28% and a permanent wilting point of 12% (both on a dry-weight basis), and a bulk density of 1.4 g/cm³, which of the following statement(s) is/are correct regarding the available water?

(A) The available water content on a mass basis is 16%

(B) The available water content on a mass basis is 40%

(C) The available water depth for a 1 m root zone is approximately 224 mm

(D) The available water depth for a 1 m root zone is approximately 160 mm

Answer: AC

Explanation:

Available water content (mass basis) = FC − PWP = 28% − 12% = 16%, confirming (A) and ruling out (B). Available water depth = (FC − PWP)/100 × bulk density × root-zone depth = 0.16 × 1.4 × 1000 mm = 224 mm, confirming (C) and ruling out (D).

Q.57

Irrigation & Drainage Engineering  ›  Irrigation Water Conveyance and Application Methods   |   NAT · 2 marks

Question:

An irrigation stream of 40 litres per second is applied to a border strip and is cut off after 90 minutes. If the water infiltrated into the root zone within the field is 150 m³, calculate the application efficiency, in percent. (Round off to one decimal place.)

Answer: 68.50 to 70.50

Explanation:

Water applied = 40 L/s × (90 × 60 s) = 40 × 5400 = 216,000 L = 216 m³. Application efficiency = (water stored in root zone / water applied) × 100 = (150/216) × 100 = 69.4%.

Q.58

Irrigation & Drainage Engineering  ›  Agricultural Drainage   |   NAT · 2 marks

Question:

A subsurface drainage system is to be designed using the steady-state equation S² = 4Kh(2D + h)/q, where S is the drain spacing (m), K is the hydraulic conductivity = 0.5 m/day, h is the height of the water table above the drains at the midpoint = 0.6 m, D is the depth of the equivalent impermeable layer below the drains = 2 m, and q is the drainage coefficient = 0.01 m/day. Calculate the drain spacing S, in metres. (Round off to one decimal place.)

Answer: 23.00 to 24.00

Explanation:

S² = 4 × 0.5 × 0.6 × (2×2 + 0.6) / 0.01 = 4 × 0.5 × 0.6 × 4.6 / 0.01 = 5.52/0.01 = 552. S = √552 = 23.5 m.

Q.59

Agricultural Process Engineering  ›  Evaporation and Drying   |   NAT · 2 marks

Question:

A batch of paddy weighing 1000 kg (wet basis) with an initial moisture content of 22% (wet basis) is to be dried to a final moisture content of 13% (wet basis). Calculate the mass of water to be removed, in kg. (Round off to one decimal place.)

Answer: 101.00 to 106.00

Explanation:

Mass of dry matter (constant during drying) = 1000 × (1 − 0.22) = 780 kg. Final total mass at 13% moisture = 780/(1 − 0.13) = 780/0.87 = 896.6 kg. Water removed = 1000 − 896.6 = 103.4 kg.

Q.60

Agricultural Process Engineering  ›  Evaporation and Drying   |   NAT · 2 marks

Question:

During the constant-rate period of drying of a food product, the drying rate is 0.15 kg water/(kg dry solid·h). A sample containing 4 kg of dry solid is dried, entirely within the constant-rate period, from a moisture content of 0.80 kg water/kg dry solid to 0.50 kg water/kg dry solid. Calculate the time required, in hours. (Round off to two decimal places.)

Answer: 1.90 to 2.10

Explanation:

Water removed per kg dry solid = 0.80 − 0.50 = 0.30 kg/kg. Total water removed = 0.30 × 4 = 1.2 kg. Time = total water removed / (drying rate × dry solid mass) = 1.2/(0.15 × 4) = 1.2/0.6 = 2.00 h.

Q.61

Agricultural Process Engineering  ›  Size Reduction and Material Handling   |   MSQ · 2 marks

Question:

In the size reduction of agricultural materials, which of the following statement(s) is/are correct?

(A) Rittinger's law assumes the energy required for size reduction is proportional to the new surface area created

(B) Kick's law assumes the energy required is proportional to the square of the reduction ratio

(C) Bond's law states that the energy required is inversely proportional to the square root of the product particle size, minus that of the feed particle size

(D) Hammer mills reduce material size primarily through shearing action between two smooth rollers

Answer: AC

Explanation:

Rittinger's law states that the energy required for size reduction is directly proportional to the new surface area created, confirming (A). Kick's law actually relates energy to the logarithm of the size-reduction ratio, not its square, so (B) is incorrect. Bond's law expresses energy input as proportional to 1/√(product size) − 1/√(feed size), confirming (C). Hammer mills reduce size primarily by impact (repeated blows from rotating hammers), not by shearing between smooth rollers (which describes roller mills), so (D) is incorrect.

Q.62

Agricultural Process Engineering  ›  Size Reduction and Material Handling   |   NAT · 2 marks

Question:

A belt conveyor, 500 mm wide, moving at 1.2 m/s, carries grain forming a uniform cross-sectional area of 0.025 m² on the belt. Given the bulk density of the grain as 750 kg/m³, calculate the conveying capacity, in tonnes per hour. (Round off to one decimal place.)

Answer: 79.50 to 82.50

Explanation:

Volumetric flow rate = cross-sectional area × belt speed = 0.025 × 1.2 = 0.03 m³/s. Mass flow rate = 0.03 × 750 = 22.5 kg/s = 22.5 × 3600 = 81,000 kg/h = 81.0 t/h.

Q.63

Dairy & Food Engineering  ›  Heat and Mass Transfer   |   NAT · 2 marks

Question:

A flat slab of a food product, 4 cm thick, has a thermal conductivity of 0.5 W/(m·K). Under steady-state conditions, the temperature on one face is 60°C and on the other face is 20°C. Calculate the heat flux through the slab, in W/m². (Round off to the nearest whole number.)

Answer: 495 to 505

Explanation:

By Fourier's law of conduction: q = k × ΔT / L = 0.5 × (60−20) / 0.04 = 0.5 × 40/0.04 = 500 W/m².

Q.64

Dairy & Food Engineering  ›  Heat and Mass Transfer   |   NAT · 2 marks

Question:

1000 kg of milk is to be cooled from 35°C to 5°C in a plate heat exchanger. Taking the specific heat of milk as 3.9 kJ/(kg·K), calculate the total heat to be removed, in MJ. (Round off to the nearest whole number.)

Answer: 115 to 119

Explanation:

Heat to be removed Q = m × c × ΔT = 1000 × 3.9 × (35−5) = 1000 × 3.9 × 30 = 117,000 kJ = 117 MJ.

Q.65

Dairy & Food Engineering  ›  Preservation of Food   |   MSQ · 2 marks

Question:

Which of the following statement(s) about the thermal processing of food is/are correct?

(A) The D-value (decimal reduction time) represents the time required at a given temperature to reduce the microbial population by 90% (one log cycle)

(B) The z-value represents the temperature change required to change the D-value by one log cycle (i.e., by a factor of 10)

(C) A higher D-value indicates that the microorganism is less heat-resistant

(D) The 12D concept, commonly applied to Clostridium botulinum in low-acid canned foods, refers to a process that reduces the spore population by 12 orders of magnitude (log cycles)

Answer: ABD

Explanation:

The D-value is, by definition, the time at a fixed temperature needed to achieve a 90% (one log cycle) reduction in microbial population, confirming (A). The z-value is the temperature increase required to reduce the D-value by a factor of 10 (one log cycle), confirming (B). A higher D-value means a longer time is needed for a 90% reduction, indicating the organism is MORE heat-resistant, not less — so (C) is incorrect. The 12D concept for Clostridium botulinum specifies a thermal process that reduces the spore population by 12 orders of magnitude (log cycles), confirming (D).

# Section D — Blueprint Verification Table

This table confirms that the delivered paper conforms exactly to the mandated section, marks, and question-type blueprint.

| Section | Q.Count | 1-mk | 2-mk | Marks | MCQ | MSQ | NAT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GA — General Aptitude | 10 | 5 | 5 | 15 | 10 | 0 | 0 |
| S1 — Engineering Mathematics | 9 | 5 | 4 | 13 | 6 | 0 | 3 |
| S2 — Farm Machinery | 7 | 3 | 4 | 11 | 3 | 0 | 4 |
| S3 — Farm Power | 9 | 4 | 5 | 14 | 3 | 0 | 6 |
| S4 — Soil & Water Conservation Engineering | 10 | 4 | 6 | 16 | 4 | 0 | 6 |
| S5 — Irrigation & Drainage Engineering | 6 | 2 | 4 | 10 | 1 | 1 | 4 |
| S6 — Agricultural Process Engineering | 9 | 5 | 4 | 13 | 2 | 1 | 6 |
| S7 — Dairy & Food Engineering | 5 | 2 | 3 | 8 | 1 | 2 | 2 |
| TOTAL | 65 | 30 | 35 | 100 | 30 | 4 | 31 |

Self-Check Confirmation

- Exactly 65 questions, exactly 100 marks. ✓

- Q.1–Q.5 and Q.11–Q.35 are 1 mark; Q.6–Q.10 and Q.36–Q.65 are 2 marks. ✓

- Section marks match the mandated blueprint exactly: GA 15, S1 13, S2 11, S3 14, S4 16, S5 10, S6 13, S7 8. ✓

- Type counts match the mandated blueprint exactly: 30 MCQ, 4 MSQ, 31 NAT. ✓

- Sub-topic marks match the mandated weightage in every section. ✓

- Every NAT question states its required unit and rounding instruction, and its key gives a tolerance range. ✓

- Every numerical answer was independently derived via formula-based calculation and cross-checked. ✓

- No question duplicates a past GATE AG question; underlying concepts follow the syllabus but all scenarios, data, and required quantities are original. ✓

- Every question is solvable using only the data supplied within it. ✓