GATE 2027

Agricultural Engineering (AG)

Full-Length Mock Paper

| Paper Code | AG (Agricultural Engineering) |
| --- | --- |
| Duration | 180 minutes (3 hours) |
| Maximum Marks | 100 |
| Total Questions | 65 (Q.1 - Q.65) |
| General Aptitude (GA) | Q.1 - Q.10 (15 marks) |
| Agricultural Engineering Subject | Q.11 - Q.65 (85 marks) |

Instructions — GATE Marking Scheme

- The examination is of 180 minutes (3 hours) duration and carries a total of 100 marks.

- General Aptitude (GA): Q.1-Q.5 carry 1 mark each; Q.6-Q.10 carry 2 marks each (Total: 15 marks). GA questions are all Multiple Choice Questions (MCQ).

- Agricultural Engineering (AG) subject, including Engineering Mathematics: Q.11-Q.35 carry 1 mark each; Q.36-Q.65 carry 2 marks each (Total: 85 marks).

- Question types: MCQ (Multiple Choice Question — exactly one of four options is correct), MSQ (Multiple Select Question — one or more of four options may be correct; no partial credit), and NAT (Numerical Answer Type — the answer is a number typed in, with no options).

- Negative marking (MCQ only): 1/3 mark is deducted for a wrong answer to a 1-mark MCQ; 2/3 mark is deducted for a wrong answer to a 2-mark MCQ.

- There is NO negative marking for MSQ or NAT questions, and no partial marking for a partially correct MSQ response.

- Calculators are permitted in the actual online interface; no physical calculators or data books are required for this paper — every constant needed is provided within the question.

# Question Paper

Q.1  [MCQ, 1 mark]

The table below shows the area (in hectares) under three crops in a village over two years.

Crop        2024   2025

Wheat        120    150

Mustard       80     60

Gram          40     50

What is the percentage increase in the total cropped area from 2024 to 2025?

(A)  6.25%

(B)  7.69%

(C)  8.33%

(D)  10.00%

Q.2  [MCQ, 1 mark]

A shopkeeper marks up the price of an item by 40% above the cost price and then offers a discount of 25% on the marked price. What is his net profit or loss percentage?

(A)  5% profit

(B)  5% loss

(C)  15% profit

(D)  10% profit

Q.3  [MCQ, 1 mark]

Five years ago, the ratio of the ages of A and B was 3:4. The ratio of their present ages is 4:5. What is B's present age (in years)?

(A)  20

(B)  25

(C)  30

(D)  35

Q.4  [MCQ, 1 mark]

Select the word most nearly OPPOSITE in meaning to "PARSIMONIOUS".

(A)  Frugal

(B)  Lavish

(C)  Prudent

(D)  Austere

Q.5  [MCQ, 1 mark]

Statement: All farmers in a village who own tractors also own tube wells. Some farmers who own tube wells do not grow wheat.

Conclusion: Some tractor owners do not grow wheat.

Which one of the following is correct?

(A)  The conclusion follows definitely from the statements

(B)  The conclusion does not follow definitely from the statements

(C)  The conclusion is the logical converse of the statements

(D)  The conclusion contradicts the statements

Q.6  [MCQ, 2 marks]

[Figure description: A large cube of side 3 units is formed by stacking 27 identical unit cubes into a 3x3x3 arrangement. The entire outer surface of the large cube is painted red, and it is then broken apart into the 27 unit cubes.]

How many of the 27 small cubes have paint on exactly two faces?

(A)  8

(B)  12

(C)  6

(D)  24

Q.7  [MCQ, 2 marks]

A tank can be filled by pipe A alone in 6 hours and by pipe B alone in 8 hours. Both pipes are opened together, but pipe A is closed after 2 hours. In how many more hours will pipe B alone fill the remaining tank?

(A)  3.00 hours

(B)  3.33 hours

(C)  3.75 hours

(D)  4.00 hours

Q.8  [MCQ, 2 marks]

The average of 11 consecutive odd numbers is 61. What is the largest number among them?

(A)  69

(B)  71

(C)  73

(D)  75

Q.9  [MCQ, 2 marks]

Select the most appropriate word to fill the blank.

"Despite the ______ evidence presented by the defense, the jury remained unconvinced of the defendant's innocence."

(A)  meager

(B)  compelling

(C)  redundant

(D)  circumstantial

Q.10  [MCQ, 2 marks]

In a certain code, TRACTOR is written as USBDUPS. How is ENGINE written in the same code?

(A)  FOHJOF

(B)  FOHIOF

(C)  FPHJOF

(D)  FOHJPF

Q.11  [MCQ, 1 mark]

Evaluate: lim (x->0) [1 - cos(3x)] / x^2

(A)  2.5

(B)  3.0

(C)  4.5

(D)  9.0

Q.12  [MCQ, 1 mark]

What is the rank of the matrix [[2,4,6],[1,2,3],[3,6,9]]?

(A)  0

(B)  1

(C)  2

(D)  3

Q.13  [MCQ, 1 mark]

A 3x3 matrix A has eigenvalues 2, -1, and 3. What is det(A)?

(A)  -6

(B)  4

(C)  6

(D)  -4

Q.14  [MCQ, 1 mark]

The differential equation dy/dx + y = 0 represents a family of curves. What type of curves are these?

(A)  Straight lines

(B)  Parabolas

(C)  Exponential curves

(D)  Circles

Q.15  [NAT, 1 mark]

Solve dy/dx = 2x with the initial condition y(0) = 3. Find y(2).

(Answer in the range 6.90 to 7.10)

Q.16  [MCQ, 1 mark]

In a belt drive, the driver pulley has a diameter of 200 mm and runs at 900 rpm. The driven pulley has a diameter of 300 mm. Neglecting slip, the speed of the driven pulley is:

(A)  450 rpm

(B)  600 rpm

(C)  750 rpm

(D)  900 rpm

Q.17  [MCQ, 1 mark]

In a seed drill, which mechanism is most commonly used to meter and deliver seeds uniformly into the furrow?

(A)  Fluted feed mechanism

(B)  Venturi mechanism

(C)  Reciprocating plunger pump

(D)  Screw conveyor

Q.18  [NAT, 1 mark]

A tractor costs Rs 8,00,000 and has an expected useful life of 10 years with a salvage value of Rs 80,000. Using the straight-line method, calculate the annual depreciation (in Rs).

(Answer in the range 71900 to 72100)

Q.19  [MCQ, 1 mark]

In a tractor power transmission system, which component allows the two rear wheels to rotate at different speeds while turning?

(A)  Clutch

(B)  Differential

(C)  Final drive

(D)  Gear box

Q.20  [MCQ, 1 mark]

The cetane number of diesel fuel is a measure of its:

(A)  Anti-knock quality in SI engines

(B)  Ignition quality in CI engines

(C)  Calorific value

(D)  Viscosity

Q.21  [NAT, 1 mark]

A biogas plant produces 3 m^3 of biogas per day, with a calorific value of 20 MJ/m^3. Calculate the total energy output per day (in MJ).

(Answer in the range 59.00 to 61.00)

Q.22  [NAT, 1 mark]

A tractor develops a drawbar pull of 12 kN while travelling at 5 km/h. Calculate the drawbar power (in kW).

(Answer in the range 16.30 to 17.00)

Q.23  [MCQ, 1 mark]

In a dumpy level survey, the back sight (BS) reading taken on a benchmark of reduced level (RL) 100.000 m is 1.500 m. What is the height of the instrument (HI)?

(A)  98.500 m

(B)  100.500 m

(C)  101.500 m

(D)  102.000 m

Q.24  [MCQ, 1 mark]

Splash erosion is primarily caused by:

(A)  Wind action alone

(B)  The impact of raindrops on bare soil

(C)  Sheet flow of water

(D)  Gully formation

Q.25  [NAT, 1 mark]

The Universal Soil Loss Equation is A = R x K x LS x C x P. For a field with R = 250, K = 0.3, LS = 1.2, C = 0.5, and P = 1, calculate the annual soil loss A (in tonnes/ha/year).

(Answer in the range 44.00 to 46.00)

Q.26  [MCQ, 1 mark]

The time taken by runoff to travel from the most hydraulically remote point of a watershed to the outlet is known as:

(A)  Lag time

(B)  Time of concentration

(C)  Recession time

(D)  Detention time

Q.27  [MCQ, 1 mark]

The coefficient of storage (storativity) of a confined aquifer typically ranges between:

(A)  0.00005 to 0.005

(B)  0.05 to 0.30

(C)  0.30 to 0.50

(D)  0.50 to 1.0

Q.28  [NAT, 1 mark]

A confined aquifer has a transmissivity of 500 m^2/day and a hydraulic gradient of 0.002. Using Darcy's law, q = T x i, calculate the discharge per unit width of aquifer (in m^3/day per m width).

(Answer in the range 0.90 to 1.10)

Q.29  [MCQ, 1 mark]

In size reduction of agricultural produce, Rittinger's law states that the energy required is proportional to:

(A)  The reduction ratio

(B)  The new surface area created

(C)  The logarithm of the size reduction ratio

(D)  The volume of material processed

Q.30  [NAT, 1 mark]

A belt conveyor transports grain at a rate of 20 tonnes/hour over a horizontal distance of 50 m at a belt speed of 1.5 m/s. Calculate the mass of grain present on the belt at any instant (in kg).

(Answer in the range 180.00 to 190.00)

Q.31  [MCQ, 1 mark]

The angle of repose of a granular agricultural material is a measure of its:

(A)  Bulk density

(B)  Frictional/flow characteristics

(C)  Moisture content

(D)  Thermal conductivity

Q.32  [NAT, 1 mark]

A sample of wheat grain weighs 500 g on a wet basis. After drying, it weighs 440 g on a dry basis. Calculate the moisture content on a wet basis (in %).

(Answer in the range 11.50 to 12.50)

Q.33  [NAT, 1 mark]

Paddy is dried from 22% to 14% moisture content (wet basis). Calculate the mass of water removed per 100 kg of initial wet paddy (in kg).

(Answer in the range 9.00 to 9.60)

Q.34  [MCQ, 1 mark]

Pasteurization of milk is primarily intended to:

(A)  Sterilize the milk completely, removing all microorganisms

(B)  Destroy pathogenic microorganisms while retaining the milk's nutritional and sensory quality

(C)  Increase the fat content of the milk

(D)  Permanently remove all enzymes present in the milk

Q.35  [NAT, 1 mark]

Milk is pasteurized by the HTST method at 72 degC for 15 seconds. If the initial bacterial load is 10^6 CFU/mL and the process achieves a 4-log reduction, calculate the final bacterial load (in CFU/mL).

(Answer in the range 95.00 to 105.00)

Q.36  [MCQ, 2 marks]

The function f(x) = x^3 - 6x^2 + 9x + 2 has a local maximum at x = ____.

(A)  0

(B)  1

(C)  2

(D)  3

Q.37  [NAT, 2 marks]

Evaluate the definite integral: Integral from x = 0 to x = 2 of (3x^2 + 2x) dx.

(Answer in the range 11.90 to 12.10)

Q.38  [NAT, 2 marks]

Find the value of lambda for which the system of equations x + 2y = 3 and 2x + lambda*y = 6 has infinitely many solutions.

(Answer in the range 3.90 to 4.10)

Q.39  [MCQ, 2 marks]

A box contains 4 red and 6 black balls. Two balls are drawn at random, without replacement. What is the probability that both balls drawn are red?

(A)  1/15

(B)  2/15

(C)  1/5

(D)  4/15

Q.40  [NAT, 2 marks]

A tillage tool has a working width of 0.3 m and operates at a depth of 0.15 m in a soil with a specific draft resistance of 40 kN/m^2. If the tool moves forward at 1.5 m/s, calculate the power required to pull the tool (in kW).

(Answer in the range 2.60 to 2.80)

Q.41  [MCQ, 2 marks]

Which of the following factors does NOT directly affect the draft required by a tillage implement?

(A)  Speed of operation

(B)  Soil moisture content

(C)  Depth of operation

(D)  Color of the implement

Q.42  [NAT, 2 marks]

A shaft transmits 15 kW of power at 300 rpm. Calculate the torque transmitted by the shaft (in N.m).

(Answer in the range 475.00 to 480.00)

Q.43  [NAT, 2 marks]

A boom sprayer has 12 nozzles spaced 0.5 m apart and travels at 4 km/h. Each nozzle discharges 0.8 L/min. Calculate the application rate (in L/ha).

(Answer in the range 235.00 to 245.00)

Q.44  [NAT, 2 marks]

A solar photovoltaic panel of area 2 m^2 receives solar radiation at 800 W/m^2. If the panel efficiency is 15%, calculate the electrical power output (in W).

(Answer in the range 235.00 to 245.00)

Q.45  [NAT, 2 marks]

A tractor weighing 18 kN has a coefficient of traction of 0.6 on a given soil. If the total motion resistance is 3 kN, calculate the maximum drawbar pull available (in kN).

(Answer in the range 7.60 to 8.00)

Q.46  [NAT, 2 marks]

A four-stroke single-cylinder diesel engine has a bore of 100 mm and a stroke of 120 mm. Calculate the swept volume of the cylinder (in cm^3).

(Answer in the range 935.00 to 950.00)

Q.47  [NAT, 2 marks]

An Otto cycle engine has a compression ratio of 8. Calculate the theoretical air-standard efficiency (in %), taking gamma = 1.4.

(Answer in the range 55.50 to 57.50)

Q.48  [MCQ, 2 marks]

Which of the following is the recommended safe practice while operating a tractor-mounted power take-off (PTO) shaft?

(A)  Operating without a PTO shield to improve visibility

(B)  Wearing loose clothing for comfort while working near the PTO

(C)  Ensuring the PTO shield is in place and never stepping over a rotating shaft

(D)  Disengaging the master shield only during operation and refitting it afterward

Q.49  [NAT, 2 marks]

Water flows through a horizontal pipe that tapers from a diameter of 200 mm to 100 mm. If the velocity in the larger section is 1 m/s, calculate the velocity in the smaller section (in m/s).

(Answer in the range 3.90 to 4.10)

Q.50  [NAT, 2 marks]

Water flows through a venturimeter with an inlet diameter of 300 mm and a throat diameter of 150 mm. The inlet velocity is 2 m/s. Calculate the velocity-head difference (v2^2 - v1^2)/2g between the throat and the inlet (in m). Take g = 9.81 m/s^2.

(Answer in the range 2.95 to 3.15)

Q.51  [NAT, 2 marks]

A rectangular check-dam spillway must pass a peak discharge of 2.5 m^3/s. Using the weir formula Q = 1.84 x L x H^1.5 with head H = 0.5 m, calculate the required length of the weir L (in m).

(Answer in the range 3.70 to 4.00)

Q.52  [NAT, 2 marks]

A watershed has an area of 25 km^2. During a storm, the total rainfall was 60 mm, of which the runoff coefficient is 0.35. Calculate the total volume of direct runoff (in m^3).

(Answer in the range 520000 to 530000)

Q.53  [NAT, 2 marks]

A watershed of area 180 km^2 has a time-to-peak of 10 hours for its unit hydrograph. Using the SCS synthetic unit hydrograph relation Qp = 2.08 x A / Tp (Qp in m^3/s per cm of runoff, A in km^2, Tp in hours), calculate the peak discharge of the unit hydrograph (in m^3/s).

(Answer in the range 36.50 to 38.50)

Q.54  [MCQ, 2 marks]

A saturated soil sample has a void ratio of 0.6 and specific gravity of solids 2.65. Taking gamma_w = 9.81 kN/m^3, the saturated unit weight of the soil is closest to:

(A)  16.5 kN/m^3

(B)  18.2 kN/m^3

(C)  19.9 kN/m^3

(D)  21.4 kN/m^3

Q.55  [NAT, 2 marks]

A crop field has a field capacity of 28% and a permanent wilting point of 12% (by weight). The bulk density of the soil is 1.4 g/cm^3, and the effective root-zone depth is 60 cm. Calculate the total available water in the root zone (in mm).

(Answer in the range 130.00 to 140.00)

Q.56  [NAT, 2 marks]

A lined irrigation channel with a wetted perimeter of 3 m conveys water over a length of 5 km. The seepage loss rate is 2.5 m^3/s per million m^2 of wetted area. Calculate the total seepage loss over the channel length (in m^3/s).

(Answer in the range 0.03 to 0.05)

Q.57  [NAT, 2 marks]

A tube well pumps water at 30 L/s against a total head of 25 m. If the pump efficiency is 70%, calculate the power input required (in kW). Take rho*g = 9.81 kN/m^3.

(Answer in the range 10.20 to 10.80)

Q.58  [MSQ, 2 marks]

Which of the following statements regarding subsurface agricultural drainage are correct?

(A)  Drain spacing decreases as soil hydraulic conductivity decreases, for a given drainage coefficient

(B)  Deeper drains generally allow wider spacing between them

(C)  The drainage coefficient represents the rate of water table drawdown (or discharge) required to remove excess water in a specified time

(D)  Perforated PVC pipes are never suitable for subsurface drains

Q.59  [NAT, 2 marks]

A single-effect evaporator concentrates milk from 10% total solids to 40% total solids. If the feed rate is 2000 kg/h, calculate the rate of water evaporation (in kg/h).

(Answer in the range 1480.00 to 1520.00)

Q.60  [NAT, 2 marks]

In a cross-flow grain dryer, drying air enters at 60 degC and leaves at 35 degC. The specific heat of the moist air is 1.02 kJ/(kg.K), and its mass flow rate is 5 kg/s. Calculate the rate of heat given up by the air (in kW).

(Answer in the range 125.00 to 130.00)

Q.61  [NAT, 2 marks]

Using Bond's law, the energy required to reduce a material's particle size from 10 mm to 2 mm is 15 kJ/kg. Using the same Bond's work index, calculate the energy required (in kJ/kg) to further reduce the size from 2 mm to 1 mm.

(Answer in the range 10.50 to 12.00)

Q.62  [MSQ, 2 marks]

Which of the following factors contribute to increased spoilage of grain during storage?

(A)  High moisture content of the grain

(B)  High storage temperature

(C)  Presence of insect infestation

(D)  Low relative humidity of the storage environment

Q.63  [MSQ, 2 marks]

In the context of heat transfer during food processing, which of the following statements are correct?

(A)  Thermal conductivity of food materials generally increases with increasing moisture content

(B)  The convective heat transfer coefficient depends on the fluid flow characteristics around the food surface

(C)  Fourier's law describes heat transfer by convection

(D)  The Biot number is used to determine whether internal (conductive) or surface (convective) resistance controls the heat transfer process

Q.64  [NAT, 2 marks]

A food slab of thickness 4 cm has one face maintained at 80 degC and the other at 20 degC at steady state. If the thermal conductivity of the food material is 0.5 W/(m.K), calculate the heat flux through the slab (in W/m^2).

(Answer in the range 740.00 to 760.00)

Q.65  [MSQ, 2 marks]

Which of the following are recognized methods of food preservation based primarily on moisture reduction or control?

(A)  Sun drying

(B)  Freeze drying (lyophilization)

(C)  Osmotic dehydration

(D)  Ultra-high temperature (UHT) processing

# Answer Key

| Q.No. | Section | Sub-topic | Type | Marks | Answer |
| --- | --- | --- | --- | --- | --- |
| 1 | General Aptitude | Data Interpretation | MCQ | 1 | C |
| 2 | General Aptitude | Quantitative Aptitude | MCQ | 1 | A |
| 3 | General Aptitude | Quantitative Aptitude | MCQ | 1 | B |
| 4 | General Aptitude | Verbal Aptitude | MCQ | 1 | B |
| 5 | General Aptitude | Analytical Aptitude | MCQ | 1 | B |
| 6 | General Aptitude | Spatial Aptitude | MCQ | 2 | B |
| 7 | General Aptitude | Quantitative Aptitude | MCQ | 2 | B |
| 8 | General Aptitude | Quantitative Aptitude | MCQ | 2 | B |
| 9 | General Aptitude | Verbal Aptitude | MCQ | 2 | B |
| 10 | General Aptitude | Analytical Aptitude | MCQ | 2 | A |
| 11 | Engineering Mathematics | Calculus | MCQ | 1 | C |
| 12 | Engineering Mathematics | Linear Algebra | MCQ | 1 | B |
| 13 | Engineering Mathematics | Linear Algebra | MCQ | 1 | A |
| 14 | Engineering Mathematics | Differential Equations | MCQ | 1 | C |
| 15 | Engineering Mathematics | Differential Equations | NAT | 1 | 7.00 (accepted range: 6.90 to 7.10) |
| 16 | Farm Machinery | Machine Design (machine elements) | MCQ | 1 | B |
| 17 | Farm Machinery | Sowing/planting/fertilizer application equipment | MCQ | 1 | A |
| 18 | Farm Machinery | Cost analysis of implements and tractors | NAT | 1 | 72000.00 (accepted range: 71900 to 72100) |
| 19 | Farm Power | Power transmission (clutch, gear box, differential, final drive, brakes) | MCQ | 1 | B |
| 20 | Farm Power | Fuels, combustion and lubricants | MCQ | 1 | B |
| 21 | Farm Power | Sources of Power (wind, solar, biomass, bio-fuels, human, animal) | NAT | 1 | 60.00 (accepted range: 59.00 to 61.00) |
| 22 | Farm Power | Tractors & Power Tillers (traction theory) | NAT | 1 | 16.67 (accepted range: 16.30 to 17.00) |
| 23 | Soil & Water Conservation Engineering | Surveying and Leveling | MCQ | 1 | C |
| 24 | Soil & Water Conservation Engineering | Soil and Water Erosion | MCQ | 1 | B |
| 25 | Soil & Water Conservation Engineering | Soil and Water Erosion | NAT | 1 | 45.00 (accepted range: 44.00 to 46.00) |
| 26 | Soil & Water Conservation Engineering | Hydrology | MCQ | 1 | B |
| 27 | Irrigation & Drainage Engineering | Groundwater Hydrology | MCQ | 1 | A |
| 28 | Irrigation & Drainage Engineering | Groundwater Hydrology | NAT | 1 | 1.00 (accepted range: 0.90 to 1.10) |
| 29 | Agricultural Process Engineering | Size Reduction and Material Handling | MCQ | 1 | B |
| 30 | Agricultural Process Engineering | Size Reduction and Material Handling | NAT | 1 | 185.20 (accepted range: 180.00 to 190.00) |
| 31 | Agricultural Process Engineering | Engineering properties of agricultural produce | MCQ | 1 | B |
| 32 | Agricultural Process Engineering | Engineering properties of agricultural produce | NAT | 1 | 12.00 (accepted range: 11.50 to 12.50) |
| 33 | Agricultural Process Engineering | Evaporation and Drying | NAT | 1 | 9.30 (accepted range: 9.00 to 9.60) |
| 34 | Dairy & Food Engineering | Preservation of Food | MCQ | 1 | B |
| 35 | Dairy & Food Engineering | Preservation of Food | NAT | 1 | 100.00 (accepted range: 95.00 to 105.00) |
| 36 | Engineering Mathematics | Calculus | MCQ | 2 | B |
| 37 | Engineering Mathematics | Calculus | NAT | 2 | 12.00 (accepted range: 11.90 to 12.10) |
| 38 | Engineering Mathematics | Linear Algebra | NAT | 2 | 4.00 (accepted range: 3.90 to 4.10) |
| 39 | Engineering Mathematics | Probability and Statistics | MCQ | 2 | B |
| 40 | Farm Machinery | Farm Machinery (tillage forces) | NAT | 2 | 2.70 (accepted range: 2.60 to 2.80) |
| 41 | Farm Machinery | Farm Machinery (tillage forces) | MCQ | 2 | D |
| 42 | Farm Machinery | Machine Design (machine elements) | NAT | 2 | 477.46 (accepted range: 475.00 to 480.00) |
| 43 | Farm Machinery | Equipment for spraying | NAT | 2 | 240.00 (accepted range: 235.00 to 245.00) |
| 44 | Farm Power | Sources of Power (wind, solar, biomass, bio-fuels, human, animal) | NAT | 2 | 240.00 (accepted range: 235.00 to 245.00) |
| 45 | Farm Power | Tractors & Power Tillers (traction theory) | NAT | 2 | 7.80 (accepted range: 7.60 to 8.00) |
| 46 | Farm Power | Farm Power: engine components & I.C. engine systems | NAT | 2 | 942.48 (accepted range: 935.00 to 950.00) |
| 47 | Farm Power | Thermodynamic principles & I.C. engine cycles | NAT | 2 | 56.47 (accepted range: 55.50 to 57.50) |
| 48 | Farm Power | Human engineering, ergonomics and safety | MCQ | 2 | C |
| 49 | Soil & Water Conservation Engineering | Fluid Mechanics | NAT | 2 | 4.00 (accepted range: 3.90 to 4.10) |
| 50 | Soil & Water Conservation Engineering | Fluid Mechanics | NAT | 2 | 3.06 (accepted range: 2.95 to 3.15) |
| 51 | Soil & Water Conservation Engineering | Soil and Water Erosion | NAT | 2 | 3.84 (accepted range: 3.70 to 4.00) |
| 52 | Soil & Water Conservation Engineering | Hydrology | NAT | 2 | 525000.00 (accepted range: 520000 to 530000) |
| 53 | Soil & Water Conservation Engineering | Hydrology | NAT | 2 | 37.44 (accepted range: 36.50 to 38.50) |
| 54 | Soil & Water Conservation Engineering | Soil Mechanics | MCQ | 2 | C |
| 55 | Irrigation & Drainage Engineering | Soil-Water-Plant Relationship | NAT | 2 | 134.40 (accepted range: 130.00 to 140.00) |
| 56 | Irrigation & Drainage Engineering | Irrigation Water Conveyance and Application Methods | NAT | 2 | 0.04 (accepted range: 0.03 to 0.05) |
| 57 | Irrigation & Drainage Engineering | Wells and Pumps | NAT | 2 | 10.51 (accepted range: 10.20 to 10.80) |
| 58 | Irrigation & Drainage Engineering | Agricultural Drainage | MSQ | 2 | A, B, C |
| 59 | Agricultural Process Engineering | Evaporation and Drying | NAT | 2 | 1500.00 (accepted range: 1480.00 to 1520.00) |
| 60 | Agricultural Process Engineering | Evaporation and Drying | NAT | 2 | 127.50 (accepted range: 125.00 to 130.00) |
| 61 | Agricultural Process Engineering | Size Reduction and Material Handling | NAT | 2 | 11.24 (accepted range: 10.50 to 12.00) |
| 62 | Agricultural Process Engineering | Storage Systems | MSQ | 2 | A, B, C |
| 63 | Dairy & Food Engineering | Heat and Mass Transfer | MSQ | 2 | A, B, D |
| 64 | Dairy & Food Engineering | Heat and Mass Transfer | NAT | 2 | 750.00 (accepted range: 740.00 to 760.00) |
| 65 | Dairy & Food Engineering | Preservation of Food | MSQ | 2 | A, B, C |

# Detailed Solutions

Q.1

Section: General Aptitude   Topic: Data Interpretation

Type & Marks: MCQ, 1 mark

Question:

The table below shows the area (in hectares) under three crops in a village over two years.

Crop        2024   2025

Wheat        120    150

Mustard       80     60

Gram          40     50

What is the percentage increase in the total cropped area from 2024 to 2025?

(A)  6.25%

(B)  7.69%

(C)  8.33%

(D)  10.00%

Answer: C

Detailed Explanation:

Total area in 2024 = 120 + 80 + 40 = 240 ha. Total area in 2025 = 150 + 60 + 50 = 260 ha.

Increase = 260 - 240 = 20 ha. Percentage increase = (20/240) x 100 = 8.33%.

(A) 6.25% results from wrongly dividing 15 by 240. (B) 7.69% results from dividing the increase by the 2025 total (20/260) instead of the base-year total. (D) 10.00% is a rounding/guess error.

Q.2

Section: General Aptitude   Topic: Quantitative Aptitude

Type & Marks: MCQ, 1 mark

Question:

A shopkeeper marks up the price of an item by 40% above the cost price and then offers a discount of 25% on the marked price. What is his net profit or loss percentage?

(A)  5% profit

(B)  5% loss

(C)  15% profit

(D)  10% profit

Answer: A

Detailed Explanation:

Let cost price CP = 100. Marked price MP = 140. Selling price SP = 140 x 0.75 = 105.

Net result = SP - CP = 5, i.e., 5% profit.

(B) 5% loss reverses the sign. (C) 15% profit ignores the discount entirely. (D) 10% profit results from applying only part of the discount.

Q.3

Section: General Aptitude   Topic: Quantitative Aptitude

Type & Marks: MCQ, 1 mark

Question:

Five years ago, the ratio of the ages of A and B was 3:4. The ratio of their present ages is 4:5. What is B's present age (in years)?

(A)  20

(B)  25

(C)  30

(D)  35

Answer: B

Detailed Explanation:

Let ages 5 years ago be 3x and 4x. Present ages: (3x+5) and (4x+5).

Given (3x+5)/(4x+5) = 4/5 => 15x + 25 = 16x + 20 => x = 5.

B's present age = 4x + 5 = 25.

(A) 20 is B's age 5 years ago. (C) 30 and (D) 35 come from mis-solving the linear equation with a sign or cross-multiplication slip.

Q.4

Section: General Aptitude   Topic: Verbal Aptitude

Type & Marks: MCQ, 1 mark

Question:

Select the word most nearly OPPOSITE in meaning to "PARSIMONIOUS".

(A)  Frugal

(B)  Lavish

(C)  Prudent

(D)  Austere

Answer: B

Detailed Explanation:

"Parsimonious" means excessively unwilling to spend money; stingy. Its opposite is "lavish" (spending or giving generously).

(A) Frugal, (C) Prudent, and (D) Austere are close synonyms of "parsimonious," not antonyms.

Q.5

Section: General Aptitude   Topic: Analytical Aptitude

Type & Marks: MCQ, 1 mark

Question:

Statement: All farmers in a village who own tractors also own tube wells. Some farmers who own tube wells do not grow wheat.

Conclusion: Some tractor owners do not grow wheat.

Which one of the following is correct?

(A)  The conclusion follows definitely from the statements

(B)  The conclusion does not follow definitely from the statements

(C)  The conclusion is the logical converse of the statements

(D)  The conclusion contradicts the statements

Answer: B

Detailed Explanation:

Tractor owners are a subset of tube-well owners, but the tube-well owners who do not grow wheat need not include any tractor owner — they could all lie in the non-tractor-owning part of the tube-well-owning set. Hence the conclusion is not a definite (necessary) consequence.

(A) is wrong because the sets are not shown to overlap in that way. (C) misuses the term "converse." (D) is wrong since the conclusion is merely unproven, not contradictory.

Q.6

Section: General Aptitude   Topic: Spatial Aptitude

Type & Marks: MCQ, 2 marks

Question:

[Figure description: A large cube of side 3 units is formed by stacking 27 identical unit cubes into a 3x3x3 arrangement. The entire outer surface of the large cube is painted red, and it is then broken apart into the 27 unit cubes.]

How many of the 27 small cubes have paint on exactly two faces?

(A)  8

(B)  12

(C)  6

(D)  24

Answer: B

Detailed Explanation:

In an n x n x n cube (n=3), cubes with paint on exactly 2 faces lie on the edges but are not corners: count = 12 edges x (n-2) = 12 x 1 = 12.

(A) 8 is the corner-cube count (3 faces painted). (C) 6 is the face-centre count (1 face painted). (D) 24 double-counts by including all edge positions without excluding corners.

Q.7

Section: General Aptitude   Topic: Quantitative Aptitude

Type & Marks: MCQ, 2 marks

Question:

A tank can be filled by pipe A alone in 6 hours and by pipe B alone in 8 hours. Both pipes are opened together, but pipe A is closed after 2 hours. In how many more hours will pipe B alone fill the remaining tank?

(A)  3.00 hours

(B)  3.33 hours

(C)  3.75 hours

(D)  4.00 hours

Answer: B

Detailed Explanation:

Combined rate for 2 hours = 2 x (1/6 + 1/8) = 2 x (7/24) = 7/12 of the tank filled.

Remaining = 1 - 7/12 = 5/12.

Time for B alone to finish = (5/12) / (1/8) = 5/12 x 8 = 10/3 = 3.33 hours.

(A) 3.00 h ignores pipe A's contribution during the first 2 hours. (C) and (D) come from arithmetic slips in the fraction subtraction.

Q.8

Section: General Aptitude   Topic: Quantitative Aptitude

Type & Marks: MCQ, 2 marks

Question:

The average of 11 consecutive odd numbers is 61. What is the largest number among them?

(A)  69

(B)  71

(C)  73

(D)  75

Answer: B

Detailed Explanation:

For 11 consecutive odd numbers in arithmetic progression, the average equals the 6th (middle) term = 61.

The numbers run from 61 - 2x5 = 51 to 61 + 2x5 = 71. Largest = 71.

(A) 69 is the 10th term, not the largest. (C) 73 and (D) 75 overshoot by assuming a wrong middle position.

Q.9

Section: General Aptitude   Topic: Verbal Aptitude

Type & Marks: MCQ, 2 marks

Question:

Select the most appropriate word to fill the blank.

"Despite the ______ evidence presented by the defense, the jury remained unconvinced of the defendant's innocence."

(A)  meager

(B)  compelling

(C)  redundant

(D)  circumstantial

Answer: B

Detailed Explanation:

"Despite" signals a contrast between a strong cause and an unexpected effect. Evidence that was "compelling" (strong, persuasive) yet still failed to convince the jury creates that contrast.

(A) "meager" (weak) evidence failing to convince is unsurprising, so "despite" would be an odd choice. (C) and (D) do not create the required contrast as naturally.

Q.10

Section: General Aptitude   Topic: Analytical Aptitude

Type & Marks: MCQ, 2 marks

Question:

In a certain code, TRACTOR is written as USBDUPS. How is ENGINE written in the same code?

(A)  FOHJOF

(B)  FOHIOF

(C)  FPHJOF

(D)  FOHJPF

Answer: A

Detailed Explanation:

Compare TRACTOR -> USBDUPS letter by letter: T->U, R->S, A->B, C->D, T->U, O->P, R->S — each letter is shifted forward by exactly one position in the alphabet.

Applying the same shift to ENGINE: E->F, N->O, G->H, I->J, N->O, E->F, giving FOHJOF.

(B), (C), and (D) each contain one letter shifted incorrectly.

Q.11

Section: Engineering Mathematics   Topic: Calculus

Type & Marks: MCQ, 1 mark

Question:

Evaluate: lim (x->0) [1 - cos(3x)] / x^2

(A)  2.5

(B)  3.0

(C)  4.5

(D)  9.0

Answer: C

Detailed Explanation:

Using 1 - cos(theta) ~ theta^2/2 for small theta, with theta = 3x:

1 - cos(3x) ~ (3x)^2 / 2 = 9x^2/2.

Limit = (9x^2/2) / x^2 = 4.5.

(A), (B), (D) result from using the wrong coefficient (e.g., forgetting to square the 3, or forgetting the factor of 1/2).

Q.12

Section: Engineering Mathematics   Topic: Linear Algebra

Type & Marks: MCQ, 1 mark

Question:

What is the rank of the matrix [[2,4,6],[1,2,3],[3,6,9]]?

(A)  0

(B)  1

(C)  2

(D)  3

Answer: B

Detailed Explanation:

Row 1 = 2 x Row 2, and Row 3 = 3 x Row 2. All three rows are scalar multiples of [1,2,3], so only one row is linearly independent. Rank = 1.

(A) 0 would mean the zero matrix. (C) and (D) overestimate the number of independent rows.

Q.13

Section: Engineering Mathematics   Topic: Linear Algebra

Type & Marks: MCQ, 1 mark

Question:

A 3x3 matrix A has eigenvalues 2, -1, and 3. What is det(A)?

(A)  -6

(B)  4

(C)  6

(D)  -4

Answer: A

Detailed Explanation:

The determinant of a square matrix equals the product of its eigenvalues: det(A) = 2 x (-1) x 3 = -6.

(B), (C), and (D) arise from sign errors or from summing instead of multiplying the eigenvalues.

Q.14

Section: Engineering Mathematics   Topic: Differential Equations

Type & Marks: MCQ, 1 mark

Question:

The differential equation dy/dx + y = 0 represents a family of curves. What type of curves are these?

(A)  Straight lines

(B)  Parabolas

(C)  Exponential curves

(D)  Circles

Answer: C

Detailed Explanation:

Separating variables: dy/y = -dx, so ln(y) = -x + C, giving y = C1 x e^(-x), which is a family of exponential decay curves.

(A), (B), (D) do not satisfy the given first-order linear ODE.

Q.15

Section: Engineering Mathematics   Topic: Differential Equations

Type & Marks: NAT, 1 mark

Question:

Solve dy/dx = 2x with the initial condition y(0) = 3. Find y(2).

(Answer in the range 6.90 to 7.10)

Answer: 7.00 (accepted range: 6.90 to 7.10)

Detailed Explanation:

Integrating: y = x^2 + C. Using y(0) = 3 gives C = 3, so y = x^2 + 3.

At x = 2: y(2) = 4 + 3 = 7.00.

Q.16

Section: Farm Machinery   Topic: Machine Design (machine elements)

Type & Marks: MCQ, 1 mark

Question:

In a belt drive, the driver pulley has a diameter of 200 mm and runs at 900 rpm. The driven pulley has a diameter of 300 mm. Neglecting slip, the speed of the driven pulley is:

(A)  450 rpm

(B)  600 rpm

(C)  750 rpm

(D)  900 rpm

Answer: B

Detailed Explanation:

For a belt drive neglecting slip: N2 = N1 x D1/D2 = 900 x 200/300 = 600 rpm.

(A), (C), (D) result from inverting the diameter ratio or from arithmetic slips.

Q.17

Section: Farm Machinery   Topic: Sowing/planting/fertilizer application equipment

Type & Marks: MCQ, 1 mark

Question:

In a seed drill, which mechanism is most commonly used to meter and deliver seeds uniformly into the furrow?

(A)  Fluted feed mechanism

(B)  Venturi mechanism

(C)  Reciprocating plunger pump

(D)  Screw conveyor

Answer: A

Detailed Explanation:

The fluted feed roller (fluted feed mechanism) is the standard seed-metering device used in conventional seed drills, as its effective fluted length can be varied to control the seed rate.

(B) and (C) are principles used in liquid/chemical metering, not seed metering. (D) is used mainly for bulk material transport, not precision seed metering.

Q.18

Section: Farm Machinery   Topic: Cost analysis of implements and tractors

Type & Marks: NAT, 1 mark

Question:

A tractor costs Rs 8,00,000 and has an expected useful life of 10 years with a salvage value of Rs 80,000. Using the straight-line method, calculate the annual depreciation (in Rs).

(Answer in the range 71900 to 72100)

Answer: 72000.00 (accepted range: 71900 to 72100)

Detailed Explanation:

Straight-line depreciation = (Purchase price - Salvage value) / Useful life = (800000 - 80000) / 10 = 720000/10 = 72000.00 Rs/year.

Q.19

Section: Farm Power   Topic: Power transmission (clutch, gear box, differential, final drive, brakes)

Type & Marks: MCQ, 1 mark

Question:

In a tractor power transmission system, which component allows the two rear wheels to rotate at different speeds while turning?

(A)  Clutch

(B)  Differential

(C)  Final drive

(D)  Gear box

Answer: B

Detailed Explanation:

The differential permits the outer wheel to rotate faster than the inner wheel during a turn while still transmitting equal torque, preventing wheel scuffing.

(A) engages/disengages the engine from the transmission. (C) provides the final speed reduction/torque increase. (D) provides discrete speed/torque ratios.

Q.20

Section: Farm Power   Topic: Fuels, combustion and lubricants

Type & Marks: MCQ, 1 mark

Question:

The cetane number of diesel fuel is a measure of its:

(A)  Anti-knock quality in SI engines

(B)  Ignition quality in CI engines

(C)  Calorific value

(D)  Viscosity

Answer: B

Detailed Explanation:

Cetane number rates the ignition delay/self-ignition quality of diesel fuel in compression-ignition (CI) engines; a higher cetane number means shorter ignition delay.

(A) describes octane number (for SI/petrol engines). (C) and (D) are separate fuel properties unrelated to cetane number.

Q.21

Section: Farm Power   Topic: Sources of Power (wind, solar, biomass, bio-fuels, human, animal)

Type & Marks: NAT, 1 mark

Question:

A biogas plant produces 3 m^3 of biogas per day, with a calorific value of 20 MJ/m^3. Calculate the total energy output per day (in MJ).

(Answer in the range 59.00 to 61.00)

Answer: 60.00 (accepted range: 59.00 to 61.00)

Detailed Explanation:

Total energy = Volume x Calorific value = 3 m^3 x 20 MJ/m^3 = 60.00 MJ.

Q.22

Section: Farm Power   Topic: Tractors & Power Tillers (traction theory)

Type & Marks: NAT, 1 mark

Question:

A tractor develops a drawbar pull of 12 kN while travelling at 5 km/h. Calculate the drawbar power (in kW).

(Answer in the range 16.30 to 17.00)

Answer: 16.67 (accepted range: 16.30 to 17.00)

Detailed Explanation:

Speed = 5 km/h = 5000/3600 = 1.389 m/s.

Drawbar power = Pull x Speed = 12 kN x 1.389 m/s = 16.67 kW.

Q.23

Section: Soil & Water Conservation Engineering   Topic: Surveying and Leveling

Type & Marks: MCQ, 1 mark

Question:

In a dumpy level survey, the back sight (BS) reading taken on a benchmark of reduced level (RL) 100.000 m is 1.500 m. What is the height of the instrument (HI)?

(A)  98.500 m

(B)  100.500 m

(C)  101.500 m

(D)  102.000 m

Answer: C

Detailed Explanation:

Height of instrument HI = RL of benchmark + Back sight = 100.000 + 1.500 = 101.500 m.

(A) subtracts instead of adding the back sight. (B) and (D) result from arithmetic slips.

Q.24

Section: Soil & Water Conservation Engineering   Topic: Soil and Water Erosion

Type & Marks: MCQ, 1 mark

Question:

Splash erosion is primarily caused by:

(A)  Wind action alone

(B)  The impact of raindrops on bare soil

(C)  Sheet flow of water

(D)  Gully formation

Answer: B

Detailed Explanation:

Splash (rainsplash) erosion results from the kinetic energy of raindrops striking bare soil, detaching soil particles.

(A) describes wind erosion. (C) describes sheet erosion, a later stage after detachment. (D) describes a more advanced, concentrated form of erosion.

Q.25

Section: Soil & Water Conservation Engineering   Topic: Soil and Water Erosion

Type & Marks: NAT, 1 mark

Question:

The Universal Soil Loss Equation is A = R x K x LS x C x P. For a field with R = 250, K = 0.3, LS = 1.2, C = 0.5, and P = 1, calculate the annual soil loss A (in tonnes/ha/year).

(Answer in the range 44.00 to 46.00)

Answer: 45.00 (accepted range: 44.00 to 46.00)

Detailed Explanation:

A = 250 x 0.3 x 1.2 x 0.5 x 1 = 250 x 0.18 = 45.00 tonnes/ha/year.

Q.26

Section: Soil & Water Conservation Engineering   Topic: Hydrology

Type & Marks: MCQ, 1 mark

Question:

The time taken by runoff to travel from the most hydraulically remote point of a watershed to the outlet is known as:

(A)  Lag time

(B)  Time of concentration

(C)  Recession time

(D)  Detention time

Answer: B

Detailed Explanation:

Time of concentration is defined precisely as the travel time from the hydraulically most distant point in the watershed to the outlet.

(A) Lag time is measured from the centroid of rainfall excess to the peak of the hydrograph, a related but distinct concept. (C) and (D) refer to other parts of the hydrograph recession/storage process.

Q.27

Section: Irrigation & Drainage Engineering   Topic: Groundwater Hydrology

Type & Marks: MCQ, 1 mark

Question:

The coefficient of storage (storativity) of a confined aquifer typically ranges between:

(A)  0.00005 to 0.005

(B)  0.05 to 0.30

(C)  0.30 to 0.50

(D)  0.50 to 1.0

Answer: A

Detailed Explanation:

Confined aquifers release water mainly by compression of the aquifer matrix and expansion of water, giving very low storativity values, typically 5x10^-5 to 5x10^-3.

(B), (C), (D) are typical of the specific yield of unconfined (water-table) aquifers, which release water by actual dewatering of pores.

Q.28

Section: Irrigation & Drainage Engineering   Topic: Groundwater Hydrology

Type & Marks: NAT, 1 mark

Question:

A confined aquifer has a transmissivity of 500 m^2/day and a hydraulic gradient of 0.002. Using Darcy's law, q = T x i, calculate the discharge per unit width of aquifer (in m^3/day per m width).

(Answer in the range 0.90 to 1.10)

Answer: 1.00 (accepted range: 0.90 to 1.10)

Detailed Explanation:

q = T x i = 500 x 0.002 = 1.00 m^3/day per metre width.

Q.29

Section: Agricultural Process Engineering   Topic: Size Reduction and Material Handling

Type & Marks: MCQ, 1 mark

Question:

In size reduction of agricultural produce, Rittinger's law states that the energy required is proportional to:

(A)  The reduction ratio

(B)  The new surface area created

(C)  The logarithm of the size reduction ratio

(D)  The volume of material processed

Answer: B

Detailed Explanation:

Rittinger's law states that the energy required for size reduction is proportional to the new surface area created (i.e., proportional to 1/Dfinal - 1/Dinitial).

(A) describes Kick's-law-type reasoning loosely, (C) describes Kick's law exactly, and (D) is not a recognized size-reduction law.

Q.30

Section: Agricultural Process Engineering   Topic: Size Reduction and Material Handling

Type & Marks: NAT, 1 mark

Question:

A belt conveyor transports grain at a rate of 20 tonnes/hour over a horizontal distance of 50 m at a belt speed of 1.5 m/s. Calculate the mass of grain present on the belt at any instant (in kg).

(Answer in the range 180.00 to 190.00)

Answer: 185.20 (accepted range: 180.00 to 190.00)

Detailed Explanation:

Mass flow rate = 20000 kg/h = 20000/3600 = 5.556 kg/s.

Time for material to travel the 50 m belt = 50/1.5 = 33.33 s.

Mass on belt at any instant = mass flow rate x travel time = 5.556 x 33.33 = 185.20 kg.

Q.31

Section: Agricultural Process Engineering   Topic: Engineering properties of agricultural produce

Type & Marks: MCQ, 1 mark

Question:

The angle of repose of a granular agricultural material is a measure of its:

(A)  Bulk density

(B)  Frictional/flow characteristics

(C)  Moisture content

(D)  Thermal conductivity

Answer: B

Detailed Explanation:

The angle of repose is the steepest angle at which a heap of granular material remains stable without slumping; it directly reflects the internal friction and flow behaviour of the material.

(A), (C), (D) are separate engineering properties not directly measured by angle of repose (though moisture content can indirectly influence it).

Q.32

Section: Agricultural Process Engineering   Topic: Engineering properties of agricultural produce

Type & Marks: NAT, 1 mark

Question:

A sample of wheat grain weighs 500 g on a wet basis. After drying, it weighs 440 g on a dry basis. Calculate the moisture content on a wet basis (in %).

(Answer in the range 11.50 to 12.50)

Answer: 12.00 (accepted range: 11.50 to 12.50)

Detailed Explanation:

MC (wet basis) = (Wet mass - Dry mass)/Wet mass x 100 = (500 - 440)/500 x 100 = 60/500 x 100 = 12.00%.

Q.33

Section: Agricultural Process Engineering   Topic: Evaporation and Drying

Type & Marks: NAT, 1 mark

Question:

Paddy is dried from 22% to 14% moisture content (wet basis). Calculate the mass of water removed per 100 kg of initial wet paddy (in kg).

(Answer in the range 9.00 to 9.60)

Answer: 9.30 (accepted range: 9.00 to 9.60)

Detailed Explanation:

Dry matter in 100 kg initial wet paddy = 100 x (1 - 0.22) = 78 kg (dry matter is conserved during drying).

Final wet mass at 14% MC = 78 / (1 - 0.14) = 78/0.86 = 90.70 kg.

Water removed = 100 - 90.70 = 9.30 kg.

Q.34

Section: Dairy & Food Engineering   Topic: Preservation of Food

Type & Marks: MCQ, 1 mark

Question:

Pasteurization of milk is primarily intended to:

(A)  Sterilize the milk completely, removing all microorganisms

(B)  Destroy pathogenic microorganisms while retaining the milk's nutritional and sensory quality

(C)  Increase the fat content of the milk

(D)  Permanently remove all enzymes present in the milk

Answer: B

Detailed Explanation:

Pasteurization is a mild heat treatment designed to destroy pathogenic and most spoilage microorganisms while minimizing changes to nutritional value, flavour, and colour.

(A) describes sterilization (a much more severe process), and (C), (D) are not the purpose or necessary effect of pasteurization.

Q.35

Section: Dairy & Food Engineering   Topic: Preservation of Food

Type & Marks: NAT, 1 mark

Question:

Milk is pasteurized by the HTST method at 72 degC for 15 seconds. If the initial bacterial load is 10^6 CFU/mL and the process achieves a 4-log reduction, calculate the final bacterial load (in CFU/mL).

(Answer in the range 95.00 to 105.00)

Answer: 100.00 (accepted range: 95.00 to 105.00)

Detailed Explanation:

A 4-log reduction means the population is divided by 10^4.

Final load = 10^6 / 10^4 = 10^2 = 100.00 CFU/mL.

Q.36

Section: Engineering Mathematics   Topic: Calculus

Type & Marks: MCQ, 2 marks

Question:

The function f(x) = x^3 - 6x^2 + 9x + 2 has a local maximum at x = ____.

(A)  0

(B)  1

(C)  2

(D)  3

Answer: B

Detailed Explanation:

f'(x) = 3x^2 - 12x + 9 = 3(x-1)(x-3). Critical points: x = 1, x = 3.

f''(x) = 6x - 12. At x = 1: f''(1) = -6 < 0, so x = 1 is a local maximum.

At x = 3: f''(3) = 6 > 0, so x = 3 is a local minimum, ruling out (D).

(A) and (C) are not critical points at all.

Q.37

Section: Engineering Mathematics   Topic: Calculus

Type & Marks: NAT, 2 marks

Question:

Evaluate the definite integral: Integral from x = 0 to x = 2 of (3x^2 + 2x) dx.

(Answer in the range 11.90 to 12.10)

Answer: 12.00 (accepted range: 11.90 to 12.10)

Detailed Explanation:

Antiderivative: x^3 + x^2. Evaluating from 0 to 2: (2^3 + 2^2) - (0) = 8 + 4 = 12.00.

Q.38

Section: Engineering Mathematics   Topic: Linear Algebra

Type & Marks: NAT, 2 marks

Question:

Find the value of lambda for which the system of equations x + 2y = 3 and 2x + lambda*y = 6 has infinitely many solutions.

(Answer in the range 3.90 to 4.10)

Answer: 4.00 (accepted range: 3.90 to 4.10)

Detailed Explanation:

For infinitely many solutions, the two equations must be proportional: 1/2 = 2/lambda = 3/6.

Since 3/6 = 1/2, the condition 2/lambda = 1/2 gives lambda = 4.00.

Q.39

Section: Engineering Mathematics   Topic: Probability and Statistics

Type & Marks: MCQ, 2 marks

Question:

A box contains 4 red and 6 black balls. Two balls are drawn at random, without replacement. What is the probability that both balls drawn are red?

(A)  1/15

(B)  2/15

(C)  1/5

(D)  4/15

Answer: B

Detailed Explanation:

P(both red) = (4/10) x (3/9) = 12/90 = 2/15.

(A) 1/15 and (D) 4/15 result from arithmetic slips in the fraction multiplication. (C) 1/5 corresponds to drawing with replacement: (4/10)^2 = 16/100, which does not match either — it is simply a plausible-looking distractor.

Q.40

Section: Farm Machinery   Topic: Farm Machinery (tillage forces)

Type & Marks: NAT, 2 marks

Question:

A tillage tool has a working width of 0.3 m and operates at a depth of 0.15 m in a soil with a specific draft resistance of 40 kN/m^2. If the tool moves forward at 1.5 m/s, calculate the power required to pull the tool (in kW).

(Answer in the range 2.60 to 2.80)

Answer: 2.70 (accepted range: 2.60 to 2.80)

Detailed Explanation:

Draft force F = specific resistance x width x depth = 40 kN/m^2 x 0.3 m x 0.15 m = 1.8 kN.

Power = F x v = 1.8 kN x 1.5 m/s = 2.70 kW.

Q.41

Section: Farm Machinery   Topic: Farm Machinery (tillage forces)

Type & Marks: MCQ, 2 marks

Question:

Which of the following factors does NOT directly affect the draft required by a tillage implement?

(A)  Speed of operation

(B)  Soil moisture content

(C)  Depth of operation

(D)  Color of the implement

Answer: D

Detailed Explanation:

Draft depends on soil properties (moisture, texture), operating depth, tool geometry, and to a lesser extent operating speed. The color of the implement has no physical bearing on the soil-cutting forces.

(A), (B), (C) are all recognized, physically meaningful factors affecting draft.

Q.42

Section: Farm Machinery   Topic: Machine Design (machine elements)

Type & Marks: NAT, 2 marks

Question:

A shaft transmits 15 kW of power at 300 rpm. Calculate the torque transmitted by the shaft (in N.m).

(Answer in the range 475.00 to 480.00)

Answer: 477.46 (accepted range: 475.00 to 480.00)

Detailed Explanation:

Power P = 2*pi*N*T/60, so T = P x 60 / (2*pi*N) = (15000 x 60) / (2 x pi x 300) = 900000 / 1884.96 = 477.46 N.m.

Q.43

Section: Farm Machinery   Topic: Equipment for spraying

Type & Marks: NAT, 2 marks

Question:

A boom sprayer has 12 nozzles spaced 0.5 m apart and travels at 4 km/h. Each nozzle discharges 0.8 L/min. Calculate the application rate (in L/ha).

(Answer in the range 235.00 to 245.00)

Answer: 240.00 (accepted range: 235.00 to 245.00)

Detailed Explanation:

Effective boom width = 12 x 0.5 = 6 m. Forward speed = 4 km/h = 66.67 m/min.

Area covered per minute = 6 x 66.67 = 400 m^2/min.

Total discharge = 12 x 0.8 = 9.6 L/min.

Application rate = 9.6 L / 400 m^2 = 0.024 L/m^2 = 0.024 x 10000 = 240.00 L/ha.

Q.44

Section: Farm Power   Topic: Sources of Power (wind, solar, biomass, bio-fuels, human, animal)

Type & Marks: NAT, 2 marks

Question:

A solar photovoltaic panel of area 2 m^2 receives solar radiation at 800 W/m^2. If the panel efficiency is 15%, calculate the electrical power output (in W).

(Answer in the range 235.00 to 245.00)

Answer: 240.00 (accepted range: 235.00 to 245.00)

Detailed Explanation:

Power output = Area x Irradiance x Efficiency = 2 x 800 x 0.15 = 240.00 W.

Q.45

Section: Farm Power   Topic: Tractors & Power Tillers (traction theory)

Type & Marks: NAT, 2 marks

Question:

A tractor weighing 18 kN has a coefficient of traction of 0.6 on a given soil. If the total motion resistance is 3 kN, calculate the maximum drawbar pull available (in kN).

(Answer in the range 7.60 to 8.00)

Answer: 7.80 (accepted range: 7.60 to 8.00)

Detailed Explanation:

Maximum tractive effort = Coefficient of traction x Weight = 0.6 x 18 = 10.8 kN.

Drawbar pull = Tractive effort - Motion resistance = 10.8 - 3 = 7.80 kN.

Q.46

Section: Farm Power   Topic: Farm Power: engine components & I.C. engine systems

Type & Marks: NAT, 2 marks

Question:

A four-stroke single-cylinder diesel engine has a bore of 100 mm and a stroke of 120 mm. Calculate the swept volume of the cylinder (in cm^3).

(Answer in the range 935.00 to 950.00)

Answer: 942.48 (accepted range: 935.00 to 950.00)

Detailed Explanation:

Swept volume Vs = (pi/4) x D^2 x L = (pi/4) x (10 cm)^2 x 12 cm = (pi/4) x 100 x 12 = pi x 300 = 942.48 cm^3.

Q.47

Section: Farm Power   Topic: Thermodynamic principles & I.C. engine cycles

Type & Marks: NAT, 2 marks

Question:

An Otto cycle engine has a compression ratio of 8. Calculate the theoretical air-standard efficiency (in %), taking gamma = 1.4.

(Answer in the range 55.50 to 57.50)

Answer: 56.47 (accepted range: 55.50 to 57.50)

Detailed Explanation:

Air-standard efficiency of Otto cycle: eta = 1 - (1/r)^(gamma-1) = 1 - (1/8)^0.4.

(1/8)^0.4 = 8^-0.4 = exp(-0.4 x ln8) = exp(-0.4 x 2.0794) = exp(-0.8318) = 0.4353.

eta = 1 - 0.4353 = 0.5647, i.e., 56.47%.

Q.48

Section: Farm Power   Topic: Human engineering, ergonomics and safety

Type & Marks: MCQ, 2 marks

Question:

Which of the following is the recommended safe practice while operating a tractor-mounted power take-off (PTO) shaft?

(A)  Operating without a PTO shield to improve visibility

(B)  Wearing loose clothing for comfort while working near the PTO

(C)  Ensuring the PTO shield is in place and never stepping over a rotating shaft

(D)  Disengaging the master shield only during operation and refitting it afterward

Answer: C

Detailed Explanation:

PTO shafts rotate at high speed and are a major cause of entanglement injuries; the shield/guard must always be in place, and operators must never step over or reach across a rotating shaft.

(A) and (D) both involve removing/disengaging the shield, which is unsafe. (B) increases entanglement risk since loose clothing can be caught by the rotating shaft.

Q.49

Section: Soil & Water Conservation Engineering   Topic: Fluid Mechanics

Type & Marks: NAT, 2 marks

Question:

Water flows through a horizontal pipe that tapers from a diameter of 200 mm to 100 mm. If the velocity in the larger section is 1 m/s, calculate the velocity in the smaller section (in m/s).

(Answer in the range 3.90 to 4.10)

Answer: 4.00 (accepted range: 3.90 to 4.10)

Detailed Explanation:

By continuity, A1V1 = A2V2, so V2 = V1 x (D1/D2)^2 = 1 x (200/100)^2 = 1 x 4 = 4.00 m/s.

Q.50

Section: Soil & Water Conservation Engineering   Topic: Fluid Mechanics

Type & Marks: NAT, 2 marks

Question:

Water flows through a venturimeter with an inlet diameter of 300 mm and a throat diameter of 150 mm. The inlet velocity is 2 m/s. Calculate the velocity-head difference (v2^2 - v1^2)/2g between the throat and the inlet (in m). Take g = 9.81 m/s^2.

(Answer in the range 2.95 to 3.15)

Answer: 3.06 (accepted range: 2.95 to 3.15)

Detailed Explanation:

By continuity, V2 = V1 x (D1/D2)^2 = 2 x (300/150)^2 = 2 x 4 = 8 m/s.

(v2^2 - v1^2)/2g = (8^2 - 2^2) / (2 x 9.81) = (64-4)/19.62 = 60/19.62 = 3.06 m.

Q.51

Section: Soil & Water Conservation Engineering   Topic: Soil and Water Erosion

Type & Marks: NAT, 2 marks

Question:

A rectangular check-dam spillway must pass a peak discharge of 2.5 m^3/s. Using the weir formula Q = 1.84 x L x H^1.5 with head H = 0.5 m, calculate the required length of the weir L (in m).

(Answer in the range 3.70 to 4.00)

Answer: 3.84 (accepted range: 3.70 to 4.00)

Detailed Explanation:

H^1.5 = 0.5^1.5 = 0.3536.

L = Q / (1.84 x H^1.5) = 2.5 / (1.84 x 0.3536) = 2.5 / 0.6506 = 3.84 m.

Q.52

Section: Soil & Water Conservation Engineering   Topic: Hydrology

Type & Marks: NAT, 2 marks

Question:

A watershed has an area of 25 km^2. During a storm, the total rainfall was 60 mm, of which the runoff coefficient is 0.35. Calculate the total volume of direct runoff (in m^3).

(Answer in the range 520000 to 530000)

Answer: 525000.00 (accepted range: 520000 to 530000)

Detailed Explanation:

Runoff depth = 0.35 x 60 mm = 21 mm = 0.021 m.

Volume = Area x depth = (25 x 10^6 m^2) x 0.021 m = 525000.00 m^3.

Q.53

Section: Soil & Water Conservation Engineering   Topic: Hydrology

Type & Marks: NAT, 2 marks

Question:

A watershed of area 180 km^2 has a time-to-peak of 10 hours for its unit hydrograph. Using the SCS synthetic unit hydrograph relation Qp = 2.08 x A / Tp (Qp in m^3/s per cm of runoff, A in km^2, Tp in hours), calculate the peak discharge of the unit hydrograph (in m^3/s).

(Answer in the range 36.50 to 38.50)

Answer: 37.44 (accepted range: 36.50 to 38.50)

Detailed Explanation:

Qp = 2.08 x 180 / 10 = 374.4/10 = 37.44 m^3/s.

Q.54

Section: Soil & Water Conservation Engineering   Topic: Soil Mechanics

Type & Marks: MCQ, 2 marks

Question:

A saturated soil sample has a void ratio of 0.6 and specific gravity of solids 2.65. Taking gamma_w = 9.81 kN/m^3, the saturated unit weight of the soil is closest to:

(A)  16.5 kN/m^3

(B)  18.2 kN/m^3

(C)  19.9 kN/m^3

(D)  21.4 kN/m^3

Answer: C

Detailed Explanation:

gamma_sat = [(G + e)/(1 + e)] x gamma_w = [(2.65 + 0.6)/(1.6)] x 9.81 = (3.25/1.6) x 9.81 = 2.031 x 9.81 = 19.93 kN/m^3, closest to (C).

(A), (B), (D) correspond to using the wrong void-ratio term or omitting the (1+e) denominator.

Q.55

Section: Irrigation & Drainage Engineering   Topic: Soil-Water-Plant Relationship

Type & Marks: NAT, 2 marks

Question:

A crop field has a field capacity of 28% and a permanent wilting point of 12% (by weight). The bulk density of the soil is 1.4 g/cm^3, and the effective root-zone depth is 60 cm. Calculate the total available water in the root zone (in mm).

(Answer in the range 130.00 to 140.00)

Answer: 134.40 (accepted range: 130.00 to 140.00)

Detailed Explanation:

Available water (mm) = [(FC - PWP)/100] x Bulk density x Root-zone depth(mm)

= (0.28 - 0.12) x 1.4 x 600 = 0.16 x 1.4 x 600 = 134.40 mm.

Q.56

Section: Irrigation & Drainage Engineering   Topic: Irrigation Water Conveyance and Application Methods

Type & Marks: NAT, 2 marks

Question:

A lined irrigation channel with a wetted perimeter of 3 m conveys water over a length of 5 km. The seepage loss rate is 2.5 m^3/s per million m^2 of wetted area. Calculate the total seepage loss over the channel length (in m^3/s).

(Answer in the range 0.03 to 0.05)

Answer: 0.04 (accepted range: 0.03 to 0.05)

Detailed Explanation:

Wetted area = wetted perimeter x length = 3 m x 5000 m = 15000 m^2 = 0.015 million m^2.

Seepage loss = 2.5 x 0.015 = 0.0375, which rounds to 0.04 m^3/s.

Q.57

Section: Irrigation & Drainage Engineering   Topic: Wells and Pumps

Type & Marks: NAT, 2 marks

Question:

A tube well pumps water at 30 L/s against a total head of 25 m. If the pump efficiency is 70%, calculate the power input required (in kW). Take rho*g = 9.81 kN/m^3.

(Answer in the range 10.20 to 10.80)

Answer: 10.51 (accepted range: 10.20 to 10.80)

Detailed Explanation:

Hydraulic (output) power = rho*g x Q x H = 9.81 kN/m^3 x 0.030 m^3/s x 25 m = 7.3575 kW.

Input power = Output power / Efficiency = 7.3575 / 0.70 = 10.51 kW.

Q.58

Section: Irrigation & Drainage Engineering   Topic: Agricultural Drainage

Type & Marks: MSQ, 2 marks

Question:

Which of the following statements regarding subsurface agricultural drainage are correct?

(A)  Drain spacing decreases as soil hydraulic conductivity decreases, for a given drainage coefficient

(B)  Deeper drains generally allow wider spacing between them

(C)  The drainage coefficient represents the rate of water table drawdown (or discharge) required to remove excess water in a specified time

(D)  Perforated PVC pipes are never suitable for subsurface drains

Answer: A, B, C

Detailed Explanation:

(A) is correct: drain equations (e.g., Hooghoudt) show spacing is directly related to hydraulic conductivity — lower K requires closer spacing for the same drainage coefficient.

(B) is correct: deeper drains increase the flow cross-section available to groundwater, permitting wider spacing.

(C) is correct: the drainage coefficient is defined as the design rate of water removal (depth/time) needed to control the water table.

(D) is incorrect: perforated corrugated PVC/HDPE pipe is one of the most common materials used for modern subsurface drains.

Q.59

Section: Agricultural Process Engineering   Topic: Evaporation and Drying

Type & Marks: NAT, 2 marks

Question:

A single-effect evaporator concentrates milk from 10% total solids to 40% total solids. If the feed rate is 2000 kg/h, calculate the rate of water evaporation (in kg/h).

(Answer in the range 1480.00 to 1520.00)

Answer: 1500.00 (accepted range: 1480.00 to 1520.00)

Detailed Explanation:

Solids balance: solids in feed = 2000 x 0.10 = 200 kg/h (conserved).

Final concentrated product mass = 200 / 0.40 = 500 kg/h.

Water evaporated = 2000 - 500 = 1500.00 kg/h.

Q.60

Section: Agricultural Process Engineering   Topic: Evaporation and Drying

Type & Marks: NAT, 2 marks

Question:

In a cross-flow grain dryer, drying air enters at 60 degC and leaves at 35 degC. The specific heat of the moist air is 1.02 kJ/(kg.K), and its mass flow rate is 5 kg/s. Calculate the rate of heat given up by the air (in kW).

(Answer in the range 125.00 to 130.00)

Answer: 127.50 (accepted range: 125.00 to 130.00)

Detailed Explanation:

Rate of heat transfer Q = m x Cp x deltaT = 5 x 1.02 x (60-35) = 5 x 1.02 x 25 = 127.50 kW.

Q.61

Section: Agricultural Process Engineering   Topic: Size Reduction and Material Handling

Type & Marks: NAT, 2 marks

Question:

Using Bond's law, the energy required to reduce a material's particle size from 10 mm to 2 mm is 15 kJ/kg. Using the same Bond's work index, calculate the energy required (in kJ/kg) to further reduce the size from 2 mm to 1 mm.

(Answer in the range 10.50 to 12.00)

Answer: 11.24 (accepted range: 10.50 to 12.00)

Detailed Explanation:

Bond's law: E = Wi x (1/sqrt(Dp2) - 1/sqrt(Dp1)), sizes in mm.

Step 1 (10 mm -> 2 mm): 15 = Wi x (1/sqrt(2) - 1/sqrt(10)) = Wi x (0.7071 - 0.3162) = Wi x 0.3909

=> Wi = 15/0.3909 = 38.37.

Step 2 (2 mm -> 1 mm): E2 = Wi x (1/sqrt(1) - 1/sqrt(2)) = 38.37 x (1 - 0.7071) = 38.37 x 0.2929 = 11.24 kJ/kg.

Q.62

Section: Agricultural Process Engineering   Topic: Storage Systems

Type & Marks: MSQ, 2 marks

Question:

Which of the following factors contribute to increased spoilage of grain during storage?

(A)  High moisture content of the grain

(B)  High storage temperature

(C)  Presence of insect infestation

(D)  Low relative humidity of the storage environment

Answer: A, B, C

Detailed Explanation:

(A), (B), (C) all promote microbial and insect activity and hence accelerate grain deterioration during storage.

(D) is incorrect: a LOW relative humidity environment helps keep grain dry and actually reduces spoilage risk, so it is protective rather than a spoilage factor.

Q.63

Section: Dairy & Food Engineering   Topic: Heat and Mass Transfer

Type & Marks: MSQ, 2 marks

Question:

In the context of heat transfer during food processing, which of the following statements are correct?

(A)  Thermal conductivity of food materials generally increases with increasing moisture content

(B)  The convective heat transfer coefficient depends on the fluid flow characteristics around the food surface

(C)  Fourier's law describes heat transfer by convection

(D)  The Biot number is used to determine whether internal (conductive) or surface (convective) resistance controls the heat transfer process

Answer: A, B, D

Detailed Explanation:

(A) is correct: water has higher thermal conductivity than most food solids/air, so moist foods generally conduct heat better.

(B) is correct: the convective coefficient depends on flow velocity, turbulence, and fluid properties around the surface.

(C) is incorrect: Fourier's law governs conduction, not convection (Newton's law of cooling governs convection).

(D) is correct: the Biot number (hL/k) compares surface (convective) to internal (conductive) resistance.

Q.64

Section: Dairy & Food Engineering   Topic: Heat and Mass Transfer

Type & Marks: NAT, 2 marks

Question:

A food slab of thickness 4 cm has one face maintained at 80 degC and the other at 20 degC at steady state. If the thermal conductivity of the food material is 0.5 W/(m.K), calculate the heat flux through the slab (in W/m^2).

(Answer in the range 740.00 to 760.00)

Answer: 750.00 (accepted range: 740.00 to 760.00)

Detailed Explanation:

Steady-state conduction (Fourier's law): q = k x deltaT / L = 0.5 x (80-20) / 0.04 = 0.5 x 60 / 0.04 = 30/0.04 = 750.00 W/m^2.

Q.65

Section: Dairy & Food Engineering   Topic: Preservation of Food

Type & Marks: MSQ, 2 marks

Question:

Which of the following are recognized methods of food preservation based primarily on moisture reduction or control?

(A)  Sun drying

(B)  Freeze drying (lyophilization)

(C)  Osmotic dehydration

(D)  Ultra-high temperature (UHT) processing

Answer: A, B, C

Detailed Explanation:

(A), (B), (C) all preserve food chiefly by lowering the water activity/moisture content, thereby inhibiting microbial growth.

(D) UHT processing is a thermal sterilization method (very high temperature for a short time); it does not work by moisture reduction.

# Blueprint Verification Table

1. Section x Marks x Question-Type Verification

| Section | 1-mk Qs | 2-mk Qs | Total Qs | Marks | MCQ | MSQ | NAT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GA — General Aptitude | 5 | 5 | 10 | 15 | 10 | 0 | 0 |
| S1 — Engineering Mathematics | 5 | 4 | 9 | 13 | 6 | 0 | 3 |
| S2 — Farm Machinery | 3 | 4 | 7 | 11 | 3 | 0 | 4 |
| S3 — Farm Power | 4 | 5 | 9 | 14 | 3 | 0 | 6 |
| S4 — Soil & Water Conservation Engineering | 4 | 6 | 10 | 16 | 4 | 0 | 6 |
| S5 — Irrigation & Drainage Engineering | 2 | 4 | 6 | 10 | 1 | 1 | 4 |
| S6 — Agricultural Process Engineering | 5 | 4 | 9 | 13 | 2 | 1 | 6 |
| S7 — Dairy & Food Engineering | 2 | 3 | 5 | 8 | 1 | 2 | 2 |
| TOTAL | 30 | 35 | 65 | 100 | 30 | 4 | 31 |

2. Sub-topic Weightage Verification

| Section | Sub-topic | Marks |
| --- | --- | --- |
| General Aptitude | Data Interpretation | 1 |
| General Aptitude | Quantitative Aptitude | 6 |
| General Aptitude | Verbal Aptitude | 3 |
| General Aptitude | Analytical Aptitude | 3 |
| General Aptitude | Spatial Aptitude | 2 |
| Engineering Mathematics | Calculus | 5 |
| Engineering Mathematics | Linear Algebra | 4 |
| Engineering Mathematics | Differential Equations | 2 |
| Farm Machinery | Machine Design (machine elements) | 3 |
| Farm Machinery | Sowing/planting/fertilizer application equipment | 1 |
| Farm Machinery | Cost analysis of implements and tractors | 1 |
| Farm Power | Power transmission (clutch, gear box, differential, final drive, brakes) | 1 |
| Farm Power | Fuels, combustion and lubricants | 1 |
| Farm Power | Sources of Power (wind, solar, biomass, bio-fuels, human, animal) | 3 |
| Farm Power | Tractors & Power Tillers (traction theory) | 3 |
| Soil & Water Conservation Engineering | Surveying and Leveling | 1 |
| Soil & Water Conservation Engineering | Soil and Water Erosion | 4 |
| Soil & Water Conservation Engineering | Hydrology | 5 |
| Irrigation & Drainage Engineering | Groundwater Hydrology | 2 |
| Agricultural Process Engineering | Size Reduction and Material Handling | 4 |
| Agricultural Process Engineering | Engineering properties of agricultural produce | 2 |
| Agricultural Process Engineering | Evaporation and Drying | 5 |
| Dairy & Food Engineering | Preservation of Food | 4 |
| Engineering Mathematics | Probability and Statistics | 2 |
| Farm Machinery | Farm Machinery (tillage forces) | 4 |
| Farm Machinery | Equipment for spraying | 2 |
| Farm Power | Farm Power: engine components & I.C. engine systems | 2 |
| Farm Power | Thermodynamic principles & I.C. engine cycles | 2 |
| Farm Power | Human engineering, ergonomics and safety | 2 |
| Soil & Water Conservation Engineering | Fluid Mechanics | 4 |
| Soil & Water Conservation Engineering | Soil Mechanics | 2 |
| Irrigation & Drainage Engineering | Soil-Water-Plant Relationship | 2 |
| Irrigation & Drainage Engineering | Irrigation Water Conveyance and Application Methods | 2 |
| Irrigation & Drainage Engineering | Wells and Pumps | 2 |
| Irrigation & Drainage Engineering | Agricultural Drainage | 2 |
| Agricultural Process Engineering | Storage Systems | 2 |
| Dairy & Food Engineering | Heat and Mass Transfer | 4 |

3. Self-Check Confirmation

- Exactly 65 questions, exactly 100 marks. Confirmed (see TOTAL row above).

- Q.1-Q.5 and Q.11-Q.35 are 1 mark each; Q.6-Q.10 and Q.36-Q.65 are 2 marks each. Confirmed by construction.

- Section marks match the mandatory blueprint exactly: GA 15, S1 13, S2 11, S3 14, S4 16, S5 10, S6 13, S7 8.

- Type counts match the mandatory blueprint exactly: 30 MCQ, 4 MSQ, 31 NAT.

- Sub-topic marks match the mandatory blueprint (see table above); no rotation-pool swaps were used in this set (Set 1, base sub-topic list).

- Every NAT question states its required unit and rounding/tolerance instruction, and the key gives a numeric tolerance range.

- Every numerical answer was independently re-derived from first principles during paper construction and matches the key.

- No question is an exact or near-exact reproduction of any past GATE AG question; all numerical data, framing, and the quantity asked for are original to this set.

- Every question is self-contained and solvable from the data supplied within it; no question depends on the answer to another.