GATE 2027

Agricultural Engineering (AG)

Full-Length Original Mock Paper — Set 1

Paper Code: AG

Duration: 180 minutes

Maximum Marks: 100

Total Questions: 65 (GA: Q.1–Q.10, AG Subject: Q.11–Q.65)

General Instructions & Marking Scheme

1. The paper contains 65 questions carrying a total of 100 marks: General Aptitude (GA), Q.1–Q.10 (15 marks) and the Agricultural Engineering (AG) subject, Q.11–Q.65 (85 marks).

2. Q.1–Q.5 and Q.11–Q.35 carry 1 mark each. Q.6–Q.10 and Q.36–Q.65 carry 2 marks each.

3. There are three question types: Multiple Choice Questions (MCQ, four options, exactly one correct), Multiple Select Questions (MSQ, four options, one or more correct), and Numerical Answer Type (NAT, no options; answer to be typed in).

4. For MCQs: 1/3 mark is deducted for a wrong answer to a 1-mark MCQ, and 2/3 mark is deducted for a wrong answer to a 2-mark MCQ. There is no negative marking for MSQ or NAT questions, and no partial marking for MSQ questions.

5. Calculators (as permitted by GATE) may be used. No separate data/formula booklet will be provided; all necessary data are given within each question.

6. Engineering Mathematics questions are integrated within the AG subject numbering (Q.11–Q.65) and are not a separate section.

# QUESTION PAPER

General Aptitude (Q.1 – Q.10)

Q.1  [MCQ, 1 mark]  A trader marks up the price of a fertilizer bag by 25% above its cost price and then offers a discount of 20% on the marked price. Find the trader’s overall percentage profit or loss.

(A)  0% (no profit, no loss)

(B)  5% profit

(C)  5% loss

(D)  10% profit

Q.2  [MCQ, 1 mark]  The ratio of length to width of a rectangular field is 5:3. If the perimeter of the field is 320 m, find its area.

(A)  4800 m²

(B)  5400 m²

(C)  6000 m²

(D)  6400 m²

Q.3  [MCQ, 1 mark]  Choose the word most nearly OPPOSITE in meaning to “PARSIMONIOUS”.

(A)  Frugal

(B)  Lavish

(C)  Meticulous

(D)  Reticent

Q.4  [MCQ, 1 mark]  In a certain code, each letter of a word is replaced by the letter that comes one position later in the English alphabet (A→B, B→C, …, Z→A). If “WHEAT” is coded as “XIFBU”, how is “MAIZE” coded in the same scheme?

(A)  NBJAF

(B)  NBJAE

(C)  MBJAF

(D)  NCJAF

Q.5  [MCQ, 1 mark]  In a survey of 200 farmers, 120 use drip irrigation, 90 use sprinkler irrigation, and 40 use both drip and sprinkler irrigation. How many farmers use neither drip nor sprinkler irrigation?

(A)  10

(B)  20

(C)  30

(D)  40

Q.6  [MCQ, 2 marks]  A warehouse contains 900 quintals of paddy at 20% moisture content (wet basis). It is dried until the moisture content reduces to 10% (wet basis). Find the quantity of water removed, assuming no loss of dry matter.

(A)  90 quintals

(B)  100 quintals

(C)  110 quintals

(D)  120 quintals

Q.7  [MCQ, 2 marks]  Two identical irrigation pumps A and B can together fill a farm pond in 6 hours. Pump A alone takes 5 hours longer than Pump B alone to fill the pond. Find the time taken by Pump B alone to fill the pond.

(A)  8 hours

(B)  10 hours

(C)  12 hours

(D)  15 hours

Q.8  [MCQ, 2 marks]  Read the following passage and answer the question.

“Soil organic carbon (SOC) improves water-holding capacity and nutrient availability. Studies across semi-arid regions show that a 1% increase in SOC can increase available water capacity by approximately 1.5%. However, SOC levels decline rapidly under intensive tillage and residue removal, while they recover slowly, often over decades, under conservation practices.”

Which of the following can be logically inferred from the passage?

(A)  SOC can be restored as quickly as it is depleted.

(B)  Conservation practices alone guarantee high crop yields.

(C)  Loss of SOC due to intensive tillage is easier to cause than to reverse.

(D)  Available water capacity is unrelated to SOC.

Q.9  [MCQ, 2 marks]  Four agricultural engineers P, Q, R and S each specialize in a different area — Farm Machinery, Irrigation, Food Processing and Soil Conservation — and each is posted in a different state — Punjab, Kerala, Odisha and Rajasthan (not necessarily in this order).

Clues: (1) The engineer posted in Punjab specializes in Farm Machinery. (2) R specializes in Irrigation and is not posted in Kerala. (3) S is posted in Odisha. (4) Q specializes in Soil Conservation. (5) P is posted in Punjab.

Which engineer specializes in Food Processing, and in which state is that engineer posted?

(A)  Q, Kerala

(B)  S, Odisha

(C)  R, Rajasthan

(D)  P, Punjab

Q.10  [MCQ, 2 marks]  [Figure description: A solid cube is painted red on all six outer faces, then cut by two equally spaced cuts parallel to each face into 27 identical smaller cubes (a 3×3×3 arrangement), as in a Rubik’s cube.] How many of the 27 smaller cubes have exactly two faces painted red?

(A)  8

(B)  12

(C)  6

(D)  24

Agricultural Engineering — Subject Questions (Q.11 – Q.65)

Q.11  [MCQ, 1 mark]  Evaluate: lim(x→0) [ sin(3x) / tan(5x) ]

(A)  3/5

(B)  5/3

(C)  15

(D)  1

Q.12  [MCQ, 1 mark]  The differential equation dy/dx + y/x = x² is best classified as:

(A)  Linear, first order

(B)  Nonlinear, first order

(C)  Linear, second order

(D)  Exact, non-linear

Q.13  [NAT, 1 mark]  Solve the initial value problem dy/dx = 2y, y(0) = 3. Find y(1), rounded to two decimal places. (Use e = 2.71828.)

Q.14  [MCQ, 1 mark]  The mean and variance of a binomial distribution are 4 and 3 respectively. Find the number of trials n.

(A)  12

(B)  16

(C)  20

(D)  8

Q.15  [NAT, 1 mark]  A discrete random variable X takes values 1, 2, 3, 4 with probabilities 0.1, 0.2, 0.3, 0.4 respectively. Find E(X).

Q.16  [MCQ, 1 mark]  Which of the following seed-metering mechanisms is most suitable for precision (single-seed) placement of large, uniformly sized seeds such as maize and cotton?

(A)  Fluted roller metering

(B)  Cell-feed (horizontal plate) metering

(C)  Internal double-run metering

(D)  Star-wheel (auger) metering

Q.17  [NAT, 1 mark]  The fixed cost of operating a tractor is ₹1,80,000 per year and the variable cost is ₹250 per hour of operation. If the tractor is used for 600 hours in a year, find the total operating cost per hour of use, rounded to the nearest whole number (in ₹/h).

Q.18  [MCQ, 1 mark]  In a belt drive, the driver pulley has a diameter of 200 mm and runs at 1000 rpm. The driven pulley has a diameter of 400 mm. Neglecting belt slip, the speed of the driven pulley is:

(A)  250 rpm

(B)  500 rpm

(C)  1000 rpm

(D)  2000 rpm

Q.19  [MCQ, 1 mark]  Which of the following renewable energy sources is most suitable for continuous, on-demand power generation on a farm, largely independent of daily weather variation?

(A)  Solar photovoltaic

(B)  Wind

(C)  Biomass/biogas

(D)  Human power

Q.20  [MCQ, 1 mark]  For a two-wheel-drive agricultural tractor operating on firm, untilled soil, the coefficient of traction typically lies in the range of:

(A)  0.85–0.90

(B)  0.60–0.70

(C)  0.30–0.40

(D)  0.10–0.15

Q.21  [NAT, 1 mark]  A tractor gearbox-and-final-drive train has an overall gear reduction ratio of 40:1 between the engine and the rear axle. If the engine runs at 2000 rpm, find the rear axle speed (in rpm).

Q.22  [NAT, 1 mark]  The calorific value of diesel is 45 MJ/kg. Find the energy content of 8 kg of diesel (in MJ).

Q.23  [MCQ, 1 mark]  In a dumpy-level survey, the back sight (BS) reading at a point is 2.850 m and the fore sight (FS) reading on the next point is 1.640 m. The change in elevation between the two points is:

(A)  Rise of 1.210 m

(B)  Fall of 1.210 m

(C)  Rise of 4.490 m

(D)  Fall of 4.490 m

Q.24  [MCQ, 1 mark]  The time of concentration of a watershed refers to:

(A)  The total duration of a storm event

(B)  The time taken for runoff to travel from the hydraulically most remote point of the watershed to the outlet

(C)  The time between the start of rainfall and the start of runoff

(D)  The time taken for a reservoir to fill completely

Q.25  [NAT, 1 mark]  Using the Universal Soil Loss Equation (USLE), A = R×K×LS×C×P, with R = 400 MJ·mm/(ha·h·yr), K = 0.30 t·ha·h/(ha·MJ·mm), LS = 1.5, C = 0.2, and P = 1.0, find the estimated soil loss A (in t/ha/yr), rounded to one decimal place.

Q.26  [NAT, 1 mark]  A cultivated field has a slope-length/steepness (LS) factor of 1.8 for a slope length of 25 m, giving a soil loss (by USLE) of 40 t/ha/yr with all other USLE factors unchanged. If bench terracing reduces the effective slope length to 10 m, giving a new LS factor of 0.9, find the reduced soil loss (in t/ha/yr), with all other USLE factors unchanged.

Q.27  [MCQ, 1 mark]  The primary purpose of subsurface (tile) drainage in agricultural fields is to:

(A)  Increase soil erosion

(B)  Lower the water table to remove excess soil water from the root zone

(C)  Increase salinity in the root zone

(D)  Reduce the infiltration rate of the soil

Q.28  [NAT, 1 mark]  A subsurface drain system is designed to remove excess water at a drainage coefficient of 12 mm/day from a field of area 8 hectares. Find the required drainage discharge (in m³/day), rounded to the nearest whole number.

Q.29  [MCQ, 1 mark]  The falling-rate period of drying of an agricultural material is characterized by:

(A)  Constant surface moisture, with drying rate limited by external heat and mass transfer

(B)  Drying rate limited by internal moisture diffusion to the surface of the material

(C)  A drying rate that increases with time

(D)  No moisture removal from the material

Q.30  [MCQ, 1 mark]  The angle of repose of a granular agricultural material is defined as:

(A)  The angle between the direction of gravity and the normal to a free-flowing heap surface

(B)  The angle made by the free surface of a heaped material with the horizontal, at which the material is on the verge of sliding

(C)  The angle between two adjacent seeds resting in a bulk

(D)  The angle of contact between a seed and a metal surface only

Q.31  [NAT, 1 mark]  A grain kernel has axial dimensions of length 8 mm, width 4 mm and thickness 3 mm. Find its geometric mean diameter, Dg = (L×W×T)^(1/3), in mm, rounded to two decimal places.

Q.32  [NAT, 1 mark]  A cylindrical grain silo has a diameter of 4 m and is filled with grain to a height of 6 m. Taking the bulk density of the grain as 750 kg/m³, find the mass of grain stored (in tonnes), rounded to one decimal place.

Q.33  [NAT, 1 mark]  The safe moisture content for long-term storage of wheat is approximately 12% (wet basis). A 500 kg lot of wheat currently has a moisture content of 18% (wet basis). Find the mass of water that must be removed to bring it to 12% (wet basis), rounded to the nearest whole number (in kg). (Use Ww = W₁(M₁−M₂)/(100−M₂).)

Q.34  [MCQ, 1 mark]  Pasteurization of milk primarily aims to:

(A)  Sterilize milk completely, destroying all microorganisms including spores

(B)  Destroy pathogenic microorganisms and reduce spoilage organisms while retaining nutritional and sensory quality

(C)  Increase the fat content of milk

(D)  Remove water content from milk

Q.35  [NAT, 1 mark]  Milk is pasteurized by the HTST (High Temperature Short Time) method at 72°C, held for 15 seconds. If the D-value (decimal reduction time) of a target pathogen at 72°C is 3 seconds, find the number of decimal (log) reductions achieved in the pathogen population during the holding period.

Q.36  [MCQ, 2 marks]  Find the value of ∫₀^(π/2) sin²x dx.

(A)  π/2

(B)  π/4

(C)  π

(D)  1

Q.37  [NAT, 2 marks]  A function f(x) = x³ − 6x² + 9x + 2 has a local maximum at x = a. Find the value of a.

Q.38  [MCQ, 2 marks]  Find the eigenvalues of the matrix A = [[4, 1], [2, 3]].

(A)  2, 5

(B)  1, 6

(C)  3, 4

(D)  −2, 5

Q.39  [MCQ, 2 marks]  Given the matrix A = [[2, 0], [0, 3]], find det(A²).

(A)  12

(B)  18

(C)  36

(D)  9

Q.40  [MCQ, 2 marks]  A shaft transmits 15 kW of power at 300 rpm. Find the torque transmitted by the shaft (nearest whole number, in N·m).

(A)  287 N·m

(B)  358 N·m

(C)  477 N·m

(D)  573 N·m

Q.41  [NAT, 2 marks]  A mould-board plough has a working width of 0.30 m and operates at a depth of 0.20 m in a soil with a specific draft resistance of 50 kN/m². Find the draft required (in kN).

Q.42  [NAT, 2 marks]  A tractor pulls a cultivator requiring a draft of 8 kN at a forward speed of 3.6 km/h. The tractive efficiency of the tractor is 75%. Find the power that the engine must deliver to the drive wheels (in kW), rounded to two decimal places. (First compute the drawbar power, then divide by the tractive efficiency.)

Q.43  [NAT, 2 marks]  A boom sprayer has 20 nozzles spaced 0.5 m apart and travels at a speed of 5 km/h. Each nozzle discharges 0.8 L/min. Find the application rate, in litres per hectare (L/ha), rounded to the nearest whole number.

Q.44  [MCQ, 2 marks]  In a four-stroke diesel engine, the correct sequence of the four strokes is:

(A)  Suction, Power, Compression, Exhaust

(B)  Suction, Compression, Power, Exhaust

(C)  Compression, Suction, Power, Exhaust

(D)  Power, Compression, Suction, Exhaust

Q.45  [NAT, 2 marks]  A small biogas plant produces 12 m³ of biogas per day, with a calorific value of 22 MJ/m³. This biogas is used to run a dual-fuel engine with a thermal efficiency of 25%. Find the useful work output per day (in MJ), rounded to the nearest whole number.

Q.46  [NAT, 2 marks]  A tractor develops a drawbar pull of 12 kN while the theoretical (no-slip) travel speed is 6 km/h; due to wheel slip, the actual travel speed is measured as 5.4 km/h. Find the drawbar power delivered, based on the actual travel speed (in kW), rounded to two decimal places.

Q.47  [NAT, 2 marks]  An air-standard Otto cycle has a compression ratio of 8. Taking the ratio of specific heats γ = 1.4, find the thermal efficiency of the cycle, expressed as a percentage, rounded to one decimal place.

Q.48  [NAT, 2 marks]  A tractor operator is exposed to a whole-body, frequency-weighted RMS vibration acceleration of 0.8 m/s² continuously for 4 hours in a workday. Using the 8-hour energy-equivalent relation A(8) = A × √(T/8), where T is the exposure duration in hours, find A(8) in m/s², rounded to two decimal places.

Q.49  [NAT, 2 marks]  A saturated soil sample has a bulk (saturated) unit weight of 20 kN/m³. Taking the unit weight of water as 9.81 kN/m³, find the submerged (buoyant) unit weight of the soil (in kN/m³), rounded to two decimal places.

Q.50  [MCQ, 2 marks]  Water flows through a horizontal pipe that gradually contracts from a diameter of 300 mm to 150 mm. If the velocity in the larger section is 1 m/s, find the velocity in the smaller section, using the continuity equation.

(A)  2 m/s

(B)  4 m/s

(C)  0.5 m/s

(D)  8 m/s

Q.51  [NAT, 2 marks]  A rainfall of 60 mm falls uniformly over a catchment of 250 hectares in 2 hours. If the runoff coefficient is 0.35, find the total runoff volume generated (in m³), rounded to the nearest 100. (Use Volume = C × Rainfall(m) × Area(m²).)

Q.52  [NAT, 2 marks]  A small watershed of area 400 ha receives a rainfall event of intensity 30 mm/h lasting 3 hours. Using the Rational Method, Q = CIA/360 (with Q in m³/s, C dimensionless, I in mm/h, and A in hectares), and taking C = 0.4, find the peak discharge (in m³/s), rounded to two decimal places.

Q.53  [MCQ, 2 marks]  For fully developed laminar flow in a circular pipe, the ratio of the maximum velocity (at the centreline) to the average (mean) velocity is:

(A)  1

(B)  1.5

(C)  2

(D)  4

Q.54  [NAT, 2 marks]  A check dam reduces the flow velocity in a gully from 2.5 m/s to 0.5 m/s. If the sediment transport capacity of the flowing water is proportional to the cube of the flow velocity, find the percentage reduction in sediment transport capacity, rounded to one decimal place.

Q.55  [MSQ, 2 marks]  Which of the following statements regarding a confined aquifer are correct?

(A)  The piezometric surface can lie above the ground surface, resulting in flowing (artesian) wells.

(B)  A confined aquifer is bounded above and below by impermeable (or relatively impermeable) layers.

(C)  The storage coefficient of a confined aquifer is typically of the same order of magnitude as the specific yield of an unconfined aquifer.

(D)  Water is released from a confined aquifer mainly by expansion of water and compression of the aquifer matrix as pressure declines.

Q.56  [NAT, 2 marks]  A soil has a field capacity of 28% and a permanent wilting point of 12% (both on a dry-weight basis). The bulk density of the soil is 1.4 g/cm³, and the effective root zone depth is 60 cm. Find the total available water capacity in the root zone (in mm), rounded to the nearest whole number.

Q.57  [NAT, 2 marks]  A furrow irrigation system is designed to apply a net depth of 60 mm of water. If the overall (advance + application) efficiency achieved in the field is 65%, find the gross depth of water that must be applied to deliver this net depth (in mm), rounded to one decimal place.

Q.58  [NAT, 2 marks]  A centrifugal pump delivers 15 L/s of water against a total head of 18 m. If the overall efficiency of the pump is 65%, find the power input required to drive the pump (in kW), rounded to two decimal places. (Take ρ = 1000 kg/m³, g = 9.81 m/s².)

Q.59  [MSQ, 2 marks]  Which of the following statements about size-reduction of agricultural/food materials are correct?

(A)  Rittinger’s law assumes that the energy required for size reduction is proportional to the new surface area created.

(B)  Kick’s law assumes that the energy required is proportional to the logarithm of the reduction ratio (ratio of initial to final particle size), independent of particle size.

(C)  Bond’s law applies only to gaseous materials and cannot be applied to solids.

(D)  The energy required for size reduction generally increases as the final particle size decreases, for a given feed size.

Q.60  [NAT, 2 marks]  A single-effect evaporator concentrates milk from 10% total solids to 40% total solids. If the feed rate is 2000 kg/h, find the rate of water evaporation, in kg/h, rounded to the nearest whole number.

Q.61  [NAT, 2 marks]  500 kg of a food grain with an initial moisture content of 22% (wet basis) is dried to 13% (wet basis). If the latent heat of vaporization of water is 2260 kJ/kg, find the minimum theoretical heat energy required for drying (in MJ), rounded to the nearest whole number. (Ignore sensible heat effects.)

Q.62  [NAT, 2 marks]  A hammer mill grinding maize requires 12 kWh/tonne of energy to reduce feed size from 10 mm to 2 mm, following Bond’s law: E = Kb(1/√Dp − 1/√Df), where Dp and Df are the product and feed sizes (mm). Using the same mill and the same feed size (10 mm), find the specific energy required (in kWh/tonne) to grind the maize to a final size of 1 mm, rounded to one decimal place.

Q.63  [MSQ, 2 marks]  For steady-state, one-dimensional heat conduction through a composite wall of a cold storage (built of insulation and structural layers in series), which of the following statements are correct?

(A)  The overall thermal resistance of the wall is the sum of the individual layer resistances (series arrangement).

(B)  Increasing the thickness of the insulation layer decreases the overall heat transfer coefficient (U-value) of the wall.

(C)  The layer with the highest thermal conductivity offers the greatest resistance to heat flow.

(D)  For steady-state one-dimensional conduction, the heat flux is the same through each layer of the composite wall.

Q.64  [MSQ, 2 marks]  In convective heat transfer during forced-air cooling of fruits and vegetables, which of the following statements are correct?

(A)  A higher Reynolds number generally corresponds to a higher convective heat transfer coefficient.

(B)  The Nusselt number is a dimensionless ratio of convective to conductive heat transfer.

(C)  Natural convection heat transfer coefficients are generally higher than forced convection coefficients for the same fluid and surface.

(D)  Increasing the air velocity around the product generally reduces the cooling time.

Q.65  [NAT, 2 marks]  A refrigerated cold store cools 200 kg of fruit from an initial temperature of 25°C to a final temperature of 4°C. The fruit has a specific heat of 3.8 kJ/(kg·K). If the refrigeration system operates for 2 hours to achieve this cooling, find the average rate of heat removal (in kW), rounded to two decimal places.

# ANSWER KEY

| Q.No. | Section | Sub-topic | Type | Marks | Answer |
| --- | --- | --- | --- | --- | --- |
| 1 | GA – General Aptitude | Quantitative Aptitude | MCQ | 1 | A |
| 2 | GA – General Aptitude | Quantitative Aptitude | MCQ | 1 | C |
| 3 | GA – General Aptitude | Verbal Aptitude | MCQ | 1 | B |
| 4 | GA – General Aptitude | Analytical Aptitude | MCQ | 1 | A |
| 5 | GA – General Aptitude | Data Interpretation | MCQ | 1 | C |
| 6 | GA – General Aptitude | Quantitative Aptitude | MCQ | 2 | B |
| 7 | GA – General Aptitude | Quantitative Aptitude | MCQ | 2 | B |
| 8 | GA – General Aptitude | Verbal Aptitude | MCQ | 2 | C |
| 9 | GA – General Aptitude | Analytical Aptitude | MCQ | 2 | B |
| 10 | GA – General Aptitude | Spatial Aptitude | MCQ | 2 | B |
| 11 | S1 – Engineering Mathematics | Calculus | MCQ | 1 | A |
| 12 | S1 – Engineering Mathematics | Differential Equations | MCQ | 1 | A |
| 13 | S1 – Engineering Mathematics | Differential Equations | NAT | 1 | 22.10 to 22.25 |
| 14 | S1 – Engineering Mathematics | Probability and Statistics | MCQ | 1 | B |
| 15 | S1 – Engineering Mathematics | Probability and Statistics | NAT | 1 | 2.95 to 3.05 |
| 16 | S2 – Farm Machinery | Equipment for sowing, planting and fertilizer application | MCQ | 1 | B |
| 17 | S2 – Farm Machinery | Cost analysis of implements and tractors | NAT | 1 | 548 to 552 |
| 18 | S2 – Farm Machinery | Machine Design (machine elements) | MCQ | 1 | B |
| 19 | S3 – Farm Power | Sources of Power | MCQ | 1 | C |
| 20 | S3 – Farm Power | Tractors and Power Tillers: traction theory | MCQ | 1 | B |
| 21 | S3 – Farm Power | Power transmission (clutch, gear box, differential, final drive, brakes) | NAT | 1 | 49 to 51 |
| 22 | S3 – Farm Power | Fuels, combustion and lubricants | NAT | 1 | 355 to 365 |
| 23 | S4 – Soil & Water Conservation Engineering | Surveying and Leveling | MCQ | 1 | A |
| 24 | S4 – Soil & Water Conservation Engineering | Hydrology | MCQ | 1 | B |
| 25 | S4 – Soil & Water Conservation Engineering | Soil and Water Erosion | NAT | 1 | 35.5 to 36.5 |
| 26 | S4 – Soil & Water Conservation Engineering | Soil and Water Erosion | NAT | 1 | 19 to 21 |
| 27 | S5 – Irrigation & Drainage Engineering | Agricultural Drainage | MCQ | 1 | B |
| 28 | S5 – Irrigation & Drainage Engineering | Agricultural Drainage | NAT | 1 | 955 to 965 |
| 29 | S6 – Agricultural Process Engineering | Evaporation and Drying | MCQ | 1 | B |
| 30 | S6 – Agricultural Process Engineering | Engineering properties of agricultural produce | MCQ | 1 | B |
| 31 | S6 – Agricultural Process Engineering | Engineering properties of agricultural produce | NAT | 1 | 4.50 to 4.65 |
| 32 | S6 – Agricultural Process Engineering | Storage Systems | NAT | 1 | 56.0 to 57.0 |
| 33 | S6 – Agricultural Process Engineering | Storage Systems | NAT | 1 | 33 to 35 |
| 34 | S7 – Dairy & Food Engineering | Preservation of Food | MCQ | 1 | B |
| 35 | S7 – Dairy & Food Engineering | Preservation of Food | NAT | 1 | 4.8 to 5.2 |
| 36 | S1 – Engineering Mathematics | Calculus | MCQ | 2 | B |
| 37 | S1 – Engineering Mathematics | Calculus | NAT | 2 | 0.95 to 1.05 |
| 38 | S1 – Engineering Mathematics | Linear Algebra | MCQ | 2 | A |
| 39 | S1 – Engineering Mathematics | Linear Algebra | MCQ | 2 | C |
| 40 | S2 – Farm Machinery | Machine Design (machine elements) | MCQ | 2 | C |
| 41 | S2 – Farm Machinery | Farm Machinery: soil tillage and forces on a tillage tool | NAT | 2 | 2.9 to 3.1 |
| 42 | S2 – Farm Machinery | Farm Machinery: soil tillage and forces on a tillage tool | NAT | 2 | 10.60 to 10.75 |
| 43 | S2 – Farm Machinery | Equipment for spraying | NAT | 2 | 190 to 194 |
| 44 | S3 – Farm Power | Farm Power: engine components and I.C. engine systems | MCQ | 2 | B |
| 45 | S3 – Farm Power | Sources of Power | NAT | 2 | 64 to 68 |
| 46 | S3 – Farm Power | Tractors and Power Tillers: traction theory | NAT | 2 | 17.90 to 18.10 |
| 47 | S3 – Farm Power | Farm Power: thermodynamic principles and I.C. engine cycles | NAT | 2 | 56.0 to 57.0 |
| 48 | S3 – Farm Power | Human engineering, ergonomics and safety | NAT | 2 | 0.55 to 0.58 |
| 49 | S4 – Soil & Water Conservation Engineering | Soil Mechanics | NAT | 2 | 10.05 to 10.35 |
| 50 | S4 – Soil & Water Conservation Engineering | Fluid Mechanics | MCQ | 2 | B |
| 51 | S4 – Soil & Water Conservation Engineering | Hydrology | NAT | 2 | 52000 to 53000 |
| 52 | S4 – Soil & Water Conservation Engineering | Hydrology | NAT | 2 | 13.20 to 13.45 |
| 53 | S4 – Soil & Water Conservation Engineering | Fluid Mechanics | MCQ | 2 | C |
| 54 | S4 – Soil & Water Conservation Engineering | Soil and Water Erosion | NAT | 2 | 97.5 to 99.0 |
| 55 | S5 – Irrigation & Drainage Engineering | Groundwater Hydrology | MSQ | 2 | A, B, D |
| 56 | S5 – Irrigation & Drainage Engineering | Soil-Water-Plant Relationship | NAT | 2 | 133 to 136 |
| 57 | S5 – Irrigation & Drainage Engineering | Irrigation Water Conveyance and Application Methods | NAT | 2 | 91.5 to 93.0 |
| 58 | S5 – Irrigation & Drainage Engineering | Wells and Pumps | NAT | 2 | 4.00 to 4.15 |
| 59 | S6 – Agricultural Process Engineering | Size Reduction and Material Handling | MSQ | 2 | A, B, D |
| 60 | S6 – Agricultural Process Engineering | Evaporation and Drying | NAT | 2 | 1495 to 1505 |
| 61 | S6 – Agricultural Process Engineering | Evaporation and Drying | NAT | 2 | 114 to 120 |
| 62 | S6 – Agricultural Process Engineering | Size Reduction and Material Handling | NAT | 2 | 20.5 to 21.5 |
| 63 | S7 – Dairy & Food Engineering | Heat and Mass Transfer | MSQ | 2 | A, B, D |
| 64 | S7 – Dairy & Food Engineering | Heat and Mass Transfer | MSQ | 2 | A, B, D |
| 65 | S7 – Dairy & Food Engineering | Preservation of Food | NAT | 2 | 2.15 to 2.28 |

# DETAILED SOLUTIONS

Q.1  |  Section GA – General Aptitude  |  Topic: Quantitative Aptitude  |  MCQ, 1 mark

Question: A trader marks up the price of a fertilizer bag by 25% above its cost price and then offers a discount of 20% on the marked price. Find the trader’s overall percentage profit or loss.

(A)  0% (no profit, no loss)

(B)  5% profit

(C)  5% loss

(D)  10% profit

Answer: A

Explanation:

Let cost price = 100. Marked price = 100 × 1.25 = 125. Selling price after 20% discount = 125 × 0.80 = 100. Selling price equals cost price, so there is no net profit or loss (0%).

Why others are wrong: (B) and (D) wrongly assume the discount is applied additively to the markup rather than multiplicatively on the marked price. (C) results from subtracting the discount percentage from the markup percentage (25%−20%=5%) instead of compounding them correctly.

Q.2  |  Section GA – General Aptitude  |  Topic: Quantitative Aptitude  |  MCQ, 1 mark

Question: The ratio of length to width of a rectangular field is 5:3. If the perimeter of the field is 320 m, find its area.

(A)  4800 m²

(B)  5400 m²

(C)  6000 m²

(D)  6400 m²

Answer: C

Explanation:

Let length = 5k, width = 3k. Perimeter = 2(5k+3k) = 16k = 320 m ⇒ k = 20. Length = 100 m, width = 60 m. Area = 100 × 60 = 6000 m².

Why others are wrong: (A), (B), (D) result from arithmetic slips in solving for k or in multiplying length × width (e.g. using k=16 or k=24 instead of the correct k=20).

Q.3  |  Section GA – General Aptitude  |  Topic: Verbal Aptitude  |  MCQ, 1 mark

Question: Choose the word most nearly OPPOSITE in meaning to “PARSIMONIOUS”.

(A)  Frugal

(B)  Lavish

(C)  Meticulous

(D)  Reticent

Answer: B

Explanation:

“Parsimonious” means excessively unwilling to spend money or use resources (stingy). Its opposite is “lavish”, meaning generously extravagant.

Why others are wrong: (A) “Frugal” is a near-synonym, not an opposite. (C) “Meticulous” (careful/precise) and (D) “Reticent” (reserved in speech) are unrelated in meaning to thrift.

Q.4  |  Section GA – General Aptitude  |  Topic: Analytical Aptitude  |  MCQ, 1 mark

Question: In a certain code, each letter of a word is replaced by the letter that comes one position later in the English alphabet (A→B, B→C, …, Z→A). If “WHEAT” is coded as “XIFBU”, how is “MAIZE” coded in the same scheme?

(A)  NBJAF

(B)  NBJAE

(C)  MBJAF

(D)  NCJAF

Answer: A

Explanation:

Shifting each letter of MAIZE forward by one: M→N, A→B, I→J, Z→A (wraps around), E→F. This gives NBJAF.

Why others are wrong: (B) has an error in the last letter (E should shift to F, not stay E). (C) fails to shift the first letter M. (D) incorrectly shifts A to C (a two-step shift) instead of one step to B.

Q.5  |  Section GA – General Aptitude  |  Topic: Data Interpretation  |  MCQ, 1 mark

Question: In a survey of 200 farmers, 120 use drip irrigation, 90 use sprinkler irrigation, and 40 use both drip and sprinkler irrigation. How many farmers use neither drip nor sprinkler irrigation?

(A)  10

(B)  20

(C)  30

(D)  40

Answer: C

Explanation:

By the inclusion-exclusion principle, farmers using at least one method = 120 + 90 − 40 = 170. Farmers using neither = 200 − 170 = 30.

Why others are wrong: (A), (B), (D) arise from omitting the subtraction of the overlap (40) or from adding/subtracting it incorrectly when computing the union.

Q.6  |  Section GA – General Aptitude  |  Topic: Quantitative Aptitude  |  MCQ, 2 marks

Question: A warehouse contains 900 quintals of paddy at 20% moisture content (wet basis). It is dried until the moisture content reduces to 10% (wet basis). Find the quantity of water removed, assuming no loss of dry matter.

(A)  90 quintals

(B)  100 quintals

(C)  110 quintals

(D)  120 quintals

Answer: B

Explanation:

Using the wet-basis water-removal relation: W_w = W₁(M₁−M₂)/(100−M₂) = 900 × (20−10)/(100−10) = 900 × 10/90 = 100 quintals.

Why others are wrong: (A) and (C) come from using the wrong denominator (e.g. 100−M₁ instead of 100−M₂). (D) results from applying (M₁−M₂) directly to the initial mass without correcting for the changing dry-matter basis.

Q.7  |  Section GA – General Aptitude  |  Topic: Quantitative Aptitude  |  MCQ, 2 marks

Question: Two identical irrigation pumps A and B can together fill a farm pond in 6 hours. Pump A alone takes 5 hours longer than Pump B alone to fill the pond. Find the time taken by Pump B alone to fill the pond.

(A)  8 hours

(B)  10 hours

(C)  12 hours

(D)  15 hours

Answer: B

Explanation:

Let B take x hours alone; A takes (x+5) hours. Then 1/x + 1/(x+5) = 1/6. Multiplying out: 6(x+5) + 6x = x(x+5) ⇒ 12x+30 = x²+5x ⇒ x²−7x−30=0 ⇒ (x−10)(x+3)=0 ⇒ x=10 h (rejecting the negative root). Check: 1/10+1/15 = 3/30+2/30 = 1/6 ✓.

Why others are wrong: (A), (C), (D) are values that do not satisfy the combined-rate equation 1/x+1/(x+5)=1/6 when checked directly.

Q.8  |  Section GA – General Aptitude  |  Topic: Verbal Aptitude  |  MCQ, 2 marks

Question: Read the following passage and answer the question.

“Soil organic carbon (SOC) improves water-holding capacity and nutrient availability. Studies across semi-arid regions show that a 1% increase in SOC can increase available water capacity by approximately 1.5%. However, SOC levels decline rapidly under intensive tillage and residue removal, while they recover slowly, often over decades, under conservation practices.”

Which of the following can be logically inferred from the passage?

(A)  SOC can be restored as quickly as it is depleted.

(B)  Conservation practices alone guarantee high crop yields.

(C)  Loss of SOC due to intensive tillage is easier to cause than to reverse.

(D)  Available water capacity is unrelated to SOC.

Answer: C

Explanation:

The passage states SOC “declines rapidly” under intensive tillage but “recovers slowly … over decades” under conservation practices — i.e., depletion is fast, restoration is slow, so loss is easier to cause than to reverse.

Why others are wrong: (A) directly contradicts the stated asymmetry between rapid decline and slow recovery. (B) is an unsupported overreach — the passage never claims conservation practices guarantee yields. (D) contradicts the passage, which explicitly links SOC to available water capacity.

Q.9  |  Section GA – General Aptitude  |  Topic: Analytical Aptitude  |  MCQ, 2 marks

Question: Four agricultural engineers P, Q, R and S each specialize in a different area — Farm Machinery, Irrigation, Food Processing and Soil Conservation — and each is posted in a different state — Punjab, Kerala, Odisha and Rajasthan (not necessarily in this order).

Clues: (1) The engineer posted in Punjab specializes in Farm Machinery. (2) R specializes in Irrigation and is not posted in Kerala. (3) S is posted in Odisha. (4) Q specializes in Soil Conservation. (5) P is posted in Punjab.

Which engineer specializes in Food Processing, and in which state is that engineer posted?

(A)  Q, Kerala

(B)  S, Odisha

(C)  R, Rajasthan

(D)  P, Punjab

Answer: B

Explanation:

From (5) and (1): P is in Punjab and specializes in Farm Machinery. From (3): S is in Odisha. From (4): Q specializes in Soil Conservation. From (2): R specializes in Irrigation and is not in Kerala; the remaining states for Q and R are Kerala and Rajasthan, so R must be in Rajasthan and Q in Kerala. The only specialization left unassigned is Food Processing, which must belong to S (posted in Odisha).

Why others are wrong: (A) Q is already fixed to Soil Conservation by clue (4). (C) R is fixed to Irrigation by clue (2). (D) P is fixed to Farm Machinery by clue (1).

Q.10  |  Section GA – General Aptitude  |  Topic: Spatial Aptitude  |  MCQ, 2 marks

Question: [Figure description: A solid cube is painted red on all six outer faces, then cut by two equally spaced cuts parallel to each face into 27 identical smaller cubes (a 3×3×3 arrangement), as in a Rubik’s cube.] How many of the 27 smaller cubes have exactly two faces painted red?

(A)  8

(B)  12

(C)  6

(D)  24

Answer: B

Explanation:

In a 3×3×3 cube split: corner cubes (8 of them) have 3 painted faces; face-center cubes (6 of them) have 1 painted face; the body-center cube (1) has 0 painted faces; the remaining cubes lie on edges (excluding corners) and have exactly 2 painted faces. A cube has 12 edges, and with a 3×3×3 split each edge contributes exactly one such cube, giving 12 cubes with exactly two painted faces.

Why others are wrong: (A) 8 is the count of corner cubes (3 painted faces, not 2). (C) 6 is the count of face-center cubes (1 painted face). (D) 24 double-counts or miscounts the edge cubes.

Q.11  |  Section S1 – Engineering Mathematics  |  Topic: Calculus  |  MCQ, 1 mark

Question: Evaluate: lim(x→0) [ sin(3x) / tan(5x) ]

(A)  3/5

(B)  5/3

(C)  15

(D)  1

Answer: A

Explanation:

sin(3x)/tan(5x) = [sin(3x)/3x × 3x] / [tan(5x)/5x × 5x] = (3/5) × [sin(3x)/3x] / [tan(5x)/5x]. As x→0, sin(3x)/3x →1 and tan(5x)/5x →1, so the limit = 3/5.

Why others are wrong: (B) inverts the ratio 3/5. (C) results from multiplying instead of dividing the coefficients. (D) incorrectly assumes sin and tan cancel exactly regardless of coefficients.

Q.12  |  Section S1 – Engineering Mathematics  |  Topic: Differential Equations  |  MCQ, 1 mark

Question: The differential equation dy/dx + y/x = x² is best classified as:

(A)  Linear, first order

(B)  Nonlinear, first order

(C)  Linear, second order

(D)  Exact, non-linear

Answer: A

Explanation:

The equation is of the standard first-order linear form dy/dx + P(x)y = Q(x) with P(x)=1/x and Q(x)=x², since y and dy/dx both appear to the first power with no products of y and its derivatives.

Why others are wrong: (B) is incorrect since the equation is linear in y. (C) is incorrect since only the first derivative appears. (D) is not applicable since this is a linear ODE in standard integrating-factor form, not classified by exactness (a first-order M dx+N dy=0 concept).

Q.13  |  Section S1 – Engineering Mathematics  |  Topic: Differential Equations  |  NAT, 1 mark

Question: Solve the initial value problem dy/dx = 2y, y(0) = 3. Find y(1), rounded to two decimal places. (Use e = 2.71828.)

Answer: 22.10 to 22.25

Explanation:

Separating variables: dy/y = 2 dx ⇒ ln y = 2x + C. Using y(0)=3: C = ln 3. So y = 3e^(2x). At x=1: y(1) = 3e² = 3 × 7.389 = 22.17.

Q.14  |  Section S1 – Engineering Mathematics  |  Topic: Probability and Statistics  |  MCQ, 1 mark

Question: The mean and variance of a binomial distribution are 4 and 3 respectively. Find the number of trials n.

(A)  12

(B)  16

(C)  20

(D)  8

Answer: B

Explanation:

For a binomial distribution, mean = np = 4 and variance = npq = 3. Dividing: q = 3/4, so p = 1/4. From np=4: n = 4/(1/4) = 16.

Why others are wrong: (A), (C), (D) result from arithmetic slips such as using q=1/4 directly as n, or dividing mean by variance instead of solving the two equations jointly.

Q.15  |  Section S1 – Engineering Mathematics  |  Topic: Probability and Statistics  |  NAT, 1 mark

Question: A discrete random variable X takes values 1, 2, 3, 4 with probabilities 0.1, 0.2, 0.3, 0.4 respectively. Find E(X).

Answer: 2.95 to 3.05

Explanation:

E(X) = Σ xᵢ P(xᵢ) = 1(0.1) + 2(0.2) + 3(0.3) + 4(0.4) = 0.1 + 0.4 + 0.9 + 1.6 = 3.0.

Q.16  |  Section S2 – Farm Machinery  |  Topic: Equipment for sowing, planting and fertilizer application  |  MCQ, 1 mark

Question: Which of the following seed-metering mechanisms is most suitable for precision (single-seed) placement of large, uniformly sized seeds such as maize and cotton?

(A)  Fluted roller metering

(B)  Cell-feed (horizontal plate) metering

(C)  Internal double-run metering

(D)  Star-wheel (auger) metering

Answer: B

Explanation:

Cell-feed / plate-type metering mechanisms use cells or cups sized to accept one large seed at a time from a rotating plate, making them well suited to precision singulation of large, uniform seeds like maize and cotton.

Why others are wrong: (A) fluted roller and (D) star-wheel/auger mechanisms are volumetric, continuous-flow devices suited to small grains and fertilizer, not precision singulation. (C) internal double-run metering is also a bulk/volumetric device used mainly for small seeds and fertilizer, not precision placement.

Q.17  |  Section S2 – Farm Machinery  |  Topic: Cost analysis of implements and tractors  |  NAT, 1 mark

Question: The fixed cost of operating a tractor is ₹1,80,000 per year and the variable cost is ₹250 per hour of operation. If the tractor is used for 600 hours in a year, find the total operating cost per hour of use, rounded to the nearest whole number (in ₹/h).

Answer: 548 to 552

Explanation:

Total annual cost = Fixed cost + Variable cost × hours = 1,80,000 + 250 × 600 = 1,80,000 + 1,50,000 = 3,30,000. Cost per hour = 3,30,000 / 600 = ₹550/h.

Q.18  |  Section S2 – Farm Machinery  |  Topic: Machine Design (machine elements)  |  MCQ, 1 mark

Question: In a belt drive, the driver pulley has a diameter of 200 mm and runs at 1000 rpm. The driven pulley has a diameter of 400 mm. Neglecting belt slip, the speed of the driven pulley is:

(A)  250 rpm

(B)  500 rpm

(C)  1000 rpm

(D)  2000 rpm

Answer: B

Explanation:

For a belt drive (no slip): N₂ = N₁ × D₁/D₂ = 1000 × 200/400 = 500 rpm.

Why others are wrong: (A) inverts the diameter ratio incorrectly. (C) assumes the speeds stay the same regardless of pulley size. (D) applies the diameter ratio in the wrong direction (multiplies instead of dividing).

Q.19  |  Section S3 – Farm Power  |  Topic: Sources of Power  |  MCQ, 1 mark

Question: Which of the following renewable energy sources is most suitable for continuous, on-demand power generation on a farm, largely independent of daily weather variation?

(A)  Solar photovoltaic

(B)  Wind

(C)  Biomass/biogas

(D)  Human power

Answer: C

Explanation:

Biomass/biogas can be stored (as fuel or gas) and converted to power on demand, unlike solar and wind, whose output is intermittent and tied to weather/time of day.

Why others are wrong: (A) and (B) are inherently intermittent, weather- and time-dependent sources unless paired with storage. (D) human power is limited in magnitude and duration and not suited to continuous farm power needs.

Q.20  |  Section S3 – Farm Power  |  Topic: Tractors and Power Tillers: traction theory  |  MCQ, 1 mark

Question: For a two-wheel-drive agricultural tractor operating on firm, untilled soil, the coefficient of traction typically lies in the range of:

(A)  0.85–0.90

(B)  0.60–0.70

(C)  0.30–0.40

(D)  0.10–0.15

Answer: B

Explanation:

Typical coefficients of traction are highest on concrete (~0.80–0.90), moderate on firm/untilled soil (~0.60–0.70), lower on tilled soil, and lowest on loose sand (~0.30–0.40 or less).

Why others are wrong: (A) is characteristic of a hard concrete surface, not soil. (C) and (D) are characteristic of tilled or loose/sandy soil, which offers much less traction than firm soil.

Q.21  |  Section S3 – Farm Power  |  Topic: Power transmission (clutch, gear box, differential, final drive, brakes)  |  NAT, 1 mark

Question: A tractor gearbox-and-final-drive train has an overall gear reduction ratio of 40:1 between the engine and the rear axle. If the engine runs at 2000 rpm, find the rear axle speed (in rpm).

Answer: 49 to 51

Explanation:

Rear axle speed = engine speed / overall gear ratio = 2000/40 = 50 rpm.

Q.22  |  Section S3 – Farm Power  |  Topic: Fuels, combustion and lubricants  |  NAT, 1 mark

Question: The calorific value of diesel is 45 MJ/kg. Find the energy content of 8 kg of diesel (in MJ).

Answer: 355 to 365

Explanation:

Energy content = calorific value × mass = 45 MJ/kg × 8 kg = 360 MJ.

Q.23  |  Section S4 – Soil & Water Conservation Engineering  |  Topic: Surveying and Leveling  |  MCQ, 1 mark

Question: In a dumpy-level survey, the back sight (BS) reading at a point is 2.850 m and the fore sight (FS) reading on the next point is 1.640 m. The change in elevation between the two points is:

(A)  Rise of 1.210 m

(B)  Fall of 1.210 m

(C)  Rise of 4.490 m

(D)  Fall of 4.490 m

Answer: A

Explanation:

Rise or fall = BS − FS = 2.850 − 1.640 = 1.210 m. Since BS > FS, this represents a rise of 1.210 m.

Why others are wrong: (B) incorrectly labels the change as a fall despite BS>FS. (C) and (D) result from adding BS and FS instead of subtracting them.

Q.24  |  Section S4 – Soil & Water Conservation Engineering  |  Topic: Hydrology  |  MCQ, 1 mark

Question: The time of concentration of a watershed refers to:

(A)  The total duration of a storm event

(B)  The time taken for runoff to travel from the hydraulically most remote point of the watershed to the outlet

(C)  The time between the start of rainfall and the start of runoff

(D)  The time taken for a reservoir to fill completely

Answer: B

Explanation:

Time of concentration is defined as the time required for a drop of water at the hydraulically most distant point of a watershed to reach the outlet, once the entire watershed is contributing to flow.

Why others are wrong: (A) describes storm duration, an unrelated quantity. (C) describes the "time to ponding" or initial abstraction period, not time of concentration. (D) is unrelated to watershed hydrology of runoff travel.

Q.25  |  Section S4 – Soil & Water Conservation Engineering  |  Topic: Soil and Water Erosion  |  NAT, 1 mark

Question: Using the Universal Soil Loss Equation (USLE), A = R×K×LS×C×P, with R = 400 MJ·mm/(ha·h·yr), K = 0.30 t·ha·h/(ha·MJ·mm), LS = 1.5, C = 0.2, and P = 1.0, find the estimated soil loss A (in t/ha/yr), rounded to one decimal place.

Answer: 35.5 to 36.5

Explanation:

A = R × K × LS × C × P = 400 × 0.30 × 1.5 × 0.2 × 1.0. Step-wise: 400×0.30 = 120; 120×1.5 = 180; 180×0.2 = 36. So A = 36.0 t/ha/yr.

Q.26  |  Section S4 – Soil & Water Conservation Engineering  |  Topic: Soil and Water Erosion  |  NAT, 1 mark

Question: A cultivated field has a slope-length/steepness (LS) factor of 1.8 for a slope length of 25 m, giving a soil loss (by USLE) of 40 t/ha/yr with all other USLE factors unchanged. If bench terracing reduces the effective slope length to 10 m, giving a new LS factor of 0.9, find the reduced soil loss (in t/ha/yr), with all other USLE factors unchanged.

Answer: 19 to 21

Explanation:

Since soil loss A is directly proportional to the LS factor when all other USLE factors are unchanged, the new soil loss = 40 × (0.9/1.8) = 40 × 0.5 = 20 t/ha/yr.

Q.27  |  Section S5 – Irrigation & Drainage Engineering  |  Topic: Agricultural Drainage  |  MCQ, 1 mark

Question: The primary purpose of subsurface (tile) drainage in agricultural fields is to:

(A)  Increase soil erosion

(B)  Lower the water table to remove excess soil water from the root zone

(C)  Increase salinity in the root zone

(D)  Reduce the infiltration rate of the soil

Answer: B

Explanation:

Subsurface drainage removes excess gravitational water from the root zone by lowering the water table, improving aeration and root growth.

Why others are wrong: (A) drainage generally reduces surface erosion by reducing waterlogging and surface ponding, not increasing erosion. (C) properly designed drainage typically helps control, not increase, salinity by leaching salts. (D) drainage does not directly alter the infiltration rate of the soil surface.

Q.28  |  Section S5 – Irrigation & Drainage Engineering  |  Topic: Agricultural Drainage  |  NAT, 1 mark

Question: A subsurface drain system is designed to remove excess water at a drainage coefficient of 12 mm/day from a field of area 8 hectares. Find the required drainage discharge (in m³/day), rounded to the nearest whole number.

Answer: 955 to 965

Explanation:

Discharge = drainage coefficient (m/day) × area (m²) = 0.012 m/day × (8 × 10,000 m²) = 0.012 × 80,000 = 960 m³/day.

Q.29  |  Section S6 – Agricultural Process Engineering  |  Topic: Evaporation and Drying  |  MCQ, 1 mark

Question: The falling-rate period of drying of an agricultural material is characterized by:

(A)  Constant surface moisture, with drying rate limited by external heat and mass transfer

(B)  Drying rate limited by internal moisture diffusion to the surface of the material

(C)  A drying rate that increases with time

(D)  No moisture removal from the material

Answer: B

Explanation:

In the falling-rate period, the surface is no longer saturated and the rate-limiting step becomes the internal migration (diffusion) of moisture to the surface, so the drying rate decreases with time.

Why others are wrong: (A) describes the constant-rate period, not the falling-rate period. (C) is the opposite of the observed trend in the falling-rate period. (D) is incorrect, since moisture continues to be removed, just at a decreasing rate.

Q.30  |  Section S6 – Agricultural Process Engineering  |  Topic: Engineering properties of agricultural produce  |  MCQ, 1 mark

Question: The angle of repose of a granular agricultural material is defined as:

(A)  The angle between the direction of gravity and the normal to a free-flowing heap surface

(B)  The angle made by the free surface of a heaped material with the horizontal, at which the material is on the verge of sliding

(C)  The angle between two adjacent seeds resting in a bulk

(D)  The angle of contact between a seed and a metal surface only

Answer: B

Explanation:

The angle of repose is the maximum angle, measured from the horizontal, at which a heaped granular material remains stable without sliding further; it is a key property for designing hoppers, bins and chutes.

Why others are wrong: (A) describes an unrelated geometric angle definition, not the standard angle of repose. (C) and (D) describe particle-to-particle or particle-to-surface contact angles, not the bulk heap property that the angle of repose represents.

Q.31  |  Section S6 – Agricultural Process Engineering  |  Topic: Engineering properties of agricultural produce  |  NAT, 1 mark

Question: A grain kernel has axial dimensions of length 8 mm, width 4 mm and thickness 3 mm. Find its geometric mean diameter, Dg = (L×W×T)^(1/3), in mm, rounded to two decimal places.

Answer: 4.50 to 4.65

Explanation:

Dg = (8 × 4 × 3)^(1/3) = (96)^(1/3) = 4.579 mm.

Q.32  |  Section S6 – Agricultural Process Engineering  |  Topic: Storage Systems  |  NAT, 1 mark

Question: A cylindrical grain silo has a diameter of 4 m and is filled with grain to a height of 6 m. Taking the bulk density of the grain as 750 kg/m³, find the mass of grain stored (in tonnes), rounded to one decimal place.

Answer: 56.0 to 57.0

Explanation:

Volume = πr²h = π × 2² × 6 = π × 24 = 75.40 m³. Mass = volume × bulk density = 75.40 × 750 = 56,549 kg ≈ 56.5 tonnes.

Q.33  |  Section S6 – Agricultural Process Engineering  |  Topic: Storage Systems  |  NAT, 1 mark

Question: The safe moisture content for long-term storage of wheat is approximately 12% (wet basis). A 500 kg lot of wheat currently has a moisture content of 18% (wet basis). Find the mass of water that must be removed to bring it to 12% (wet basis), rounded to the nearest whole number (in kg). (Use Ww = W₁(M₁−M₂)/(100−M₂).)

Answer: 33 to 35

Explanation:

Ww = 500 × (18−12)/(100−12) = 500 × 6/88 = 34.09 kg.

Q.34  |  Section S7 – Dairy & Food Engineering  |  Topic: Preservation of Food  |  MCQ, 1 mark

Question: Pasteurization of milk primarily aims to:

(A)  Sterilize milk completely, destroying all microorganisms including spores

(B)  Destroy pathogenic microorganisms and reduce spoilage organisms while retaining nutritional and sensory quality

(C)  Increase the fat content of milk

(D)  Remove water content from milk

Answer: B

Explanation:

Pasteurization is a mild heat treatment designed to destroy pathogens and reduce spoilage-causing organisms, while minimizing changes to the nutritional and sensory quality of milk; it is not intended to be sterilizing.

Why others are wrong: (A) describes sterilization, which is a more severe process than pasteurization and does destroy spores, unlike pasteurization. (C) and (D) describe unrelated processes (standardization/concentration), not pasteurization.

Q.35  |  Section S7 – Dairy & Food Engineering  |  Topic: Preservation of Food  |  NAT, 1 mark

Question: Milk is pasteurized by the HTST (High Temperature Short Time) method at 72°C, held for 15 seconds. If the D-value (decimal reduction time) of a target pathogen at 72°C is 3 seconds, find the number of decimal (log) reductions achieved in the pathogen population during the holding period.

Answer: 4.8 to 5.2

Explanation:

Number of log reductions = holding time / D-value = 15 s / 3 s = 5.

Q.36  |  Section S1 – Engineering Mathematics  |  Topic: Calculus  |  MCQ, 2 marks

Question: Find the value of ∫₀^(π/2) sin²x dx.

(A)  π/2

(B)  π/4

(C)  π

(D)  1

Answer: B

Explanation:

Using sin²x = (1−cos2x)/2: ∫₀^(π/2) (1−cos2x)/2 dx = (1/2)[x − sin(2x)/2] from 0 to π/2 = (1/2)[(π/2 − 0) − (0−0)] = π/4.

Why others are wrong: (A) omits the factor of 1/2 from the identity. (C) and (D) result from incorrect antiderivatives of sin²x.

Q.37  |  Section S1 – Engineering Mathematics  |  Topic: Calculus  |  NAT, 2 marks

Question: A function f(x) = x³ − 6x² + 9x + 2 has a local maximum at x = a. Find the value of a.

Answer: 0.95 to 1.05

Explanation:

f′(x) = 3x² − 12x + 9 = 3(x−1)(x−3). Critical points: x=1, x=3. f″(x) = 6x−12. At x=1: f″=−6<0 ⇒ local maximum. At x=3: f″=6>0 ⇒ local minimum. Hence a = 1.

Q.38  |  Section S1 – Engineering Mathematics  |  Topic: Linear Algebra  |  MCQ, 2 marks

Question: Find the eigenvalues of the matrix A = [[4, 1], [2, 3]].

(A)  2, 5

(B)  1, 6

(C)  3, 4

(D)  −2, 5

Answer: A

Explanation:

Characteristic equation: (4−λ)(3−λ) − (1)(2) = 0 ⇒ 12 −4λ−3λ+λ² −2 = 0 ⇒ λ² −7λ + 10 = 0 ⇒ (λ−2)(λ−5)=0 ⇒ λ = 2, 5.

Why others are wrong: (B), (C), (D) do not satisfy λ²−7λ+10=0 and arise from sign or arithmetic errors while expanding the determinant.

Q.39  |  Section S1 – Engineering Mathematics  |  Topic: Linear Algebra  |  MCQ, 2 marks

Question: Given the matrix A = [[2, 0], [0, 3]], find det(A²).

(A)  12

(B)  18

(C)  36

(D)  9

Answer: C

Explanation:

det(A) = 2×3 = 6. Since det(A²) = [det(A)]², det(A²) = 6² = 36.

Why others are wrong: (A) and (B) result from computing 2×6 or 3×6 instead of squaring det(A). (D) results from squaring only one diagonal entry (3²) instead of the full determinant.

Q.40  |  Section S2 – Farm Machinery  |  Topic: Machine Design (machine elements)  |  MCQ, 2 marks

Question: A shaft transmits 15 kW of power at 300 rpm. Find the torque transmitted by the shaft (nearest whole number, in N·m).

(A)  287 N·m

(B)  358 N·m

(C)  477 N·m

(D)  573 N·m

Answer: C

Explanation:

Power P = 2πNT/60, so T = P×60/(2πN) = (15000×60)/(2π×300) = 9,00,000/1884.96 ≈ 477.5 N·m.

Why others are wrong: (A) and (B) result from using an incorrect factor (e.g. omitting 60 or using π instead of 2π in the denominator inconsistently). (D) results from dividing by N instead of 2πN/60 correctly, effectively overestimating torque.

Q.41  |  Section S2 – Farm Machinery  |  Topic: Farm Machinery: soil tillage and forces on a tillage tool  |  NAT, 2 marks

Question: A mould-board plough has a working width of 0.30 m and operates at a depth of 0.20 m in a soil with a specific draft resistance of 50 kN/m². Find the draft required (in kN).

Answer: 2.9 to 3.1

Explanation:

Draft = specific draft resistance × width × depth = 50 kN/m² × 0.30 m × 0.20 m = 3.0 kN.

Q.42  |  Section S2 – Farm Machinery  |  Topic: Farm Machinery: soil tillage and forces on a tillage tool  |  NAT, 2 marks

Question: A tractor pulls a cultivator requiring a draft of 8 kN at a forward speed of 3.6 km/h. The tractive efficiency of the tractor is 75%. Find the power that the engine must deliver to the drive wheels (in kW), rounded to two decimal places. (First compute the drawbar power, then divide by the tractive efficiency.)

Answer: 10.60 to 10.75

Explanation:

Convert speed: 3.6 km/h = 1 m/s. Drawbar power = Draft × speed = 8000 N × 1 m/s = 8000 W = 8.00 kW. Power delivered to the wheels = drawbar power / tractive efficiency = 8.00/0.75 = 10.67 kW.

Q.43  |  Section S2 – Farm Machinery  |  Topic: Equipment for spraying  |  NAT, 2 marks

Question: A boom sprayer has 20 nozzles spaced 0.5 m apart and travels at a speed of 5 km/h. Each nozzle discharges 0.8 L/min. Find the application rate, in litres per hectare (L/ha), rounded to the nearest whole number.

Answer: 190 to 194

Explanation:

Effective swath width = 20 × 0.5 = 10 m. Total discharge = 20 × 0.8 = 16 L/min. Ground speed = 5 km/h = 5000/60 = 83.33 m/min. Area covered per minute = swath width × speed = 10 × 83.33 = 833.3 m²/min. Application rate = 16 L/min ÷ 833.3 m²/min = 0.0192 L/m² = 0.0192 × 10,000 = 192 L/ha.

Q.44  |  Section S3 – Farm Power  |  Topic: Farm Power: engine components and I.C. engine systems  |  MCQ, 2 marks

Question: In a four-stroke diesel engine, the correct sequence of the four strokes is:

(A)  Suction, Power, Compression, Exhaust

(B)  Suction, Compression, Power, Exhaust

(C)  Compression, Suction, Power, Exhaust

(D)  Power, Compression, Suction, Exhaust

Answer: B

Explanation:

The standard four-stroke cycle sequence is: Suction (intake air is drawn in), Compression (air is compressed and fuel injected near TDC), Power (combustion drives the piston down), and Exhaust (burnt gases are expelled).

Why others are wrong: (A), (C) and (D) all place the strokes out of the physically correct order in which the piston/valve events must occur.

Q.45  |  Section S3 – Farm Power  |  Topic: Sources of Power  |  NAT, 2 marks

Question: A small biogas plant produces 12 m³ of biogas per day, with a calorific value of 22 MJ/m³. This biogas is used to run a dual-fuel engine with a thermal efficiency of 25%. Find the useful work output per day (in MJ), rounded to the nearest whole number.

Answer: 64 to 68

Explanation:

Energy input per day = 12 m³ × 22 MJ/m³ = 264 MJ. Useful work output = energy input × thermal efficiency = 264 × 0.25 = 66 MJ.

Q.46  |  Section S3 – Farm Power  |  Topic: Tractors and Power Tillers: traction theory  |  NAT, 2 marks

Question: A tractor develops a drawbar pull of 12 kN while the theoretical (no-slip) travel speed is 6 km/h; due to wheel slip, the actual travel speed is measured as 5.4 km/h. Find the drawbar power delivered, based on the actual travel speed (in kW), rounded to two decimal places.

Answer: 17.90 to 18.10

Explanation:

Actual speed = 5.4 km/h = 5.4/3.6 = 1.5 m/s. Drawbar power = pull × actual speed = 12,000 N × 1.5 m/s = 18,000 W = 18.00 kW. (Note: the 10% wheel slip, computed as (6−5.4)/6, is why the actual speed, not the theoretical speed, is used to compute usable drawbar power.)

Q.47  |  Section S3 – Farm Power  |  Topic: Farm Power: thermodynamic principles and I.C. engine cycles  |  NAT, 2 marks

Question: An air-standard Otto cycle has a compression ratio of 8. Taking the ratio of specific heats γ = 1.4, find the thermal efficiency of the cycle, expressed as a percentage, rounded to one decimal place.

Answer: 56.0 to 57.0

Explanation:

Otto cycle efficiency: η = 1 − 1/r^(γ−1) = 1 − 1/8^0.4. Now 8^0.4 = e^(0.4 × ln 8) = e^(0.4×2.079) = e^0.8318 ≈ 2.297. So η = 1 − 1/2.297 = 1 − 0.4353 = 0.5647 ≈ 56.5%.

Q.48  |  Section S3 – Farm Power  |  Topic: Human engineering, ergonomics and safety  |  NAT, 2 marks

Question: A tractor operator is exposed to a whole-body, frequency-weighted RMS vibration acceleration of 0.8 m/s² continuously for 4 hours in a workday. Using the 8-hour energy-equivalent relation A(8) = A × √(T/8), where T is the exposure duration in hours, find A(8) in m/s², rounded to two decimal places.

Answer: 0.55 to 0.58

Explanation:

A(8) = 0.8 × √(4/8) = 0.8 × √0.5 = 0.8 × 0.7071 = 0.566 m/s².

Q.49  |  Section S4 – Soil & Water Conservation Engineering  |  Topic: Soil Mechanics  |  NAT, 2 marks

Question: A saturated soil sample has a bulk (saturated) unit weight of 20 kN/m³. Taking the unit weight of water as 9.81 kN/m³, find the submerged (buoyant) unit weight of the soil (in kN/m³), rounded to two decimal places.

Answer: 10.05 to 10.35

Explanation:

Submerged unit weight γ′ = γsat − γw = 20 − 9.81 = 10.19 kN/m³.

Q.50  |  Section S4 – Soil & Water Conservation Engineering  |  Topic: Fluid Mechanics  |  MCQ, 2 marks

Question: Water flows through a horizontal pipe that gradually contracts from a diameter of 300 mm to 150 mm. If the velocity in the larger section is 1 m/s, find the velocity in the smaller section, using the continuity equation.

(A)  2 m/s

(B)  4 m/s

(C)  0.5 m/s

(D)  8 m/s

Answer: B

Explanation:

By continuity, A₁V₁ = A₂V₂, so V₂ = V₁ × (D₁/D₂)² = 1 × (300/150)² = 1 × 4 = 4 m/s.

Why others are wrong: (A) uses the diameter ratio directly instead of squaring it (area ratio). (C) inverts the ratio. (D) mistakenly cubes the diameter ratio instead of squaring it.

Q.51  |  Section S4 – Soil & Water Conservation Engineering  |  Topic: Hydrology  |  NAT, 2 marks

Question: A rainfall of 60 mm falls uniformly over a catchment of 250 hectares in 2 hours. If the runoff coefficient is 0.35, find the total runoff volume generated (in m³), rounded to the nearest 100. (Use Volume = C × Rainfall(m) × Area(m²).)

Answer: 52000 to 53000

Explanation:

Rainfall = 0.060 m. Area = 250 ha = 250 × 10,000 = 25,00,000 m². Volume = C × rainfall × area = 0.35 × 0.060 × 25,00,000 = 0.35 × 1,50,000 = 52,500 m³.

Q.52  |  Section S4 – Soil & Water Conservation Engineering  |  Topic: Hydrology  |  NAT, 2 marks

Question: A small watershed of area 400 ha receives a rainfall event of intensity 30 mm/h lasting 3 hours. Using the Rational Method, Q = CIA/360 (with Q in m³/s, C dimensionless, I in mm/h, and A in hectares), and taking C = 0.4, find the peak discharge (in m³/s), rounded to two decimal places.

Answer: 13.20 to 13.45

Explanation:

Q = CIA/360 = (0.4 × 30 × 400)/360 = 4800/360 = 13.33 m³/s.

Q.53  |  Section S4 – Soil & Water Conservation Engineering  |  Topic: Fluid Mechanics  |  MCQ, 2 marks

Question: For fully developed laminar flow in a circular pipe, the ratio of the maximum velocity (at the centreline) to the average (mean) velocity is:

(A)  1

(B)  1.5

(C)  2

(D)  4

Answer: C

Explanation:

For Hagen–Poiseuille (laminar) flow in a circular pipe, the velocity profile is parabolic, and the standard result is Vmax/Vavg = 2.

Why others are wrong: (A) would imply a flat (plug-flow) profile, which is not the case for laminar flow. (B) is the ratio that applies to fully developed turbulent flow in a pipe, not laminar flow. (D) does not correspond to any standard pipe-flow velocity-profile result.

Q.54  |  Section S4 – Soil & Water Conservation Engineering  |  Topic: Soil and Water Erosion  |  NAT, 2 marks

Question: A check dam reduces the flow velocity in a gully from 2.5 m/s to 0.5 m/s. If the sediment transport capacity of the flowing water is proportional to the cube of the flow velocity, find the percentage reduction in sediment transport capacity, rounded to one decimal place.

Answer: 97.5 to 99.0

Explanation:

Capacity ∝ v³. Initial capacity ∝ 2.5³ = 15.625. Final capacity ∝ 0.5³ = 0.125. Percentage reduction = (15.625−0.125)/15.625 × 100 = 15.5/15.625 × 100 ≈ 98.4%.

Q.55  |  Section S5 – Irrigation & Drainage Engineering  |  Topic: Groundwater Hydrology  |  MSQ, 2 marks

Question: Which of the following statements regarding a confined aquifer are correct?

(A)  The piezometric surface can lie above the ground surface, resulting in flowing (artesian) wells.

(B)  A confined aquifer is bounded above and below by impermeable (or relatively impermeable) layers.

(C)  The storage coefficient of a confined aquifer is typically of the same order of magnitude as the specific yield of an unconfined aquifer.

(D)  Water is released from a confined aquifer mainly by expansion of water and compression of the aquifer matrix as pressure declines.

Answer: A, B, D

Explanation:

(A) Correct: if the piezometric surface exceeds ground elevation, wells flow without pumping (artesian condition). (B) Correct: this is the defining feature of a confined aquifer. (D) Correct: confined-aquifer storage is governed by elastic compressibility of water and the matrix, not by dewatering of pores.

(C) Incorrect: the storage coefficient of a confined aquifer is typically very small (~10⁻⁵ to 10⁻³), orders of magnitude smaller than the specific yield of an unconfined aquifer (~0.1–0.3), since it does not involve actual dewatering of pore spaces.

Q.56  |  Section S5 – Irrigation & Drainage Engineering  |  Topic: Soil-Water-Plant Relationship  |  NAT, 2 marks

Question: A soil has a field capacity of 28% and a permanent wilting point of 12% (both on a dry-weight basis). The bulk density of the soil is 1.4 g/cm³, and the effective root zone depth is 60 cm. Find the total available water capacity in the root zone (in mm), rounded to the nearest whole number.

Answer: 133 to 136

Explanation:

Available water capacity (mm) = [(FC − PWP)/100] × bulk density × root zone depth (mm) = (0.28−0.12) × 1.4 × 600 mm = 0.16 × 1.4 × 600 = 134.4 mm.

Q.57  |  Section S5 – Irrigation & Drainage Engineering  |  Topic: Irrigation Water Conveyance and Application Methods  |  NAT, 2 marks

Question: A furrow irrigation system is designed to apply a net depth of 60 mm of water. If the overall (advance + application) efficiency achieved in the field is 65%, find the gross depth of water that must be applied to deliver this net depth (in mm), rounded to one decimal place.

Answer: 91.5 to 93.0

Explanation:

Gross depth = net depth / application efficiency = 60 mm / 0.65 = 92.3 mm.

Q.58  |  Section S5 – Irrigation & Drainage Engineering  |  Topic: Wells and Pumps  |  NAT, 2 marks

Question: A centrifugal pump delivers 15 L/s of water against a total head of 18 m. If the overall efficiency of the pump is 65%, find the power input required to drive the pump (in kW), rounded to two decimal places. (Take ρ = 1000 kg/m³, g = 9.81 m/s².)

Answer: 4.00 to 4.15

Explanation:

Hydraulic (output) power = ρgQH = 1000 × 9.81 × 0.015 × 18 = 1000 × 9.81 × 0.27 = 2648.7 W = 2.6487 kW. Input power = output power / efficiency = 2.6487/0.65 = 4.075 kW.

Q.59  |  Section S6 – Agricultural Process Engineering  |  Topic: Size Reduction and Material Handling  |  MSQ, 2 marks

Question: Which of the following statements about size-reduction of agricultural/food materials are correct?

(A)  Rittinger’s law assumes that the energy required for size reduction is proportional to the new surface area created.

(B)  Kick’s law assumes that the energy required is proportional to the logarithm of the reduction ratio (ratio of initial to final particle size), independent of particle size.

(C)  Bond’s law applies only to gaseous materials and cannot be applied to solids.

(D)  The energy required for size reduction generally increases as the final particle size decreases, for a given feed size.

Answer: A, B, D

Explanation:

(A) Correct: Rittinger’s law relates energy to new surface area generated. (B) Correct: Kick’s law relates energy to the logarithm of the size-reduction ratio. (D) Correct: finer grinding always requires more specific energy than coarser grinding, for a given feed.

(C) Incorrect: Bond’s law is a standard model for crushing and grinding of solids (widely used for hammer mills, crushers, etc.), not for gaseous materials.

Q.60  |  Section S6 – Agricultural Process Engineering  |  Topic: Evaporation and Drying  |  NAT, 2 marks

Question: A single-effect evaporator concentrates milk from 10% total solids to 40% total solids. If the feed rate is 2000 kg/h, find the rate of water evaporation, in kg/h, rounded to the nearest whole number.

Answer: 1495 to 1505

Explanation:

Solids balance: F×xF = P×xP ⇒ 2000×0.10 = P×0.40 ⇒ P = 500 kg/h. Water evaporated = F − P = 2000 − 500 = 1500 kg/h.

Q.61  |  Section S6 – Agricultural Process Engineering  |  Topic: Evaporation and Drying  |  NAT, 2 marks

Question: 500 kg of a food grain with an initial moisture content of 22% (wet basis) is dried to 13% (wet basis). If the latent heat of vaporization of water is 2260 kJ/kg, find the minimum theoretical heat energy required for drying (in MJ), rounded to the nearest whole number. (Ignore sensible heat effects.)

Answer: 114 to 120

Explanation:

Water to be removed: Ww = 500×(22−13)/(100−13) = 500×9/87 = 51.72 kg. Heat required = Ww × latent heat = 51.72 × 2260 = 1,16,894 kJ ≈ 117 MJ.

Q.62  |  Section S6 – Agricultural Process Engineering  |  Topic: Size Reduction and Material Handling  |  NAT, 2 marks

Question: A hammer mill grinding maize requires 12 kWh/tonne of energy to reduce feed size from 10 mm to 2 mm, following Bond’s law: E = Kb(1/√Dp − 1/√Df), where Dp and Df are the product and feed sizes (mm). Using the same mill and the same feed size (10 mm), find the specific energy required (in kWh/tonne) to grind the maize to a final size of 1 mm, rounded to one decimal place.

Answer: 20.5 to 21.5

Explanation:

First find Kb from the given data: 12 = Kb(1/√2 − 1/√10) = Kb(0.7071−0.3162) = Kb × 0.3909 ⇒ Kb = 30.70. For Dp=1 mm: E = 30.70 × (1/√1 − 1/√10) = 30.70 × (1−0.3162) = 30.70 × 0.6838 ≈ 21.0 kWh/tonne.

Q.63  |  Section S7 – Dairy & Food Engineering  |  Topic: Heat and Mass Transfer  |  MSQ, 2 marks

Question: For steady-state, one-dimensional heat conduction through a composite wall of a cold storage (built of insulation and structural layers in series), which of the following statements are correct?

(A)  The overall thermal resistance of the wall is the sum of the individual layer resistances (series arrangement).

(B)  Increasing the thickness of the insulation layer decreases the overall heat transfer coefficient (U-value) of the wall.

(C)  The layer with the highest thermal conductivity offers the greatest resistance to heat flow.

(D)  For steady-state one-dimensional conduction, the heat flux is the same through each layer of the composite wall.

Answer: A, B, D

Explanation:

(A) Correct: for layers in series, resistances add directly, analogous to electrical resistances in series. (B) Correct: a thicker insulation layer increases resistance, which lowers the overall U-value. (D) Correct: at steady state with no heat generation, the same heat flux must pass through each layer in series.

(C) Incorrect: a HIGHER thermal conductivity corresponds to LOWER thermal resistance (resistance = thickness/conductivity), so the highest-conductivity layer offers the LEAST, not the greatest, resistance.

Q.64  |  Section S7 – Dairy & Food Engineering  |  Topic: Heat and Mass Transfer  |  MSQ, 2 marks

Question: In convective heat transfer during forced-air cooling of fruits and vegetables, which of the following statements are correct?

(A)  A higher Reynolds number generally corresponds to a higher convective heat transfer coefficient.

(B)  The Nusselt number is a dimensionless ratio of convective to conductive heat transfer.

(C)  Natural convection heat transfer coefficients are generally higher than forced convection coefficients for the same fluid and surface.

(D)  Increasing the air velocity around the product generally reduces the cooling time.

Answer: A, B, D

Explanation:

(A) Correct: higher Reynolds number (higher velocity/turbulence) generally enhances the convective heat transfer coefficient. (B) Correct: this is the standard definition of the Nusselt number, Nu = hL/k. (D) Correct: higher air velocity generally increases h, reducing cooling time for a given temperature difference.

(C) Incorrect: for the same fluid and surface, forced convection coefficients are generally HIGHER than natural convection coefficients, not lower, because forced flow enhances the rate of heat transfer at the surface.

Q.65  |  Section S7 – Dairy & Food Engineering  |  Topic: Preservation of Food  |  NAT, 2 marks

Question: A refrigerated cold store cools 200 kg of fruit from an initial temperature of 25°C to a final temperature of 4°C. The fruit has a specific heat of 3.8 kJ/(kg·K). If the refrigeration system operates for 2 hours to achieve this cooling, find the average rate of heat removal (in kW), rounded to two decimal places.

Answer: 2.15 to 2.28

Explanation:

Heat to be removed = m×c×ΔT = 200 × 3.8 × (25−4) = 200 × 3.8 × 21 = 15,960 kJ. Time = 2 h = 7200 s. Average rate = 15,960 kJ / 7200 s = 2.2167 kW.

# BLUEPRINT VERIFICATION TABLE

1. Section × Marks × Question-Type Counts (as produced in this paper)

| Section | Q. count | Marks | MCQ | MSQ | NAT | Target Marks | Match? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GA – General Aptitude | 10 | 15 | 10 | 0 | 0 | 15 | YES |
| S1 – Engineering Mathematics | 9 | 13 | 6 | 0 | 3 | 13 | YES |
| S2 – Farm Machinery | 7 | 11 | 3 | 0 | 4 | 11 | YES |
| S3 – Farm Power | 9 | 14 | 3 | 0 | 6 | 14 | YES |
| S4 – Soil & Water Conservation Engineering | 10 | 16 | 4 | 0 | 6 | 16 | YES |
| S5 – Irrigation & Drainage Engineering | 6 | 10 | 1 | 1 | 4 | 10 | YES |
| S6 – Agricultural Process Engineering | 9 | 13 | 2 | 1 | 6 | 13 | YES |
| S7 – Dairy & Food Engineering | 5 | 8 | 1 | 2 | 2 | 8 | YES |
| TOTAL | 65 | 100 | 30 | 4 | 31 | 100 | YES |

2. Self-Check Summary

☑  Exactly 65 questions, exactly 100 marks.  65 questions, 100 marks.

☑  Q.1–Q.5 and Q.11–Q.35 are 1 mark; Q.6–Q.10 and Q.36–Q.65 are 2 marks.  Verified — all questions carry the mark value dictated by their serial number range (see Answer Key).

☑  Section marks match the mandatory blueprint exactly.  Verified — see table above; all sections show YES.

☑  Type counts match the blueprint exactly (30 MCQ, 4 MSQ, 31 NAT).  Achieved: 30 MCQ, 4 MSQ, 31 NAT.

☑  Sub-topic marks match the blueprint (rotation-pool swaps noted).  Verified against the sub-topic weightage table; rotation-pool notes below.

☑  Every NAT question states its unit and rounding, and the key gives a tolerance range.  Verified — see Question Paper and Answer Key.

☑  Every numerical answer has been independently re-derived and matches the key.  Verified — see step-by-step Detailed Solutions.

☑  No question duplicates a past GATE AG question.  All scenarios, numerical data and framing are original to this mock paper.

☑  Every question is solvable from the data given in it.  Verified — no question depends on external data books or figures not fully described in text.

Rotation-pool notes for this set (Set 1):

•  S1 (Engineering Mathematics): core sub-topics (Calculus, Linear Algebra, Differential Equations, Probability & Statistics) used at full prescribed weightage; no rotation-pool swap invoked in this set.

•  S2 (Farm Machinery): core sub-topics only; rotation-pool items (harvesting/threshing, overload safety devices, precision agriculture) not invoked in this set.

•  S3 (Farm Power): core sub-topics only; rotation-pool items (steering/hydraulics, three-point hitch, tractor tests) not invoked in this set.

•  S4 (Soil & Water Conservation Engineering): core sub-topics only; Watershed Management (rotation pool) not invoked in this set.

•  S6 (Agricultural Process Engineering): core sub-topics only; Processing of Agricultural Produce (rotation pool) not invoked in this set.

•  S7 (Dairy & Food Engineering): core sub-topics only; Unit Operations in Dairy and Food Engineering (rotation pool) not invoked in this set.