GATE 2027 — Agricultural Engineering (AG)

Full-Length Mock Paper — SET 5

Trend-Calibrated Edition · Difficulty: ACTUAL EXAM LEVEL

| Paper Code | AG (Agricultural Engineering) |
| --- | --- |
| Duration | 180 minutes (3 hours) |
| Total Questions | 65 |
| Maximum Marks | 100 |
| Sections | General Aptitude (GA) + Agricultural Engineering (AG) subject |

## Instructions — GATE Marking Scheme

• Q.1–Q.5 (GA) carry 1 mark each; Q.6–Q.10 (GA) carry 2 marks each. GA is 100% MCQ.

• Q.11–Q.35 (AG subject) carry 1 mark each (25 questions); Q.36–Q.65 (AG subject) carry 2 marks each (30 questions).

• MCQ (four options, exactly one correct): −1/3 mark for a wrong 1-mark MCQ; −2/3 mark for a wrong 2-mark MCQ.

• MSQ (four options, one or more correct): no negative marking; no partial credit — all correct options must be selected.

• NAT (numerical answer type, no options): no negative marking. Required unit and rounding instruction are stated in each question.

## Basis for This Paper — Trend-Analysis Rationale

This paper is calibrated against GATE AG syllabus-weightage analyses and post-exam paper-analysis reports spanning 2019–2025, rather than an idealized or generic syllabus split. Key calibration points:

• Soil & Water Conservation Engineering (SWCE) is the highest-weighted technical section (16 marks here), with Hydrology and Soil/Water Erosion as its heaviest sub-topics.

• Farm Power is the next-heaviest technical section (14 marks), driven by IC engine cycles, tractor traction theory, and power transmission.

• Engineering Mathematics holds steady at 13 marks (9 questions), calculative in nature, with Calculus and Linear Algebra dominant; this set uses the standard split with no rotation-pool swap-in.

• Farm Machinery (11 marks) and Agricultural Process Engineering (13 marks) are mid-weighted.

• Irrigation & Drainage Engineering (10 marks) and Dairy & Food Engineering (8 marks) remain the lowest-weighted technical sections.

• General Aptitude is fixed at 15 marks (Q.1–Q.10).

• Question-type mix follows the observed 30 MCQ / 4 MSQ / 31 NAT pattern.

DIFFICULTY NOTE: This is Set 5, generated at ACTUAL EXAM LEVEL — calibrated to match the observed "easy-to-moderate" difficulty of the real GATE AG paper rather than the harder moderate-to-hard default used for earlier sets. Almost every question applies a single governing formula directly with one substitution step; multi-step chaining is minimal and used only where the real exam itself typically requires it (e.g. a two-quantity mass/energy balance). No rotation-pool swap-in was used for this set — Engineering Mathematics follows its standard Calculus/Linear-Algebra/Differential-Equations/Probability split.

# Question Paper

General Aptitude

Q.1  (1 mark)  [MCQ]

Choose the word most nearly SIMILAR in meaning to "DILIGENT".

(A) Lazy

(B) Industrious

(C) Careless

(D) Hesitant

Q.2  (1 mark)  [MCQ]

Fill in the blank: "The new harvester was found to be highly ______, completing the work in half the usual time."

(A) inefficient

(B) efficient

(C) fragile

(D) obsolete

Q.3  (1 mark)  [MCQ]

Choose the correctly spelled word.

(A) Recieve

(B) Receive

(C) Receeve

(D) Receve

Q.4  (1 mark)  [MCQ]

Find the next number in the series: 2, 4, 8, 16, ___?

(A) 32

(B) 24

(C) 30

(D) 20

Q.5  (1 mark)  [MCQ]

The table below shows the monthly rainfall recorded at a station:

Month | Rainfall (mm)

January | 20

February | 15

March | 40

April | 55

Which month recorded the HIGHEST rainfall?

(A) January

(B) February

(C) March

(D) April

Q.6  (2 marks)  [MCQ]

A crop yield increased from 2400 kg/ha to 3000 kg/ha. What is the percentage increase?

(A) 25%

(B) 20%

(C) 30%

(D) 15%

Q.7  (2 marks)  [MCQ]

The average of five numbers is 24. If one number is excluded, the average of the remaining four becomes 22. What is the excluded number?

(A) 32

(B) 28

(C) 30

(D) 34

Q.8  (2 marks)  [MCQ]

A works twice as fast as B. Working together, they complete a task in 6 days. How many days would B alone take to complete the task?

(A) 18 days

(B) 12 days

(C) 9 days

(D) 24 days

Q.9  (2 marks)  [MCQ]

A person walks 5 km north, then 3 km east, then 5 km south. How far is he from the starting point, and in which direction?

(A) 3 km, East

(B) 8 km, East

(C) 3 km, North

(D) 5 km, East

Q.10  (2 marks)  [MCQ]

A square sheet of paper is folded in half twice (so that it is folded into four layers), and a small triangle is cut from the corner where all the folds meet. When the paper is fully unfolded, how many holes will appear in the sheet?

(A) 4

(B) 2

(C) 8

(D) 1

Engineering Mathematics

Q.11  (1 mark)  [MCQ]

Find d/dx [sin(x²)].

(A) 2x cos(x²)

(B) cos(x²)

(C) 2x sin(x²)

(D) −2x cos(x²)

Q.12  (1 mark)  [MCQ]

Evaluate ∫₀⁴ x dx.

(A) 8

(B) 16

(C) 4

(D) 32

Q.13  (1 mark)  [MCQ]

The determinant of the matrix [[2,0],[0,3]] is:

(A) 6

(B) 5

(C) 0

(D) −6

Q.14  (1 mark)  [MCQ]

If A is a 2×2 identity matrix, its eigenvalues are:

(A) 1 and 1

(B) 0 and 0

(C) 1 and 0

(D) 2 and 2

Q.15  (1 mark)  [NAT]

If f(x) = 3x² − 2x + 1, find f′(2).

Farm Machinery

Q.16  (1 mark)  [MCQ]

The horizontal component of the force required to pull a tillage implement through the soil is termed:

(A) Draft

(B) Vertical force

(C) Side draft

(D) Rolling resistance

Q.17  (1 mark)  [MCQ]

If the annual fixed cost of a farm implement is ₹40,000 and it is used for 400 hours per year, the fixed cost per hour is:

(A) ₹100

(B) ₹80

(C) ₹120

(D) ₹160

Q.18  (1 mark)  [NAT]

A driving pulley of diameter 300 mm rotates at 600 rpm and drives a second pulley of diameter 200 mm via a belt (no slip). Find the speed of the second pulley (rpm).

Farm Power

Q.19  (1 mark)  [MCQ]

A wind turbine rated at 5 kW operates at an average capacity factor of 30%. Find the average power output.

(A) 1.5 kW

(B) 3 kW

(C) 0.5 kW

(D) 5 kW

Q.20  (1 mark)  [MCQ]

The ratio of the drawbar pull to the dynamic weight on the driving wheels is called the:

(A) Coefficient of traction

(B) Slip ratio

(C) Rolling resistance coefficient

(D) Motion resistance ratio

Q.21  (1 mark)  [NAT]

A four-stroke engine has a bore of 80 mm and a stroke of 100 mm. Calculate the swept volume per cylinder (cm³). (Round off to two decimal places.)

Q.22  (1 mark)  [NAT]

A farm worker performs manual weeding at an energy expenditure rate of 4.5 kcal/min. Calculate the total energy expended during a 45-minute weeding session (kcal).

Soil & Water Conservation Engineering

Q.23  (1 mark)  [MCQ]

The portion of rainfall that does not contribute to direct runoff or infiltration but is lost to interception, depression storage, and evaporation is termed:

(A) Abstraction losses

(B) Effective rainfall

(C) Base flow

(D) Direct runoff

Q.24  (1 mark)  [MCQ]

Bernoulli's equation is derived by applying the principle of conservation of:

(A) Energy

(B) Mass

(C) Momentum

(D) Angular momentum

Q.25  (1 mark)  [NAT]

Using the USLE, with R = 200, K = 0.4, LS = 1.0, C = 0.6, and P = 1.0, calculate the estimated soil loss (t/ha/yr).

Q.26  (1 mark)  [NAT]

A soil sample has a bulk density of 1.5 g/cm³ and a particle density of 2.60 g/cm³. Calculate its porosity (%). (Round off to two decimal places.)

Irrigation & Drainage Engineering

Q.27  (1 mark)  [MCQ]

The soil moisture content at which plants can no longer extract water and wilt permanently is called:

(A) Permanent wilting point

(B) Field capacity

(C) Saturation point

(D) Hygroscopic coefficient

Q.28  (1 mark)  [NAT]

An aquifer has a hydraulic conductivity of 18 m/day and a hydraulic gradient of 0.006. Calculate the specific discharge (m/day). (Round off to three decimal places.)

Agricultural Process Engineering

Q.29  (1 mark)  [MCQ]

The ratio of the surface area of a particle to its volume is termed:

(A) Specific surface area

(B) Sphericity

(C) Bulk density

(D) Porosity

Q.30  (1 mark)  [MCQ]

The safe moisture content (wet basis) for long-term storage of paddy (rice) is approximately:

(A) 13–14%

(B) 20%

(C) 8%

(D) 18%

Q.31  (1 mark)  [NAT]

Using Rittinger's law E = KR(1/Df − 1/Di), if KR = 10 kJ·mm/kg, Di = 6 mm, and Df = 2 mm, calculate the energy required (kJ/kg). (Round off to two decimal places.)

Q.32  (1 mark)  [NAT]

A juice concentrator processes 500 kg/h of juice, concentrating it from 12% to 45% total solids. Calculate the mass of water evaporated (kg/h). (Round off to two decimal places.)

Q.33  (1 mark)  [NAT]

Wheat grains have axial dimensions L = 6 mm, W = 3 mm, T = 2.5 mm. Calculate the geometric mean diameter (mm). (Round off to two decimal places.)

Dairy & Food Engineering

Q.34  (1 mark)  [MCQ]

Ultra-High Temperature (UHT) processing of milk typically involves heating to about:

(A) 135–140°C for a few seconds

(B) 72°C for 15 seconds

(C) 63°C for 30 minutes

(D) 100°C for 10 minutes

Q.35  (1 mark)  [NAT]

Milk flowing at 1500 kg/h is heated from 10°C to 65°C. If the specific heat of milk is 3.9 kJ/(kg·K), calculate the heat required (kW). (Round off to two decimal places.)

Engineering Mathematics

Q.36  (2 marks)  [MCQ]

Evaluate ∫₀² 3x² dx.

(A) 8

(B) 12

(C) 6

(D) 4

Q.37  (2 marks)  [MCQ]

The determinant of the matrix [[1,2],[3,4]] is:

(A) −2

(B) 2

(C) 10

(D) −10

Q.38  (2 marks)  [NAT]

Solve the initial value problem dy/dx = 3x², y(0) = 1. Find y(2).

Q.39  (2 marks)  [NAT]

A fair six-sided die is rolled once. What is the probability of getting a number greater than 4? (Round off to two decimal places.)

Farm Machinery

Q.40  (2 marks)  [MCQ]

A plough requires a draft of 6 kN while operating at a forward speed of 1 m/s. What is the drawbar power required?

(A) 6 kW

(B) 3 kW

(C) 12 kW

(D) 0.6 kW

Q.41  (2 marks)  [NAT]

A simple gear pair has a driving gear with 40 teeth rotating at 500 rpm, meshed with a driven gear of 20 teeth. Find the driven gear speed (rpm).

Q.42  (2 marks)  [NAT]

A knapsack sprayer discharges at a rate of 0.5 L/min and is used to cover a plot of 500 m² in 10 minutes. Calculate the application rate (L/ha).

Q.43  (2 marks)  [NAT]

A seed drill covers a working width of 2.1 m at a forward speed of 4 km/h, with a field efficiency of 75%. Calculate the effective field capacity (ha/h). (Round off to two decimal places.)

Farm Power

Q.44  (2 marks)  [MCQ]

A tractor has a dynamic rear-wheel load of 20 kN and a coefficient of traction of 0.6. Ignoring rolling resistance, calculate the maximum tractive force.

(A) 12 kN

(B) 20 kN

(C) 6 kN

(D) 33.3 kN

Q.45  (2 marks)  [NAT]

Using the Otto cycle efficiency formula η = 1 − (1/r^(γ−1)), calculate the thermal efficiency (%) for a compression ratio r = 6 and γ = 1.4. (Round off to two decimal places.)

Q.46  (2 marks)  [NAT]

A shaft transmits 20 kW of power at 300 rpm. Calculate the torque transmitted (N·m). (Round off to two decimal places.)

Q.47  (2 marks)  [NAT]

Diesel has a calorific value of 45 MJ/kg and a density of 0.84 kg/L. Calculate the energy content per litre of diesel (MJ/L).

Q.48  (2 marks)  [NAT]

A biogas plant produces gas with a calorific value of 22 MJ/m³. If 15 m³ of biogas is produced per day, calculate the total energy content (MJ/day).

Soil & Water Conservation Engineering

Q.49  (2 marks)  [MCQ]

A 6-hour unit hydrograph (defined for 1 cm of effective rainfall) has a peak discharge of 40 m³/s. Estimate the peak discharge for a storm producing 3 cm of effective rainfall in the same duration.

(A) 120 m³/s

(B) 13.3 m³/s

(C) 43 m³/s

(D) 80 m³/s

Q.50  (2 marks)  [MCQ]

Water discharges through a sharp-edged orifice of diameter 50 mm under a head of 2 m. Using Cd = 0.62 and Q = Cd·A·√(2gh), calculate the discharge.

(A) 7.62 L/s

(B) 12.3 L/s

(C) 4.9 L/s

(D) 15.2 L/s

Q.51  (2 marks)  [NAT]

A field's soil loss without any conservation practice (C=1, P=1) is 160 t/ha/yr by USLE. If a cover-management practice with C=0.25 is adopted (P remains 1), calculate the new soil loss (t/ha/yr).

Q.52  (2 marks)  [NAT]

The reduced level (RL) of a benchmark is 150.000 m. A backsight of 2.115 m is taken on the benchmark, and a foresight of 1.320 m is taken on the next point. Find the RL of the new point (m). (Round off to two decimal places.)

Q.53  (2 marks)  [NAT]

Using Horton's infiltration equation f = fc + (f0−fc)e^(−kt), with f0 = 12 cm/h, fc = 3 cm/h, and k = 0.5/h, calculate the infiltration rate at t = 2 hours. (Round off to two decimal places.)

Q.54  (2 marks)  [NAT]

Water flows in a rectangular channel of width 2 m and depth 1 m, with Manning's n = 0.020 and bed slope 0.001. Calculate the mean velocity using V = (1/n)R^(2/3)S^(1/2). (Round off to two decimal places.)

Irrigation & Drainage Engineering

Q.55  (2 marks)  [MSQ]

Which of the following statements about tube wells and pumps used in irrigation are CORRECT?

(A) Specific capacity of a well is defined as the discharge per unit drawdown.

(B) An increase in drawdown always results in a proportionally equal increase in discharge, regardless of aquifer or well characteristics.

(C) A submersible pump has its motor coupled directly to the pump, with both installed below the water level.

(D) The efficiency of a centrifugal pump is generally highest near its rated (design) discharge, known as the best efficiency point (BEP).

Q.56  (2 marks)  [NAT]

A sprinkler system applies water at a rate of 12 mm/h. Calculate the time needed to apply a total depth of 30 mm (hours). (Round off to two decimal places.)

Q.57  (2 marks)  [NAT]

A tube well has a specific capacity of 8 L/s per metre of drawdown. Calculate the discharge (L/s) for a drawdown of 4 m.

Q.58  (2 marks)  [NAT]

Using the simplified steady-state drain-spacing equation S² = 4K·de·h/q, with K = 1.0 m/day, de = 2 m, h = 0.4 m, and q = 0.005 m/day, calculate the drain spacing S (m). (Round off to two decimal places.)

Agricultural Process Engineering

Q.59  (2 marks)  [MSQ]

Which of the following statements about moisture sorption isotherms of food materials are CORRECT?

(A) The equilibrium moisture content (EMC) of a hygroscopic material increases with increasing relative humidity of the surrounding air, at constant temperature.

(B) At a given relative humidity, the EMC obtained during desorption (drying) is generally equal to or greater than that obtained during adsorption (wetting) — a phenomenon known as hysteresis.

(C) The monolayer moisture value, often estimated using the BET equation, generally represents the moisture content at which a food is most stable against oxidative and microbial deterioration.

(D) Water activity (aw) is defined as the ratio of the vapour pressure of water in food to the vapour pressure of pure water at the same temperature, and always exceeds 1 for moist foods.

Q.60  (2 marks)  [NAT]

Using Bond's law E = 10·Wi·(1/√P80 − 1/√F80), with Wi = 10 kWh/tonne, F80 = 4000 µm, and P80 = 1000 µm, calculate the specific energy required (kWh/tonne). (Round off to two decimal places.)

Q.61  (2 marks)  [NAT]

Grain at 20% moisture content (wet basis) is dried to 14% (wet basis). The initial mass is 3000 kg. Calculate the mass of water removed (kg). (Round off to two decimal places.)

Q.62  (2 marks)  [NAT]

A dryer removes moisture at a constant rate of 0.8 kg/(m²·h) over a tray area of 4 m². Calculate the time required to remove 6.4 kg of water during the constant-rate period (h). (Round off to two decimal places.)

Dairy & Food Engineering

Q.63  (2 marks)  [MSQ]

Which of the following statements about thermal processing of foods are CORRECT?

(A) The z-value represents the temperature change required to change the D-value by a factor of 10.

(B) Commercial sterilization aims to destroy all pathogenic and toxin-forming organisms, including their spores, under normal storage conditions, though it does not necessarily achieve absolute sterility.

(C) Blanching before freezing or canning is primarily done to inactivate enzymes that would otherwise cause quality deterioration during storage.

(D) The F0-value is numerically identical to the D-value of the most heat-resistant spore present in the product, regardless of the actual process time used.

Q.64  (2 marks)  [MSQ]

Which of the following statements regarding heat transfer in food processing are CORRECT?

(A) Thermal diffusivity (α) is defined as α = k/(ρ·cp), where k is thermal conductivity, ρ is density, and cp is specific heat.

(B) A food product with a higher thermal diffusivity will heat up or cool down FASTER under the same conditions, compared to one with lower thermal diffusivity.

(C) The convective heat transfer coefficient (h) depends only on the properties of the food product and not on the surrounding fluid flow conditions.

(D) In canning, the geometric center of the can is generally the "cold point" — the last location to reach the target temperature during conduction-heating.

Q.65  (2 marks)  [NAT]

In a can-retorting process, the heating rate index (fh) = 12 minutes. Using the simplified relation log10[(TR−T0)/(TR−T)] = t/fh (assuming a lag factor of 1), with (TR−T0) = 80°C, calculate the process time t (minutes) required for (TR−T) to reduce to 8°C. (Round off to two decimal places.)

# Answer Key

| Q.No. | Section | Sub-topic | Type | Marks | Answer |
| --- | --- | --- | --- | --- | --- |
| 1 | General Aptitude | Verbal Aptitude | MCQ | 1 | B |
| 2 | General Aptitude | Verbal Aptitude | MCQ | 1 | B |
| 3 | General Aptitude | Verbal Aptitude | MCQ | 1 | B |
| 4 | General Aptitude | Analytical Aptitude | MCQ | 1 | A |
| 5 | General Aptitude | Data Interpretation | MCQ | 1 | D |
| 6 | General Aptitude | Quantitative Aptitude | MCQ | 2 | A |
| 7 | General Aptitude | Quantitative Aptitude | MCQ | 2 | A |
| 8 | General Aptitude | Quantitative Aptitude | MCQ | 2 | A |
| 9 | General Aptitude | Analytical Aptitude | MCQ | 2 | A |
| 10 | General Aptitude | Spatial Aptitude | MCQ | 2 | A |
| 11 | Engineering Mathematics | Calculus | MCQ | 1 | A |
| 12 | Engineering Mathematics | Calculus | MCQ | 1 | A |
| 13 | Engineering Mathematics | Linear Algebra | MCQ | 1 | A |
| 14 | Engineering Mathematics | Linear Algebra | MCQ | 1 | A |
| 15 | Engineering Mathematics | Calculus | NAT | 1 | 10 (Range: 9.50 to 10.50) |
| 16 | Farm Machinery | Soil tillage & forces on a tillage tool | MCQ | 1 | A |
| 17 | Farm Machinery | Cost analysis of implements and tractors | MCQ | 1 | A |
| 18 | Farm Machinery | Machine Design: machine elements | NAT | 1 | 900 (Range: 890 to 910) |
| 19 | Farm Power | Sources of Power | MCQ | 1 | A |
| 20 | Farm Power | Tractors & Power Tillers: traction theory | MCQ | 1 | A |
| 21 | Farm Power | Engine components & I.C. engine systems | NAT | 1 | 502.65 (Range: 498 to 508) |
| 22 | Farm Power | Human engineering, ergonomics and safety | NAT | 1 | 202.50 (Range: 198 to 207) |
| 23 | Soil & Water Conservation Engineering | Hydrology | MCQ | 1 | A |
| 24 | Soil & Water Conservation Engineering | Fluid Mechanics | MCQ | 1 | A |
| 25 | Soil & Water Conservation Engineering | Soil and Water Erosion | NAT | 1 | 48.00 (Range: 46.50 to 49.50) |
| 26 | Soil & Water Conservation Engineering | Soil Mechanics | NAT | 1 | 42.31 (Range: 41.00 to 43.60) |
| 27 | Irrigation & Drainage Engineering | Soil-Water-Plant Relationship | MCQ | 1 | A |
| 28 | Irrigation & Drainage Engineering | Groundwater Hydrology | NAT | 1 | 0.108 (Range: 0.100 to 0.116) |
| 29 | Agricultural Process Engineering | Engineering properties of agricultural produce | MCQ | 1 | A |
| 30 | Agricultural Process Engineering | Storage Systems | MCQ | 1 | A |
| 31 | Agricultural Process Engineering | Size Reduction and Material Handling | NAT | 1 | 3.33 (Range: 3.15 to 3.55) |
| 32 | Agricultural Process Engineering | Evaporation and Drying | NAT | 1 | 366.67 (Range: 360 to 373) |
| 33 | Agricultural Process Engineering | Engineering properties of agricultural produce | NAT | 1 | 3.56 (Range: 3.45 to 3.65) |
| 34 | Dairy & Food Engineering | Preservation of Food | MCQ | 1 | A |
| 35 | Dairy & Food Engineering | Heat and Mass Transfer | NAT | 1 | 89.38 (Range: 87.50 to 91.50) |
| 36 | Engineering Mathematics | Calculus | MCQ | 2 | A |
| 37 | Engineering Mathematics | Linear Algebra | MCQ | 2 | A |
| 38 | Engineering Mathematics | Differential Equations | NAT | 2 | 9.00 (Range: 8.50 to 9.50) |
| 39 | Engineering Mathematics | Probability and Statistics | NAT | 2 | 0.33 (Range: 0.30 to 0.36) |
| 40 | Farm Machinery | Soil tillage & forces on a tillage tool | MCQ | 2 | A |
| 41 | Farm Machinery | Machine Design: machine elements | NAT | 2 | 1000 (Range: 980 to 1020) |
| 42 | Farm Machinery | Equipment for spraying | NAT | 2 | 100.00 (Range: 95 to 105) |
| 43 | Farm Machinery | Equipment for sowing/planting/fertilizer application | NAT | 2 | 0.63 (Range: 0.60 to 0.66) |
| 44 | Farm Power | Tractors & Power Tillers: traction theory | MCQ | 2 | A |
| 45 | Farm Power | Thermodynamic principles & I.C. engine cycles | NAT | 2 | 51.16 (Range: 50.00 to 52.50) |
| 46 | Farm Power | Power transmission | NAT | 2 | 636.62 (Range: 630 to 645) |
| 47 | Farm Power | Fuels, combustion and lubricants | NAT | 2 | 37.80 (Range: 37.00 to 38.60) |
| 48 | Farm Power | Sources of Power | NAT | 2 | 330.00 (Range: 325 to 335) |
| 49 | Soil & Water Conservation Engineering | Hydrology | MCQ | 2 | A |
| 50 | Soil & Water Conservation Engineering | Fluid Mechanics | MCQ | 2 | A |
| 51 | Soil & Water Conservation Engineering | Soil and Water Erosion | NAT | 2 | 40.00 (Range: 38 to 42) |
| 52 | Soil & Water Conservation Engineering | Surveying and Leveling | NAT | 2 | 150.80 (Range: 150.65 to 150.95) |
| 53 | Soil & Water Conservation Engineering | Hydrology | NAT | 2 | 6.31 (Range: 6.10 to 6.55) |
| 54 | Soil & Water Conservation Engineering | Fluid Mechanics | NAT | 2 | 1.00 (Range: 0.93 to 1.07) |
| 55 | Irrigation & Drainage Engineering | Wells and Pumps | MSQ | 2 | A, C, D |
| 56 | Irrigation & Drainage Engineering | Irrigation Water Conveyance and Application Methods | NAT | 2 | 2.50 (Range: 2.35 to 2.65) |
| 57 | Irrigation & Drainage Engineering | Wells and Pumps | NAT | 2 | 32.00 (Range: 30.50 to 33.50) |
| 58 | Irrigation & Drainage Engineering | Agricultural Drainage | NAT | 2 | 25.30 (Range: 24.50 to 26.10) |
| 59 | Agricultural Process Engineering | Evaporation and Drying | MSQ | 2 | A, B, C |
| 60 | Agricultural Process Engineering | Size Reduction and Material Handling | NAT | 2 | 1.58 (Range: 1.48 to 1.70) |
| 61 | Agricultural Process Engineering | Storage Systems | NAT | 2 | 209.30 (Range: 200 to 220) |
| 62 | Agricultural Process Engineering | Evaporation and Drying | NAT | 2 | 2.00 (Range: 1.85 to 2.15) |
| 63 | Dairy & Food Engineering | Preservation of Food | MSQ | 2 | A, B, C |
| 64 | Dairy & Food Engineering | Heat and Mass Transfer | MSQ | 2 | A, B, D |
| 65 | Dairy & Food Engineering | Heat and Mass Transfer | NAT | 2 | 12.00 (Range: 11.50 to 12.50) |

# Detailed Solutions

General Aptitude

Q.1   |  Section: General Aptitude  |  Topic: Verbal Aptitude  |  Type: MCQ  |  Marks: 1

Choose the word most nearly SIMILAR in meaning to "DILIGENT".

(A) Lazy

(B) Industrious

(C) Careless

(D) Hesitant

Answer: (B)

Detailed Explanation:

"Diligent" means showing careful and persistent effort in work. "Industrious" (hard-working) is the closest synonym.

Why the other options are wrong:

(A) Lazy is the opposite of diligent.

(C) Careless is the opposite of diligent (which implies careful effort).

(D) Hesitant relates to indecision, unrelated to diligence.

Q.2   |  Section: General Aptitude  |  Topic: Verbal Aptitude  |  Type: MCQ  |  Marks: 1

Fill in the blank: "The new harvester was found to be highly ______, completing the work in half the usual time."

(A) inefficient

(B) efficient

(C) fragile

(D) obsolete

Answer: (B)

Detailed Explanation:

Completing work in half the usual time indicates high efficiency, so "efficient" fits the context.

Why the other options are wrong:

(A) Inefficient contradicts the stated fact that the work was completed faster.

(C) Fragile refers to physical weakness, unrelated to work speed.

(D) Obsolete means outdated, unrelated to the described performance.

Q.3   |  Section: General Aptitude  |  Topic: Verbal Aptitude  |  Type: MCQ  |  Marks: 1

Choose the correctly spelled word.

(A) Recieve

(B) Receive

(C) Receeve

(D) Receve

Answer: (B)

Detailed Explanation:

The correct spelling follows the "i before e except after c" pattern: R-E-C-E-I-V-E.

Why the other options are wrong:

(A) Reverses the 'ei' to 'ie', which is incorrect after 'c'.

(C) Adds an extra incorrect vowel combination.

(D) Omits the 'i' entirely.

Q.4   |  Section: General Aptitude  |  Topic: Analytical Aptitude  |  Type: MCQ  |  Marks: 1

Find the next number in the series: 2, 4, 8, 16, ___?

(A) 32

(B) 24

(C) 30

(D) 20

Answer: (A)

Detailed Explanation:

Each term is double the previous term (a geometric series with ratio 2). 16 × 2 = 32.

Why the other options are wrong:

(B) Does not follow the doubling pattern.

(C) Does not follow the doubling pattern.

(D) Does not follow the doubling pattern.

Q.5   |  Section: General Aptitude  |  Topic: Data Interpretation  |  Type: MCQ  |  Marks: 1

The table below shows the monthly rainfall recorded at a station:

Month | Rainfall (mm)

January | 20

February | 15

March | 40

April | 55

Which month recorded the HIGHEST rainfall?

(A) January

(B) February

(C) March

(D) April

Answer: (D)

Detailed Explanation:

A direct comparison of the table values shows April (55 mm) is the highest among the four months listed.

Why the other options are wrong:

(A) January (20 mm) is lower than April.

(B) February (15 mm) is the lowest value in the table.

(C) March (40 mm) is lower than April's 55 mm.

Q.6   |  Section: General Aptitude  |  Topic: Quantitative Aptitude  |  Type: MCQ  |  Marks: 2

A crop yield increased from 2400 kg/ha to 3000 kg/ha. What is the percentage increase?

(A) 25%

(B) 20%

(C) 30%

(D) 15%

Answer: (A)

Detailed Explanation:

Percentage increase = [(3000−2400)/2400] × 100 = (600/2400) × 100 = 25%.

Why the other options are wrong:

(B) Uses the final value instead of the original value as the base for the percentage calculation.

(C) An arbitrary nearby value not obtained from the correct calculation.

(D) An arbitrary nearby value not obtained from the correct calculation.

Q.7   |  Section: General Aptitude  |  Topic: Quantitative Aptitude  |  Type: MCQ  |  Marks: 2

The average of five numbers is 24. If one number is excluded, the average of the remaining four becomes 22. What is the excluded number?

(A) 32

(B) 28

(C) 30

(D) 34

Answer: (A)

Detailed Explanation:

Sum of 5 numbers = 5×24 = 120. Sum of remaining 4 numbers = 4×22 = 88. Excluded number = 120 − 88 = 32.

Why the other options are wrong:

(B) Does not satisfy the given sum condition when checked.

(C) Does not satisfy the given sum condition when checked.

(D) Does not satisfy the given sum condition when checked.

Q.8   |  Section: General Aptitude  |  Topic: Quantitative Aptitude  |  Type: MCQ  |  Marks: 2

A works twice as fast as B. Working together, they complete a task in 6 days. How many days would B alone take to complete the task?

(A) 18 days

(B) 12 days

(C) 9 days

(D) 24 days

Answer: (A)

Detailed Explanation:

Let B's rate = 1/x, so A's rate = 2/x. Combined rate = 3/x = 1/6, giving x = 18 days.

Why the other options are wrong:

(B) Results from an incorrect combined-rate equation.

(C) Results from an incorrect combined-rate equation.

(D) Results from an incorrect combined-rate equation.

Q.9   |  Section: General Aptitude  |  Topic: Analytical Aptitude  |  Type: MCQ  |  Marks: 2

A person walks 5 km north, then 3 km east, then 5 km south. How far is he from the starting point, and in which direction?

(A) 3 km, East

(B) 8 km, East

(C) 3 km, North

(D) 5 km, East

Answer: (A)

Detailed Explanation:

The 5 km north and 5 km south movements cancel out, leaving only the 3 km eastward displacement. Final position: 3 km east of the start.

Why the other options are wrong:

(B) Incorrectly adds the north and south distances instead of recognising they cancel out.

(C) Incorrectly retains a residual northward component after the north-south movements cancel.

(D) Uses the wrong distance value instead of the net eastward displacement.

Q.10   |  Section: General Aptitude  |  Topic: Spatial Aptitude  |  Type: MCQ  |  Marks: 2

A square sheet of paper is folded in half twice (so that it is folded into four layers), and a small triangle is cut from the corner where all the folds meet. When the paper is fully unfolded, how many holes will appear in the sheet?

(A) 4

(B) 2

(C) 8

(D) 1

Answer: (A)

Detailed Explanation:

Folding twice creates 4 layers of paper at that corner. Cutting through all 4 layers produces 4 separate holes when unfolded.

Why the other options are wrong:

(B) Corresponds to folding only once (2 layers), not twice.

(C) Would require 8 layers (three folds), not two folds.

(D) Corresponds to no folding at all, i.e., a single layer.

Engineering Mathematics

Q.11   |  Section: Engineering Mathematics  |  Topic: Calculus  |  Type: MCQ  |  Marks: 1

Find d/dx [sin(x²)].

(A) 2x cos(x²)

(B) cos(x²)

(C) 2x sin(x²)

(D) −2x cos(x²)

Answer: (A)

Detailed Explanation:

By the chain rule, d/dx[sin(u)] = cos(u)·(du/dx) with u = x², du/dx = 2x. So the derivative is 2x·cos(x²).

Why the other options are wrong:

(B) Omits the chain-rule factor of 2x from differentiating the inner function x².

(C) Differentiates sin as if it were cos(x²) directly, an incorrect substitution.

(D) Has the correct magnitude but the wrong sign.

Q.12   |  Section: Engineering Mathematics  |  Topic: Calculus  |  Type: MCQ  |  Marks: 1

Evaluate ∫₀⁴ x dx.

(A) 8

(B) 16

(C) 4

(D) 32

Answer: (A)

Detailed Explanation:

∫x dx = x²/2. Evaluated from 0 to 4: (4²/2) − (0²/2) = 16/2 = 8.

Why the other options are wrong:

(B) Uses x² directly instead of x²/2, omitting the division by 2 from integration.

(C) Uses only the upper limit value x=4 rather than the definite integral.

(D) Squares the upper limit incorrectly.

Q.13   |  Section: Engineering Mathematics  |  Topic: Linear Algebra  |  Type: MCQ  |  Marks: 1

The determinant of the matrix [[2,0],[0,3]] is:

(A) 6

(B) 5

(C) 0

(D) −6

Answer: (A)

Detailed Explanation:

For a diagonal matrix, the determinant is the product of the diagonal entries: 2 × 3 = 6.

Why the other options are wrong:

(B) Sums the diagonal entries (giving the trace, 5) instead of multiplying them.

(C) Would only be correct if one diagonal entry were zero, which is not the case here.

(D) Has the correct magnitude but an incorrect sign.

Q.14   |  Section: Engineering Mathematics  |  Topic: Linear Algebra  |  Type: MCQ  |  Marks: 1

If A is a 2×2 identity matrix, its eigenvalues are:

(A) 1 and 1

(B) 0 and 0

(C) 1 and 0

(D) 2 and 2

Answer: (A)

Detailed Explanation:

For the identity matrix, Ax = x for every vector x, so every value is an eigenvalue equal to 1. Both eigenvalues are 1.

Why the other options are wrong:

(B) Would be the eigenvalues of a zero matrix, not the identity matrix.

(C) Mixes an eigenvalue of the identity (1) with an incorrect second value (0).

(D) Eigenvalues of the identity matrix are always 1, not 2.

Q.15   |  Section: Engineering Mathematics  |  Topic: Calculus  |  Type: NAT  |  Marks: 1

If f(x) = 3x² − 2x + 1, find f′(2).

Answer: 10   (Accepted range: 9.50 to 10.50)

Detailed Explanation:

f'(x) = 6x − 2. f'(2) = 6(2) − 2 = 12 − 2 = 10.

Farm Machinery

Q.16   |  Section: Farm Machinery  |  Topic: Soil tillage & forces on a tillage tool  |  Type: MCQ  |  Marks: 1

The horizontal component of the force required to pull a tillage implement through the soil is termed:

(A) Draft

(B) Vertical force

(C) Side draft

(D) Rolling resistance

Answer: (A)

Detailed Explanation:

Draft is defined as the horizontal component of the total soil resistance force acting on a tillage implement in the direction of travel.

Why the other options are wrong:

(B) Vertical force acts perpendicular to the direction of travel, not along it.

(C) Side draft is the lateral (sideways) component, not the forward horizontal component.

(D) Rolling resistance relates to the motion of wheels, not the soil-cutting force on the implement.

Q.17   |  Section: Farm Machinery  |  Topic: Cost analysis of implements and tractors  |  Type: MCQ  |  Marks: 1

If the annual fixed cost of a farm implement is ₹40,000 and it is used for 400 hours per year, the fixed cost per hour is:

(A) ₹100

(B) ₹80

(C) ₹120

(D) ₹160

Answer: (A)

Detailed Explanation:

Fixed cost per hour = Total fixed cost / Annual usage hours = 40,000 / 400 = ₹100 per hour.

Why the other options are wrong:

(B) An arbitrary nearby value not obtained from the correct division.

(C) An arbitrary nearby value not obtained from the correct division.

(D) An arbitrary nearby value not obtained from the correct division.

Q.18   |  Section: Farm Machinery  |  Topic: Machine Design: machine elements  |  Type: NAT  |  Marks: 1

A driving pulley of diameter 300 mm rotates at 600 rpm and drives a second pulley of diameter 200 mm via a belt (no slip). Find the speed of the second pulley (rpm).

Answer: 900   (Accepted range: 890 to 910)

Detailed Explanation:

N₂ = N₁ × D₁/D₂ = 600 × 300/200 = 900 rpm.

Farm Power

Q.19   |  Section: Farm Power  |  Topic: Sources of Power  |  Type: MCQ  |  Marks: 1

A wind turbine rated at 5 kW operates at an average capacity factor of 30%. Find the average power output.

(A) 1.5 kW

(B) 3 kW

(C) 0.5 kW

(D) 5 kW

Answer: (A)

Detailed Explanation:

Average power output = Rated power × Capacity factor = 5 × 0.30 = 1.5 kW.

Why the other options are wrong:

(B) Uses an incorrect capacity factor value in the multiplication.

(C) Uses the rated power divided incorrectly instead of multiplying by the capacity factor.

(D) Ignores the capacity factor altogether, using the rated power as-is.

Q.20   |  Section: Farm Power  |  Topic: Tractors & Power Tillers: traction theory  |  Type: MCQ  |  Marks: 1

The ratio of the drawbar pull to the dynamic weight on the driving wheels is called the:

(A) Coefficient of traction

(B) Slip ratio

(C) Rolling resistance coefficient

(D) Motion resistance ratio

Answer: (A)

Detailed Explanation:

The coefficient of traction is defined as the ratio of the maximum available drawbar pull to the dynamic weight on the driving wheels.

Why the other options are wrong:

(B) Slip ratio relates travel-reduction to theoretical travel, not force to weight.

(C) Rolling resistance coefficient relates resistance force to weight for a towed/free-rolling wheel, a different concept.

(D) This is not a standard defined term matching the given ratio.

Q.21   |  Section: Farm Power  |  Topic: Engine components & I.C. engine systems  |  Type: NAT  |  Marks: 1

A four-stroke engine has a bore of 80 mm and a stroke of 100 mm. Calculate the swept volume per cylinder (cm³). (Round off to two decimal places.)

Answer: 502.65   (Accepted range: 498 to 508)

Detailed Explanation:

Vs = (π/4)D²L = (π/4)(8 cm)²(10 cm) = (π/4)(640) = 502.65 cm³.

Q.22   |  Section: Farm Power  |  Topic: Human engineering, ergonomics and safety  |  Type: NAT  |  Marks: 1

A farm worker performs manual weeding at an energy expenditure rate of 4.5 kcal/min. Calculate the total energy expended during a 45-minute weeding session (kcal).

Answer: 202.50   (Accepted range: 198 to 207)

Detailed Explanation:

Total energy = Rate × Time = 4.5 kcal/min × 45 min = 202.5 kcal.

Soil & Water Conservation Engineering

Q.23   |  Section: Soil & Water Conservation Engineering  |  Topic: Hydrology  |  Type: MCQ  |  Marks: 1

The portion of rainfall that does not contribute to direct runoff or infiltration but is lost to interception, depression storage, and evaporation is termed:

(A) Abstraction losses

(B) Effective rainfall

(C) Base flow

(D) Direct runoff

Answer: (A)

Detailed Explanation:

These initial losses (interception, depression storage, evaporation) that reduce the rainfall available for runoff are collectively termed abstraction losses (or initial abstractions).

Why the other options are wrong:

(B) Effective rainfall is the portion that DOES produce direct runoff — the opposite of what is described.

(C) Base flow refers to groundwater contribution to streamflow, unrelated to these surface losses.

(D) Direct runoff is the rainfall that reaches the stream directly, not the losses.

Q.24   |  Section: Soil & Water Conservation Engineering  |  Topic: Fluid Mechanics  |  Type: MCQ  |  Marks: 1

Bernoulli's equation is derived by applying the principle of conservation of:

(A) Energy

(B) Mass

(C) Momentum

(D) Angular momentum

Answer: (A)

Detailed Explanation:

Bernoulli's equation is an energy-conservation statement for an ideal fluid along a streamline, accounting for pressure, kinetic, and potential energy.

Why the other options are wrong:

(B) Conservation of mass gives the continuity equation, a separate principle.

(C) Conservation of momentum gives equations like the momentum/impulse equation, not Bernoulli's equation.

(D) Angular momentum conservation applies to rotational systems, not to Bernoulli's equation.

Q.25   |  Section: Soil & Water Conservation Engineering  |  Topic: Soil and Water Erosion  |  Type: NAT  |  Marks: 1

Using the USLE, with R = 200, K = 0.4, LS = 1.0, C = 0.6, and P = 1.0, calculate the estimated soil loss (t/ha/yr).

Answer: 48.00   (Accepted range: 46.50 to 49.50)

Detailed Explanation:

A = R×K×LS×C×P = 200 × 0.4 × 1.0 × 0.6 × 1.0 = 48.00 t/ha/yr.

Q.26   |  Section: Soil & Water Conservation Engineering  |  Topic: Soil Mechanics  |  Type: NAT  |  Marks: 1

A soil sample has a bulk density of 1.5 g/cm³ and a particle density of 2.60 g/cm³. Calculate its porosity (%). (Round off to two decimal places.)

Answer: 42.31   (Accepted range: 41.00 to 43.60)

Detailed Explanation:

Porosity = (1 − ρb/ρs) × 100 = (1 − 1.5/2.60) × 100 = (1 − 0.5769) × 100 = 42.31%.

Irrigation & Drainage Engineering

Q.27   |  Section: Irrigation & Drainage Engineering  |  Topic: Soil-Water-Plant Relationship  |  Type: MCQ  |  Marks: 1

The soil moisture content at which plants can no longer extract water and wilt permanently is called:

(A) Permanent wilting point

(B) Field capacity

(C) Saturation point

(D) Hygroscopic coefficient

Answer: (A)

Detailed Explanation:

The permanent wilting point is defined as the soil moisture level below which plants can no longer extract water and remain permanently wilted, even under saturated atmospheric humidity.

Why the other options are wrong:

(B) Field capacity is the upper limit of plant-available water, not the point of permanent wilting.

(C) Saturation point refers to all pore spaces being filled with water — the opposite extreme.

(D) Hygroscopic coefficient refers to water held very tightly by soil particles, a related but distinct concept.

Q.28   |  Section: Irrigation & Drainage Engineering  |  Topic: Groundwater Hydrology  |  Type: NAT  |  Marks: 1

An aquifer has a hydraulic conductivity of 18 m/day and a hydraulic gradient of 0.006. Calculate the specific discharge (m/day). (Round off to three decimal places.)

Answer: 0.108   (Accepted range: 0.100 to 0.116)

Detailed Explanation:

By Darcy's law, q = K×i = 18 × 0.006 = 0.108 m/day.

Agricultural Process Engineering

Q.29   |  Section: Agricultural Process Engineering  |  Topic: Engineering properties of agricultural produce  |  Type: MCQ  |  Marks: 1

The ratio of the surface area of a particle to its volume is termed:

(A) Specific surface area

(B) Sphericity

(C) Bulk density

(D) Porosity

Answer: (A)

Detailed Explanation:

Specific surface area is defined as the ratio of the surface area of a particle to its volume (or mass), an important property in drying, storage, and heat/mass transfer analysis.

Why the other options are wrong:

(B) Sphericity compares the particle to an equivalent sphere; it is a shape descriptor, not a surface-to-volume ratio.

(C) Bulk density is mass per unit bulk volume of a granular mass, unrelated to this surface-to-volume ratio.

(D) Porosity is the fraction of void space in a bulk material, a different property entirely.

Q.30   |  Section: Agricultural Process Engineering  |  Topic: Storage Systems  |  Type: MCQ  |  Marks: 1

The safe moisture content (wet basis) for long-term storage of paddy (rice) is approximately:

(A) 13–14%

(B) 20%

(C) 8%

(D) 18%

Answer: (A)

Detailed Explanation:

For safe long-term storage, paddy is typically dried to about 13–14% moisture content (wet basis) to prevent mould growth and insect infestation.

Why the other options are wrong:

(B) Far too high; paddy at this moisture level would spoil quickly in storage.

(C) Too low; this level risks excessive drying costs and increased grain breakage for paddy.

(D) Higher than the recommended safe storage range.

Q.31   |  Section: Agricultural Process Engineering  |  Topic: Size Reduction and Material Handling  |  Type: NAT  |  Marks: 1

Using Rittinger's law E = KR(1/Df − 1/Di), if KR = 10 kJ·mm/kg, Di = 6 mm, and Df = 2 mm, calculate the energy required (kJ/kg). (Round off to two decimal places.)

Answer: 3.33   (Accepted range: 3.15 to 3.55)

Detailed Explanation:

E = 10 × (1/2 − 1/6) = 10 × (0.5 − 0.1667) = 10 × 0.3333 = 3.33 kJ/kg.

Q.32   |  Section: Agricultural Process Engineering  |  Topic: Evaporation and Drying  |  Type: NAT  |  Marks: 1

A juice concentrator processes 500 kg/h of juice, concentrating it from 12% to 45% total solids. Calculate the mass of water evaporated (kg/h). (Round off to two decimal places.)

Answer: 366.67   (Accepted range: 360 to 373)

Detailed Explanation:

Solids balance: 500 × 0.12 = 60 kg solids/h (constant). Final product mass = 60/0.45 = 133.33 kg/h. Water evaporated = 500 − 133.33 = 366.67 kg/h.

Q.33   |  Section: Agricultural Process Engineering  |  Topic: Engineering properties of agricultural produce  |  Type: NAT  |  Marks: 1

Wheat grains have axial dimensions L = 6 mm, W = 3 mm, T = 2.5 mm. Calculate the geometric mean diameter (mm). (Round off to two decimal places.)

Answer: 3.56   (Accepted range: 3.45 to 3.65)

Detailed Explanation:

GMD = (L×W×T)^(1/3) = (6×3×2.5)^(1/3) = (45)^(1/3) = 3.56 mm.

Dairy & Food Engineering

Q.34   |  Section: Dairy & Food Engineering  |  Topic: Preservation of Food  |  Type: MCQ  |  Marks: 1

Ultra-High Temperature (UHT) processing of milk typically involves heating to about:

(A) 135–140°C for a few seconds

(B) 72°C for 15 seconds

(C) 63°C for 30 minutes

(D) 100°C for 10 minutes

Answer: (A)

Detailed Explanation:

UHT processing uses very high temperatures (typically 135–140°C) for a very short holding time (a few seconds) to achieve commercial sterility while minimizing quality loss.

Why the other options are wrong:

(B) This describes HTST pasteurization, not UHT processing.

(C) This describes LTLT (batch/vat) pasteurization, not UHT.

(D) This is far too mild and prolonged to represent UHT conditions.

Q.35   |  Section: Dairy & Food Engineering  |  Topic: Heat and Mass Transfer  |  Type: NAT  |  Marks: 1

Milk flowing at 1500 kg/h is heated from 10°C to 65°C. If the specific heat of milk is 3.9 kJ/(kg·K), calculate the heat required (kW). (Round off to two decimal places.)

Answer: 89.38   (Accepted range: 87.50 to 91.50)

Detailed Explanation:

Q = m·cp·ΔT = 1500 × 3.9 × 55 = 321,750 kJ/h = 321,750/3600 = 89.38 kW.

Engineering Mathematics

Q.36   |  Section: Engineering Mathematics  |  Topic: Calculus  |  Type: MCQ  |  Marks: 2

Evaluate ∫₀² 3x² dx.

(A) 8

(B) 12

(C) 6

(D) 4

Answer: (A)

Detailed Explanation:

∫3x² dx = x³. Evaluated from 0 to 2: 2³ − 0³ = 8.

Why the other options are wrong:

(B) Multiplies the correct antiderivative evaluation by an extra factor.

(C) Uses only 2x³ evaluation logic incorrectly.

(D) An arithmetic slip in evaluating the cube.

Q.37   |  Section: Engineering Mathematics  |  Topic: Linear Algebra  |  Type: MCQ  |  Marks: 2

The determinant of the matrix [[1,2],[3,4]] is:

(A) −2

(B) 2

(C) 10

(D) −10

Answer: (A)

Detailed Explanation:

det = (1×4) − (2×3) = 4 − 6 = −2.

Why the other options are wrong:

(B) Has the correct magnitude but the wrong sign, from reversing the subtraction order.

(C) Sums the diagonal products instead of subtracting them.

(D) Sums the diagonal products with an incorrect sign.

Q.38   |  Section: Engineering Mathematics  |  Topic: Differential Equations  |  Type: NAT  |  Marks: 2

Solve the initial value problem dy/dx = 3x², y(0) = 1. Find y(2).

Answer: 9.00   (Accepted range: 8.50 to 9.50)

Detailed Explanation:

Integrating: y = x³ + C. Using y(0)=1 gives C=1, so y = x³ + 1. At x=2: y(2) = 8 + 1 = 9.

Q.39   |  Section: Engineering Mathematics  |  Topic: Probability and Statistics  |  Type: NAT  |  Marks: 2

A fair six-sided die is rolled once. What is the probability of getting a number greater than 4? (Round off to two decimal places.)

Answer: 0.33   (Accepted range: 0.30 to 0.36)

Detailed Explanation:

Favourable outcomes are {5, 6}, i.e. 2 out of 6 equally likely outcomes. Probability = 2/6 = 0.33.

Farm Machinery

Q.40   |  Section: Farm Machinery  |  Topic: Soil tillage & forces on a tillage tool  |  Type: MCQ  |  Marks: 2

A plough requires a draft of 6 kN while operating at a forward speed of 1 m/s. What is the drawbar power required?

(A) 6 kW

(B) 3 kW

(C) 12 kW

(D) 0.6 kW

Answer: (A)

Detailed Explanation:

Drawbar power = Draft × speed = 6000 N × 1 m/s = 6000 W = 6 kW.

Why the other options are wrong:

(B) Half the correct value, from an arithmetic slip.

(C) Double the correct value.

(D) Off by a factor of 10, from a decimal-placement error.

Q.41   |  Section: Farm Machinery  |  Topic: Machine Design: machine elements  |  Type: NAT  |  Marks: 2

A simple gear pair has a driving gear with 40 teeth rotating at 500 rpm, meshed with a driven gear of 20 teeth. Find the driven gear speed (rpm).

Answer: 1000   (Accepted range: 980 to 1020)

Detailed Explanation:

N₂ = N₁ × T₁/T₂ = 500 × 40/20 = 1000 rpm.

Q.42   |  Section: Farm Machinery  |  Topic: Equipment for spraying  |  Type: NAT  |  Marks: 2

A knapsack sprayer discharges at a rate of 0.5 L/min and is used to cover a plot of 500 m² in 10 minutes. Calculate the application rate (L/ha).

Answer: 100.00   (Accepted range: 95 to 105)

Detailed Explanation:

Total discharge = 0.5 × 10 = 5 L. Area covered = 500 m² = 0.05 ha. Application rate = 5/0.05 = 100 L/ha.

Q.43   |  Section: Farm Machinery  |  Topic: Equipment for sowing/planting/fertilizer application  |  Type: NAT  |  Marks: 2

A seed drill covers a working width of 2.1 m at a forward speed of 4 km/h, with a field efficiency of 75%. Calculate the effective field capacity (ha/h). (Round off to two decimal places.)

Answer: 0.63   (Accepted range: 0.60 to 0.66)

Detailed Explanation:

Theoretical field capacity = (Width × Speed)/10 = (2.1×4)/10 = 0.84 ha/h. Effective field capacity = 0.84 × 0.75 = 0.63 ha/h.

Farm Power

Q.44   |  Section: Farm Power  |  Topic: Tractors & Power Tillers: traction theory  |  Type: MCQ  |  Marks: 2

A tractor has a dynamic rear-wheel load of 20 kN and a coefficient of traction of 0.6. Ignoring rolling resistance, calculate the maximum tractive force.

(A) 12 kN

(B) 20 kN

(C) 6 kN

(D) 33.3 kN

Answer: (A)

Detailed Explanation:

Maximum tractive force = Coefficient of traction × Dynamic load = 0.6 × 20 = 12 kN.

Why the other options are wrong:

(B) This is the given dynamic load itself, not the tractive force.

(C) Uses an incorrect coefficient value.

(D) Divides instead of multiplying by the coefficient of traction.

Q.45   |  Section: Farm Power  |  Topic: Thermodynamic principles & I.C. engine cycles  |  Type: NAT  |  Marks: 2

Using the Otto cycle efficiency formula η = 1 − (1/r^(γ−1)), calculate the thermal efficiency (%) for a compression ratio r = 6 and γ = 1.4. (Round off to two decimal places.)

Answer: 51.16   (Accepted range: 50.00 to 52.50)

Detailed Explanation:

r^(γ−1) = 6^0.4 = 2.0477. η = 1 − 1/2.0477 = 1 − 0.4884 = 0.5116 = 51.16%.

Q.46   |  Section: Farm Power  |  Topic: Power transmission  |  Type: NAT  |  Marks: 2

A shaft transmits 20 kW of power at 300 rpm. Calculate the torque transmitted (N·m). (Round off to two decimal places.)

Answer: 636.62   (Accepted range: 630 to 645)

Detailed Explanation:

T = P×60/(2πN) = (20000×60)/(2π×300) = 1,200,000/1884.96 = 636.62 N·m.

Q.47   |  Section: Farm Power  |  Topic: Fuels, combustion and lubricants  |  Type: NAT  |  Marks: 2

Diesel has a calorific value of 45 MJ/kg and a density of 0.84 kg/L. Calculate the energy content per litre of diesel (MJ/L).

Answer: 37.80   (Accepted range: 37.00 to 38.60)

Detailed Explanation:

Energy per litre = Calorific value × Density = 45 × 0.84 = 37.80 MJ/L.

Q.48   |  Section: Farm Power  |  Topic: Sources of Power  |  Type: NAT  |  Marks: 2

A biogas plant produces gas with a calorific value of 22 MJ/m³. If 15 m³ of biogas is produced per day, calculate the total energy content (MJ/day).

Answer: 330.00   (Accepted range: 325 to 335)

Detailed Explanation:

Total energy = Calorific value × Volume = 22 × 15 = 330 MJ/day.

Soil & Water Conservation Engineering

Q.49   |  Section: Soil & Water Conservation Engineering  |  Topic: Hydrology  |  Type: MCQ  |  Marks: 2

A 6-hour unit hydrograph (defined for 1 cm of effective rainfall) has a peak discharge of 40 m³/s. Estimate the peak discharge for a storm producing 3 cm of effective rainfall in the same duration.

(A) 120 m³/s

(B) 13.3 m³/s

(C) 43 m³/s

(D) 80 m³/s

Answer: (A)

Detailed Explanation:

By the proportionality principle of the unit hydrograph, ordinates scale directly with rainfall depth: Peak = 40 × 3 = 120 m³/s.

Why the other options are wrong:

(B) Divides instead of multiplying by the rainfall depth.

(C) Adds the rainfall depth to the peak discharge instead of scaling by it.

(D) Uses an incorrect scaling factor.

Q.50   |  Section: Soil & Water Conservation Engineering  |  Topic: Fluid Mechanics  |  Type: MCQ  |  Marks: 2

Water discharges through a sharp-edged orifice of diameter 50 mm under a head of 2 m. Using Cd = 0.62 and Q = Cd·A·√(2gh), calculate the discharge.

(A) 7.62 L/s

(B) 12.3 L/s

(C) 4.9 L/s

(D) 15.2 L/s

Answer: (A)

Detailed Explanation:

A = (π/4)(0.05)² = 0.0019635 m². √(2×9.81×2) = √39.24 = 6.264 m/s. Q = 0.62 × 0.0019635 × 6.264 = 0.00762 m³/s = 7.62 L/s.

Why the other options are wrong:

(B) Uses an incorrect discharge coefficient or area value, overstating the discharge.

(C) Roughly two-thirds of the correct value, from an arithmetic slip.

(D) Roughly double the correct value, from an arithmetic slip.

Q.51   |  Section: Soil & Water Conservation Engineering  |  Topic: Soil and Water Erosion  |  Type: NAT  |  Marks: 2

A field's soil loss without any conservation practice (C=1, P=1) is 160 t/ha/yr by USLE. If a cover-management practice with C=0.25 is adopted (P remains 1), calculate the new soil loss (t/ha/yr).

Answer: 40.00   (Accepted range: 38 to 42)

Detailed Explanation:

New soil loss = Original soil loss × new C-factor (since only C changes) = 160 × 0.25 = 40.00 t/ha/yr.

Q.52   |  Section: Soil & Water Conservation Engineering  |  Topic: Surveying and Leveling  |  Type: NAT  |  Marks: 2

The reduced level (RL) of a benchmark is 150.000 m. A backsight of 2.115 m is taken on the benchmark, and a foresight of 1.320 m is taken on the next point. Find the RL of the new point (m). (Round off to two decimal places.)

Answer: 150.80   (Accepted range: 150.65 to 150.95)

Detailed Explanation:

HI = RL(BM) + BS = 150.000 + 2.115 = 152.115 m. RL(new point) = HI − FS = 152.115 − 1.320 = 150.795 ≈ 150.80 m.

Q.53   |  Section: Soil & Water Conservation Engineering  |  Topic: Hydrology  |  Type: NAT  |  Marks: 2

Using Horton's infiltration equation f = fc + (f0−fc)e^(−kt), with f0 = 12 cm/h, fc = 3 cm/h, and k = 0.5/h, calculate the infiltration rate at t = 2 hours. (Round off to two decimal places.)

Answer: 6.31   (Accepted range: 6.10 to 6.55)

Detailed Explanation:

f = 3 + (12−3)e^(−0.5×2) = 3 + 9e^(−1) = 3 + 9(0.36788) = 3 + 3.311 = 6.31 cm/h.

Q.54   |  Section: Soil & Water Conservation Engineering  |  Topic: Fluid Mechanics  |  Type: NAT  |  Marks: 2

Water flows in a rectangular channel of width 2 m and depth 1 m, with Manning's n = 0.020 and bed slope 0.001. Calculate the mean velocity using V = (1/n)R^(2/3)S^(1/2). (Round off to two decimal places.)

Answer: 1.00   (Accepted range: 0.93 to 1.07)

Detailed Explanation:

A = 2×1 = 2 m², P = 2+2(1) = 4 m, R = 0.5 m. R^(2/3) = 0.630. V = (1/0.02)(0.630)(√0.001) = 50×0.630×0.03162 = 1.00 m/s.

Irrigation & Drainage Engineering

Q.55   |  Section: Irrigation & Drainage Engineering  |  Topic: Wells and Pumps  |  Type: MSQ  |  Marks: 2

Which of the following statements about tube wells and pumps used in irrigation are CORRECT?

(A) Specific capacity of a well is defined as the discharge per unit drawdown.

(B) An increase in drawdown always results in a proportionally equal increase in discharge, regardless of aquifer or well characteristics.

(C) A submersible pump has its motor coupled directly to the pump, with both installed below the water level.

(D) The efficiency of a centrifugal pump is generally highest near its rated (design) discharge, known as the best efficiency point (BEP).

Answer: (A), (C), (D)

Status of all four options:

(A) [CORRECT] True. Specific capacity is defined precisely as discharge per unit drawdown (Q/s), a standard well-performance parameter.

(B) [INCORRECT] False. Specific capacity typically DECREASES as drawdown/discharge increases, because well losses (which vary with the square of discharge) become proportionally larger — the relationship is not strictly proportional.

(C) [CORRECT] True. This is the standard definition of a submersible pump — motor and pump unit are coupled together and both operate submerged below the water level.

(D) [CORRECT] True. Pump efficiency peaks at the best efficiency point (BEP), which corresponds to the rated/design discharge; efficiency falls off at both lower and higher discharges.

Q.56   |  Section: Irrigation & Drainage Engineering  |  Topic: Irrigation Water Conveyance and Application Methods  |  Type: NAT  |  Marks: 2

A sprinkler system applies water at a rate of 12 mm/h. Calculate the time needed to apply a total depth of 30 mm (hours). (Round off to two decimal places.)

Answer: 2.50   (Accepted range: 2.35 to 2.65)

Detailed Explanation:

Time = Depth required / Application rate = 30/12 = 2.50 h.

Q.57   |  Section: Irrigation & Drainage Engineering  |  Topic: Wells and Pumps  |  Type: NAT  |  Marks: 2

A tube well has a specific capacity of 8 L/s per metre of drawdown. Calculate the discharge (L/s) for a drawdown of 4 m.

Answer: 32.00   (Accepted range: 30.50 to 33.50)

Detailed Explanation:

Discharge = Specific capacity × Drawdown = 8 × 4 = 32.00 L/s.

Q.58   |  Section: Irrigation & Drainage Engineering  |  Topic: Agricultural Drainage  |  Type: NAT  |  Marks: 2

Using the simplified steady-state drain-spacing equation S² = 4K·de·h/q, with K = 1.0 m/day, de = 2 m, h = 0.4 m, and q = 0.005 m/day, calculate the drain spacing S (m). (Round off to two decimal places.)

Answer: 25.30   (Accepted range: 24.50 to 26.10)

Detailed Explanation:

S² = 4×1.0×2×0.4/0.005 = 3.2/0.005 = 640. S = √640 = 25.30 m.

Agricultural Process Engineering

Q.59   |  Section: Agricultural Process Engineering  |  Topic: Evaporation and Drying  |  Type: MSQ  |  Marks: 2

Which of the following statements about moisture sorption isotherms of food materials are CORRECT?

(A) The equilibrium moisture content (EMC) of a hygroscopic material increases with increasing relative humidity of the surrounding air, at constant temperature.

(B) At a given relative humidity, the EMC obtained during desorption (drying) is generally equal to or greater than that obtained during adsorption (wetting) — a phenomenon known as hysteresis.

(C) The monolayer moisture value, often estimated using the BET equation, generally represents the moisture content at which a food is most stable against oxidative and microbial deterioration.

(D) Water activity (aw) is defined as the ratio of the vapour pressure of water in food to the vapour pressure of pure water at the same temperature, and always exceeds 1 for moist foods.

Answer: (A), (B), (C)

Status of all four options:

(A) [CORRECT] True. EMC rises with increasing surrounding relative humidity at a given temperature — the defining shape of a sorption isotherm.

(B) [CORRECT] True. Sorption hysteresis is a well-documented phenomenon in which the desorption curve lies above the adsorption curve at the same relative humidity.

(C) [CORRECT] True. The BET monolayer value is widely used as an indicator of the moisture content offering maximum storage stability against oxidation and microbial growth.

(D) [INCORRECT] False. Water activity is defined as p/p₀ (vapour pressure of water in food ÷ vapour pressure of pure water) and by definition ranges from 0 to 1 for foods in equilibrium; it never exceeds 1.

Q.60   |  Section: Agricultural Process Engineering  |  Topic: Size Reduction and Material Handling  |  Type: NAT  |  Marks: 2

Using Bond's law E = 10·Wi·(1/√P80 − 1/√F80), with Wi = 10 kWh/tonne, F80 = 4000 µm, and P80 = 1000 µm, calculate the specific energy required (kWh/tonne). (Round off to two decimal places.)

Answer: 1.58   (Accepted range: 1.48 to 1.70)

Detailed Explanation:

1/√1000 = 0.031623; 1/√4000 = 0.015811. Difference = 0.015812. E = 10×10×0.015812 = 1.58 kWh/tonne.

Q.61   |  Section: Agricultural Process Engineering  |  Topic: Storage Systems  |  Type: NAT  |  Marks: 2

Grain at 20% moisture content (wet basis) is dried to 14% (wet basis). The initial mass is 3000 kg. Calculate the mass of water removed (kg). (Round off to two decimal places.)

Answer: 209.30   (Accepted range: 200 to 220)

Detailed Explanation:

Dry matter (constant) = 3000×0.80 = 2400 kg. Final mass = 2400/0.86 = 2790.70 kg. Water removed = 3000 − 2790.70 = 209.30 kg.

Q.62   |  Section: Agricultural Process Engineering  |  Topic: Evaporation and Drying  |  Type: NAT  |  Marks: 2

A dryer removes moisture at a constant rate of 0.8 kg/(m²·h) over a tray area of 4 m². Calculate the time required to remove 6.4 kg of water during the constant-rate period (h). (Round off to two decimal places.)

Answer: 2.00   (Accepted range: 1.85 to 2.15)

Detailed Explanation:

Total drying rate = 0.8 × 4 = 3.2 kg/h. Time = 6.4/3.2 = 2.00 h.

Dairy & Food Engineering

Q.63   |  Section: Dairy & Food Engineering  |  Topic: Preservation of Food  |  Type: MSQ  |  Marks: 2

Which of the following statements about thermal processing of foods are CORRECT?

(A) The z-value represents the temperature change required to change the D-value by a factor of 10.

(B) Commercial sterilization aims to destroy all pathogenic and toxin-forming organisms, including their spores, under normal storage conditions, though it does not necessarily achieve absolute sterility.

(C) Blanching before freezing or canning is primarily done to inactivate enzymes that would otherwise cause quality deterioration during storage.

(D) The F0-value is numerically identical to the D-value of the most heat-resistant spore present in the product, regardless of the actual process time used.

Answer: (A), (B), (C)

Status of all four options:

(A) [CORRECT] True. The z-value is defined as the temperature increase needed to reduce the D-value (decimal reduction time) by a factor of 10, characterising the temperature-sensitivity of microbial destruction.

(B) [CORRECT] True. Commercial sterilization targets destruction of pathogens and toxin-formers (and spoilage organisms of concern) under normal storage, while some highly resistant, non-pathogenic spores may still survive — hence it is not absolute sterility.

(C) [CORRECT] True. Blanching is primarily used to inactivate enzymes (e.g., peroxidase, polyphenol oxidase) that would otherwise cause off-flavours, discoloration, and nutrient loss during frozen or canned storage.

(D) [INCORRECT] False. The F0-value depends on the actual time-temperature history of the process (it is the equivalent lethality delivered), not simply on the D-value of the target spore alone — a longer or shorter process changes F0 even for the same organism.

Q.64   |  Section: Dairy & Food Engineering  |  Topic: Heat and Mass Transfer  |  Type: MSQ  |  Marks: 2

Which of the following statements regarding heat transfer in food processing are CORRECT?

(A) Thermal diffusivity (α) is defined as α = k/(ρ·cp), where k is thermal conductivity, ρ is density, and cp is specific heat.

(B) A food product with a higher thermal diffusivity will heat up or cool down FASTER under the same conditions, compared to one with lower thermal diffusivity.

(C) The convective heat transfer coefficient (h) depends only on the properties of the food product and not on the surrounding fluid flow conditions.

(D) In canning, the geometric center of the can is generally the "cold point" — the last location to reach the target temperature during conduction-heating.

Answer: (A), (B), (D)

Status of all four options:

(A) [CORRECT] True. This is the standard defining equation for thermal diffusivity, combining conductive (k) and storage (ρ·cp) properties into a single rate parameter.

(B) [CORRECT] True. Since thermal diffusivity is a measure of how quickly heat propagates through a material relative to its heat storage, a higher α means faster heating/cooling under identical boundary conditions.

(C) [INCORRECT] False. The convective heat transfer coefficient depends strongly on the surrounding fluid flow conditions (velocity, turbulence, fluid properties), not solely on the food product's own properties.

(D) [CORRECT] True. For conduction-heated canned products, the geometric center is the point furthest from all surfaces and therefore heats up last, making it the standard 'cold point' used for process design.

Q.65   |  Section: Dairy & Food Engineering  |  Topic: Heat and Mass Transfer  |  Type: NAT  |  Marks: 2

In a can-retorting process, the heating rate index (fh) = 12 minutes. Using the simplified relation log10[(TR−T0)/(TR−T)] = t/fh (assuming a lag factor of 1), with (TR−T0) = 80°C, calculate the process time t (minutes) required for (TR−T) to reduce to 8°C. (Round off to two decimal places.)

Answer: 12.00   (Accepted range: 11.50 to 12.50)

Detailed Explanation:

log10(80/8) = log10(10) = 1. t = fh × 1 = 12 × 1 = 12.00 minutes.

# Blueprint Verification Table

Section × Marks × Question-Type Counts (as actually produced in this paper)

| Section | Questions (Actual / Target) | Marks (Actual / Target) | MCQ | MSQ | NAT |
| --- | --- | --- | --- | --- | --- |
| General Aptitude | 10 / 10 | 15 / 15 | 10 | 0 | 0 |
| Engineering Mathematics | 9 / 9 | 13 / 13 | 6 | 0 | 3 |
| Farm Machinery | 7 / 7 | 11 / 11 | 3 | 0 | 4 |
| Farm Power | 9 / 9 | 14 / 14 | 3 | 0 | 6 |
| Soil & Water Conservation Engineering | 10 / 10 | 16 / 16 | 4 | 0 | 6 |
| Irrigation & Drainage Engineering | 6 / 6 | 10 / 10 | 1 | 1 | 4 |
| Agricultural Process Engineering | 9 / 9 | 13 / 13 | 2 | 1 | 6 |
| Dairy & Food Engineering | 5 / 5 | 8 / 8 | 1 | 2 | 2 |
| TOTAL | 65 / 65 | 100 / 100 | 30 | 4 | 31 |

## Self-Check Confirmation

☑ Exactly 65 questions, exactly 100 marks (verified above).

☑ Q.1–Q.5 and Q.11–Q.35 are 1 mark; Q.6–Q.10 and Q.36–Q.65 are 2 marks — confirmed by construction.

☑ Section marks match the mandatory blueprint exactly for every section (no ±2 swing was needed or used in this set).

☑ Type counts match the mandatory blueprint: 30 MCQ, 4 MSQ, 31 NAT (verified above).

☑ Sub-topic marks follow §4 closely, with the following NOTED, deliberate minor swings (all within the same section's total, drawn from genuine sub-topic rotation rather than convenience):

• Engineering Mathematics: no rotation-pool swap was used in this set — Calculus, Linear Algebra, Differential Equations, and Probability & Statistics follow the standard nominal split exactly (5/4/2/2).

• Farm Power: Engine components & IC systems and Human engineering/ergonomics each run 1 mark lighter than the nominal split, with Power transmission and Fuels/combustion/lubricants each 1 mark heavier — a within-section reallocation, section total (14) unchanged.

• Irrigation & Drainage: Wells and Pumps runs 2 marks heavier (drawing one MSQ item) while Groundwater Hydrology and Soil-Water-Plant Relationship each run 1 mark lighter — a within-section reallocation, section total (10) unchanged.

☑ The large majority of questions apply a single governing formula with one direct substitution, consistent with the ACTUAL EXAM LEVEL difficulty requested for this set — a deliberate step down from the moderate-to-hard default and a bigger step down from Set 4's HARD calibration. Only a small number of items involve a genuine two-quantity balance or an unavoidable second step (e.g. Q.32, Q.45, Q.50, Q.54, Q.61), and even these resolve in a single clean calculation rather than a chained multi-stage derivation.

☑ Every NAT question states its required unit and rounding instruction, and the key gives a tolerance band.

☑ Every numerical answer was independently re-derived during paper construction and matches the key.

☑ No question reproduces any past GATE AG paper question — all numerical data, framing, and answers are original to this set.

☑ Every question is solvable purely from the data supplied within it; no external data book is assumed.

☑ All tables in this document use explicit visible borders with identical column widths applied to every cell in every row.