SIZE REDUCTION & MILLING

Detailed GATE Agricultural Engineering (AG) Notes

Section 6/7 — Agricultural Process Engineering

Compiled from lecture slides: cleaning/size-reduction principles, size-reduction laws, machinery, and rice milling

## Contents at a Glance

- Milling — meaning and scope

- Size reduction — definition, objectives, advantages

- Fineness modulus and average particle size

- Factors affecting size reduction

- Mechanisms of size reduction

- Principles & crushing efficiency

- Size reduction laws — Rittinger's, Kick's, Bond's (with solved numericals)

- Size reduction machinery — crushers, grinders, cutting machines

- Oil expression and extraction

- Rice milling — parboiling, milling equipment, flow

- Quick-revision formula sheet

# 1. Milling — Overview

Milling is a general trade term for the reduction of food grains into end products usable for consumption or further processing.

Milling includes:

- Cleaning

- Sorting

- Polishing

- Dehusking

- Size reduction

The exact meaning of "milling" varies from crop to crop — e.g. rice milling emphasises dehusking and polishing, while wheat milling emphasises grinding into flour.

# 2. Size Reduction — Definition & Objectives

Definition

Size reduction is the operation of reducing bigger particles into smaller ones of desired size and shape using external forces.

- Size = physical dimensions of an object

- Reduction = process of decreasing that size

- COMMINUTION is the alternate technical term for size reduction

Objectives of Size Reduction

- Increase surface area — reaction rate for solid-particle reactions is directly proportional to contact area with the second phase

- Break material into small particles to separate valuable constituents

- Achieve intimate mixing of two or more solids

- Dispose of solid wastes easily

- Improve handling characteristics of the material

Advantages of Size Reduction

- Content uniformity

- Uniform flow

- Effective drying

- Increased surface area / increased viscosity

- Uniform mixing and drying

- Improved rate of absorption (smaller particles → greater absorption)

- Improved dissolution rate

# 3. Fineness Modulus (FM)

Fineness modulus indicates the uniformity of grind in the resultant product.

Determination

FM is obtained by adding the weight fractions retained above each sieve in a stack and dividing the cumulative sum by 100.

Average Particle Size

Dp = 0.135 × (1.366)^FM     (Dp in mm)

GATE tip: Dp increases roughly exponentially with FM — memorize the constants 0.135 and 1.366 exactly; GATE numericals often ask you to back-calculate FM from a given Dp or vice-versa.

Fig. Relationship between fineness modulus and average particle size

Solved Problem — Determination of Fineness Modulus

A sieve analysis gave the following weight retained on each IS sieve. Determine the fineness modulus and average particle size.

| IS Sieve No. | Wt. of material retained (g) | % retained | Fineness modulus contribution |
| --- | --- | --- | --- |
| 100 | 0.0 | 0.0 | 7 × 0.0 = 0.0 |
| 70 | 1.4 | 0.56 | 6 × 0.56 = 3.36 |
| 50 | 16.7 | 6.71 | 5 × 6.71 = 33.53 |
| 40 | 36.7 | 14.45 | 4 × 14.45 = 57.80 |
| 30 | 82.2 | 33.01 | 3 × 33.01 = 99.03 |
| 20 | 96.0 | 38.55 | 2 × 38.55 = 77.10 |
| 15 | 8.0 | 3.21 | 1 × 3.21 = 3.21 |
| Pan | 8.7 | 3.50 | 0 × 3.50 = 0.0 |
| Total | 249.7 (≈) | 100 | 274.03 |

Fineness modulus, FM = 274.03 / 100 = 2.7403

Dp = 0.135 × (1.366)^2.7403 ≈ 0.317 mm

# 4. Factors Affecting Size Reduction

| Factor | Effect / Description |
| --- | --- |
| Hardness | A surface property of the material; harder material is more difficult to reduce in size. |
| Material structure | Homogeneous substances behave uniformly; vegetables have a cellular structure often giving long, fibrous particles. |
| Abrasiveness | A property of hard materials; abrasive materials wear equipment faster. |
| Softening temperature | Heat generated during size reduction may soften some materials; controlled by cooling the mill (water jacket / air stream). |
| Moisture content | Materials with 5–50% moisture flow poorly and tend to cake into balls; grinding works well outside this range. |
| Crushing strength | Power required for crushing is almost directly proportional to the crushing strength of the material. |
| Friability | Tendency to fracture during normal handling; crystalline materials break along defined planes, and power required rises as particle size is reduced. |

GATE tip: The 5–50% moisture "caking zone" is a favourite objective-type question — remember grinding is efficient OUTSIDE this band, not inside it.

# 5. Mechanisms of Size Reduction

| Mechanism | Description | Typical analogy / equipment |
| --- | --- | --- |
| Impact | Particle concussion by a sudden single rigid force | Hammer |
| Compression | Particle disintegration by two rigid forces | Nutcracker |
| Shear | Particle compressed between edges of two hard surfaces moving tangentially | Scissor action |
| Attrition | Particles scraping against one another or against a rigid surface | A file |
| Cutting | A sharp knife forced through the material with minimum deformation/rupture | Fruits & vegetables |

Fig. Impact, attrition, shear, and compression mechanisms

# 6. Principles of Size Reduction

Crushers and grinders are the equipment mostly used for size reduction. An ideal size reducer should have:

- Large capacity

- Ability to yield a pre-desired product or range of sizes

- Small power requirement per unit product handled

- Easy and trouble-free operation

Crushing Efficiency

Crushing efficiency = Surface energy created by crushing / Energy absorbed by the solid

# 7. Size Reduction Theories (Energy Laws)

Energy requirement for particle size reduction is a function of the input and output particle size, hardness, strength, and other properties of the solid. Three classical theories relate energy to particle size — Rittinger's Law, Kick's Law, and Bond's Law.

General differential form

Fig. 5.3 — Reduction of feed to a symmetrical smaller particle

dE = − c · (dX / X^n)

where E = energy, X = particle size, c = constant, and n depends on the theory (n = 2 for Rittinger, n = 1 for Kick, n = 1.5 for Bond).

Fig. Particle size reduction stages vs dispersion energy — from large agglomerates down to particle fragments

7.1 Rittinger's Law (n = 2)

- Work required in crushing is proportional to the new surface area created

- Assumes size reduction is a shearing process

- Energy requirement is proportional to the square of the common linear dimension

E = c ( 1/Xp − 1/Xf )

where Xp and Xf = length of product and feed, respectively.

7.2 Kick's Law (n = 1)

- Based on stress analysis of plastic deformation within the elastic limit

- Assumes energy requirement is a function of the common linear dimension of the material

- Within the elastic limit, work required to crush a given quantity of material is constant for the same reduction ratio, irrespective of the original size

- Reduction ratio = ratio of feed opening to discharge opening

E = c · ln (Xf / Xp)

7.3 Bond's Law

- Estimates the power required for crushing and grinding operations

- Work required to form particles of size Dp from a very large feed is proportional to the square root of the surface-to-volume ratio of the product

P/f = 0.3162 · wi · ( 1/√Dp − 1/√Df )

where P = power (kW), f = feed rate (t/h), Dp = sieve opening through which 80% of product passes (mm), Df = sieve opening through which 80% of feed passes (mm), wi = work index.

Work index (wi): the gross energy requirement in kWh per tonne of feed needed to reduce a very large feed to a size such that 80% of the product passes through a 100 μm screen. wi is found experimentally from laboratory crushing/grinding tests.

GATE tip: Bond's Law is the most commonly examined of the three because it is closest to industrial practice — make sure the 0.3162 constant and the "80% passing" definition of Dp/Df are second nature.

Solved Problem 1 — Rittinger's and Kick's Law

In a wheat milling experiment it was found that to grind 4.33 mm sized grains to IS sieve 30 (0.351 mm opening), the power required was 8 kW. Calculate the power requirement for milling wheat grains to IS sieve 13 (0.157 mm opening) using (i) Rittinger's Law and (ii) Kick's Law. Feed rate of milling = 200 kg/hr.

(i) According to Rittinger's Law:

E = c ( 1/Xp − 1/Xf )

8/0.2 = c (1/0.351 − 1/4.33)  ⇒  40 = c (2.619)  ⇒  c = 15.278

Putting the value of c for the second condition:

P/0.2 = 15.278 (1/0.157 − 1/4.33)

P = 18.75 kW

(ii) According to Kick's Law:

E = c · ln (Xf / Xp)

8/0.2 = c · ln(4.33/0.351)  ⇒  40 = c (2.5107)  ⇒  c = 15.93

Putting the value of c for the second condition:

P/0.2 = 15.93 · ln(4.33/0.157)

P = 10.56 kW

Solved Problem 2 — Bond's Law

How much power is required to crush 2 t/hr of a material if 80% of the feed passes through IS sieve No. 480 (4.75 mm opening) and 80% of the product passes through IS sieve No. 50 (0.5 mm opening)? Work index of the material = 6.30.

P/f = 0.3162 · wi ( 1/√Dp − 1/√Df )

Given: wi = 6.30, f = 2 t/hr, Dp = 0.50 mm, Df = 4.75 mm

P/2 = 0.3162 × 6.30 ( 1/√0.5 − 1/√4.75 )

P = 7.129 kW

# 8. Size Reduction Machinery

Size reduction machinery is broadly grouped into three classes: Crushers, Grinders, and Cutting machines.

8.1 Crushers

- Squeeze the material until it breaks; break large pieces into small lumps

- Mainly used in mining-type industries

- Types: jaw crusher, gyratory crusher, crushing rolls (smooth and serrated)

- Working mechanisms: compression, impact, and shearing

8.2 Grinders

- Used to mill grains into powder

- Types: attrition mill, hammer mill, ball mill

Ball Mill — Centrifuging & Critical Speed

Centrifuging occurs beyond a critical rotational speed of the mill.

nc = (1 / 2π) × √( g / (R − r) )

where nc = critical speed (revolutions/s), g = acceleration due to gravity (9.80 m/s²), R = radius of the mill (m), r = radius of the ball (m).

GATE tip: Operating speed for ball mills is normally kept at 65–80% of critical speed — lower values are used for wet grinding in viscous suspension. This range (65–80%) is a frequently tested fact.

8.3 Cutting Machines

- Usually used for making slices of fruits and vegetables

- Types: rotary knife cutter, Reitz mill (disintegrator)

# 9. Oil Expression and Extraction

Used to extract oil from oilseeds such as sunflower, mustard, groundnut, etc.

Mechanical Expression

- Hydraulic press, screw press

- Chemical-free process

Solvent (Chemical) Extraction

- Uses a low-boiling-point solvent to extract oil through diffusion

- High extraction efficiency (~99%), more protein retained, better preservation quality

- Solvent is recovered from the cake by heating (evaporation)

- Low toxicity of the solvents used

# 10. Rice Milling

Overall flow

Paddy → Cleaner → Storage → Sheller / Husk aspirator → Paddy separator → Whitener/Polisher → Grader → Bran aspirator → Head rice, Brokens, Bran, Bagging/Storage.

| Operation | Purpose |
| --- | --- |
| Cleaning/cleaner | Removing foreign matter from paddy |
| Shelling / rubber-roll sheller | Separating husk from the grain (paddy) |
| Husk separation / husk aspirator | Separation of husk from the product obtained from sheller |
| Paddy separation / paddy separator | Separation of paddy from brown rice |
| Bran removal / polisher-whitener | Removal of bran layers from brown rice |
| Bran aspiration / bran aspirator | Removal of bran adhering to the rice kernel |
| Grading / grader | Separation of broken rice from head rice |
| Handling equipment | Conveying paddy/rice to various processing units |

Fig. Basic flow chart of rice processing in a modern mill

Parboiling

An optional treatment given to paddy before milling. Paddy grain is composed of starch granules whose voids are filled with air/moisture.

Objective: maximize recovery of head rice and minimize breakage.

- Soaking — in normal or hot water for some time

- Steaming — temperature raised up to ~70°C

- Drying — slow drying, usually in shade or in mechanical dryers

Advantages of Parboiling

- Increases milling yield, improves quality, makes grain structure vitreous and shiny, increases shelf life

- Grains harden — greater resistance to insect attack, no germination, no atmospheric adsorption

- More oil in bran, easier shelling, more digestible

- Less sticky, swells more

- Absorbs less oil during cooking; doesn't turn rancid easily after cooking

Disadvantages of Parboiling

- Heat treatment destroys natural antioxidants; rancidity may develop during long-term storage

- Rice takes more time to cook and develops a peculiar off-flavour

- Extra capital investment (drying equipment)

- More power required during polishing

- Longer soaking time makes paddy prone to attack by spores

Parboiling Methods

| Method | Procedure |
| --- | --- |
| Traditional | Soaking in normal water for 1–2 days; steaming under atmospheric pressure; sun drying. |
| CFTRI method (modern) | Water heated to 85°C by steam; paddy added and soaked for 3–4 h; followed by drying (mechanical/sun). |
| Pressure parboiling (modern) | Soaking for 40 min at 85°C; subjected to pressurized steam for 18 min; followed by drying. Gives rice a pleasing yellowish colour. |

Head Rice vs Broken Rice

- Head rice: kernel length > 3/4 of the whole grain

- Broken rice: kernel length < 3/4 of the whole grain

Broken rice is further classified as:

  - Big brokens — 50–75% of kernel length

  - Small brokens — 12.5–50% of kernel length

  - Points — < 12.5% of kernel length

Traditional milling equipment includes hand and foot pounders.

Rice Milling / Dehusking Equipment

| Equipment | Key Features |
| --- | --- |
| Engleberg huller | Most common equipment; ribbed cast-iron roller operating at 600–900 rpm; yields 56% to up to 64%. |
| Rubber roll sheller | Two rubber rolls rotate in opposite directions (10–13 m/s and 8–10 m/s, ~25% speed differential); yields up to 95% rice; subject to major wear. |
| Under-runner disk huller | Two horizontal cast-iron disks partly covered with an abrasive emery layer; top disk fixed, bottom disk rotates; clearance is adjustable. |

Fig. 5.25 — Engleberg huller (1. hulling roller 2. screen 3. knife 4. balancing pulley 5. casing 6. straight ribs 7. spiral ribs)

Fig. 5.28 — Rubber-roll sheller

Fig. 5.26 — Under-runner disk huller

Other Milling Operations

- Husk separator

- Paddy separator (compartment type)

- Whitening — horizontal and vertical whitening cones, jet pearlers

- Glazing

- Grading — using indent-cylinder separators

GATE tip: Yield hierarchy to remember: rubber-roll sheller (~95%) > Engleberg huller (56–64%). GATE frequently tests this ranking along with the huller's rpm range (600–900 rpm).

# 11. Quick-Revision Formula Sheet

| Concept | Formula |
| --- | --- |
| Average particle size from FM | Dp = 0.135 (1.366)^FM |
| General energy law | dE = −c (dX / X^n) |
| Rittinger's Law (n = 2) | E = c (1/Xp − 1/Xf) |
| Kick's Law (n = 1) | E = c · ln(Xf / Xp) |
| Bond's Law | P/f = 0.3162 · wi (1/√Dp − 1/√Df) |
| Ball mill critical speed | nc = (1/2π) √(g/(R−r)) |

Key numbers to remember

- Moisture caking range where grinding is poor: 5–50%

- Ball mill operating speed: 65–80% of critical speed (lower for wet/viscous grinding)

- Engleberg huller speed: 600–900 rpm; yield 56–64%

- Rubber roll sheller yield: up to 95%; roll speed differential ≈ 25%

- CFTRI parboiling: soak 3–4 h at 85°C water heated by steam

- Pressure parboiling: soak 40 min at 85°C + pressurized steam for 18 min

- Bond's work index (wi) definition: energy (kWh/tonne) to reduce very large feed so 80% of product passes a 100 μm screen

End of notes — Size Reduction & Milling (GATE AG).